3GPP TSG RAN WG1 Meeting #86	R1-167208

Gothenburg, Sweden 22nd -26th August 2016

Agenda Item:	8.1.5

Source:	Huawei, HiSilicon

Title:	Discussion on NR UL MIMO

Document for:	Discussion and decision

Introduction

In LTE, UL MIMO is a key feature that has been supported to boost the uplink throughput. Comparing with maximum four transmit antennas supported at UE in LTE, new antenna architecture for UE has been discussed and agreed considering a wide range of carrier frequency supported in NR [1]. On the other hand, from the waveform perspective, OFDM is now a promising candidate waveform to be adopted for uplink which can further facilitate the MIMO performance. Taking all this factors into account, there should be some new considerations for UL MIMO design in NR. In RAN1#85 meeting, the agreements for UL MIMO are as follows

- The following techniques are studied for NR UL MIMO

  - Uplink transmission/reception schemes for data channels

      - Non reciprocity based UL MIMO (e.g. PMI based)

      - Reciprocity based UL MIMO. E.g. UE derives precoder based on downlink RS measurement (including partial reciprocity)

      - Support of MU-MIMO

      - Open-loop/Close-loop single/Multi point spatial multiplexing

        - e.g. for multi point SM, multi layer is received either jointly or independently by different TRPs

        - Note: for multi point SM, multiple point may have coordination

      - Single/Multi panel spatial diversity

      - Uplink antenna/panel switching (UE side)

      - UL beamforming management for analog implementation

      - Combination of above techniques

  - UL RS design considering the below functions

    - Sounding

    - Demodulation

    - Phase noise compensation

  - UL transmit power/timing advance control in the context of UL MIMO

  - Transmission scheme(s) for carrying UL control information

  - Other UL MIMO and related techniques are not precluded.

In this contribution, we will present our views on the UL MIMO design.

UL MIMO design in NR

Transmission schemes

In LTE, two transmission schemes with single antenna transmission and closed loop MIMO transmission are supported for PUSCH. Closed loop MIMO is codebook based transmission. The eNB indicates the precoding information in the UL grant. A single precoding matrix indicated by this PMI will be applied for the data transmission of the whole scheduled bandwidth by the UE. However, the system bandwidth can increased to 100MHz for NR in contrast with 20MHz in LTE. The channel property varies significantly across the whole system bandwidth. The rank and precoding matrix of different subband in large system bandwidth could be different. As shown in Figure 1, the rank distribution across different subbands changes obviously.

Figure 1.Rank distribution in 100MHz system bandwidth

Observation 1: The rank and precoding matrix of different subband in large system bandwidth e.g.100MHz, varies significantly.

In LTE, reporting mode of PUSCH 3-2 with subband PMI and subband CQI is introduced .It brings significant performance gain especially when TAE exist. This reporting mode provides the flexibility of frequency selective precoding for downlink transmission in different PRG. The precoder is identical with one PRG, but different precoders can be applied for different PRG.

However, in LTE, uplink transmission still adopts a single precoder across the whole bandwidth which is lack of flexibility and it should be enhanced for NR.

Proposal 1: Frequency selective precoding for uplink transmission should be supported for NR.

In the DCI design, it should consider the signalling to indicate the precoding information for different subband. However, the moderate overhead should also be guaranteed when designing the signalling.

In LTE, DMRS based transmission provides the feasibility for non codebook based precoding for downlink transmission. The eNB can obtain the precoder based on the measurement on SRS for TDD system, and the precoder may not necessarily be the precoding matrix defined in the codebook. The precoder based on the eigen vector of the channel can better reflect the channel characteristic and therefore it has the performance gain over the codebook based precoding.

For UL transmission, non codebook based precoding should also be considered as downlink transmission to increase the uplink throughput. But it is not practical to indicate the non codebook precoder in the UL grant directly as it will cost huge amount of signalling. One feasible way is to rely on the channel reciprocity, and UE acquires the precoder based on the measurement on the downlink reference signal. However, it is risky to completely rely on UE’s determination of the precoder on its own. The interference to other UEs will be randomly and uncontrollable without the coordination of the eNB. On the other hand, even if the UE obtains the precoder information through the measurement on DL RS, it still can not apply it for PUSCH transmission directly without the knowledge of scheduling information especially the MCS information. And MCS should be determined by the eNB on the basis of the assumption with a specific UL precoder. Thus, the SRS ports precoded with a set of precoders must be transmitted for eNB to determine the MCS.

Figure 2.Non codebook based transmission for PUSCH

The procedure of non codebook based transmission for PUSCH is shown in Figure2.Firstly, UE acquires the downlink channel based on the measurement on the downlink RS and derives multiple candidate precoders for uplink transmission based on reciprocity. Then the SRS ports precoded with the multiple candidate precoders are transmitted on the configured SRS resources. eNB measure the multiple SRS ports precoded with the candidate precoders and indicate the selected ones to the UE in the UL grant. UE will use the indicated precoder to implement the beamforming for the PUSCH.

Proposal 2: Non codebook based transmission for PUSCH should be considered in NR.

For low mobility scenarios, channel dependent precoding in the spatial multiplexing mode is clearly better as the UE is able to track the instantaneous channel realization thereby focusing the transmission in “directions” which are strong, thus providing array gain and raising the SINR on the eNodeB side. For high mobility scenario, the long-term property of the channel can still be utilized if significant correlation is possible. Then, semi open loop MIMO can be applied for UL transmission to enjoy the array gain.DCI for supporting semi open loop MIMO should be design to indicate a set of precoders for cycling to UE.

Proposal 3: Semi open loop MIMO should be supported for UL transmission when long-term property of the channel can still be utilized.

Transmit diversity for UL transmission is not supported in LTE as no sufficient gain are achieved comparing with single antenna transmission. In NR, transmit diversity should be evaluated based on the new waveform to see if the potential performance gain can justify it.

Proposal 4: Transmit diversity should also be considered if sufficient gain can justify it.

DCI design

To support the multiple potential transmission schemes as discussed above, corresponding DCIs should be designed. To support the frequency selective precoding for closed loop MIMO, the PMI for different subbands should be indicated. For the non codebook based transmission, a field to inform the SRS port selection for PUSCH should be designed. A set of precoders for the cycling in open loop MIMO transmission should be indicated in the related DCI.

In LTE, the switching among all the different transmission schemes is configured through the RRC signaling. The RRC reconfiguration is semi static and not flexible enough. In NR, the dynamic switching among all the transmission schemes should be supported to fast adapt to the channel variation.

A unified DCI structure should be the designed to incorporate all the transmission schemes aiming to enable the dynamic switching between all the transmission schemes. As the signaling fields to support different transmission schemes are diverse targeting to support different functionality, a single DCI to incorporate all the signaling field for all the transmission schemes is not efficient enough as its overhead is large and some signaling field is redundant for a certain transmission scheme.

A hierarchical DCI structure such as two levels DCI structure can be considered. The first level DCI can support the common signaling field for all the transmission schemes. And the second level DCI supports the transmission scheme specific signaling field. The first level DCI can indicate whether the second level DCI exists or not. Some transmission schemes only have the first level DCI and the UE can stop the detection of the second level DCI.

Proposal 5: A unified DCI structure should be the designed aiming to enable the dynamic switching between all the transmission schemes.

Proposal 6: A two levels DCI structure can be considered with the first level DCI to support transmission scheme common signalling and the second level DCI to support transmission scheme specific signalling.

Conclusion

From the above discussion, we have the following observations and proposals:

Observation 1: The rank and precoding matrix of different subband in large system bandwidth e.g.100MHz varies significantly.

Proposal 1: Frequency selective precoding for uplink transmission should be supported for NR.

Proposal 2: Non codebook based transmission for PUSCH should be considered in NR.

Proposal 3: Semi open loop MIMO should be supported for UL transmission when long-term property of the channel can still be utilized.

Proposal 4: Transmit diversity should also be considered if sufficient gain can justify it.

Proposal 5: A unified DCI structure should be the designed aiming to enable the dynamic switching between all the transmission schemes.

Proposal 6: A two levels DCI structure can be considered with the first level DCI to support transmission scheme common signalling and the second level DCI to support transmission scheme specific signalling.

- References

3GPP, “Draft Minutes report of 3GPP TSG RAN WG1 #85 v0.1.0”, Nanjing, China, May 23-27, 2016.

- Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Channel Model | 3D channel model refere to 36.873 |
| Velocity | 3km/h |
| eNB Antenna Configuration | 8 Tx /8Rx cross polarized array with 0.5λ antenna spacing |
| UE Antenna Configuration | 4 Tx/4Rx cross polarized array with 0.5λ antenna spacing |
| CP | Normal |
| Signal Bandwidth | 100MHz |
| Channel Estimation | Non-Ideal |
| Receiver | MMSE receiver |