3GPP TSG RAN WG1 Meeting #86	R1-167239

Gothenburg, Sweden, August 22nd - 26th, 2016

Agenda Item:	8.1.4.1

Source:	Huawei, HiSilicon

Title:	Field test results of channel coding schemes

Document for:	Discussion and Decision

# Introduction

In [1] simulation results of Polar code for eMBB scenario were shown and it was concluded that “Polar code exhibits a great degree of flexibility by performing well in a wide range of eMBB scenarios requiring a large range of code lengths and code rates”. In [2] simulation results of Polar Code, LTE Turbo Code and LDPC for eMBB scenario were shown and it was concluded that “Based on the evaluation and comparison with other candidate schemes, the polar coding scheme should be further studied for the NR eMBB scenarios.”

As FEC is an inseparable part in a radio communication system, a standalone simulation of channel code is not sufficient to investigate its performance. In order to evaluate the performance of Polar Code at the system level and demonstrate the feasibility of this solution, we constructed a field test environment to evaluate polar codes.

Polar codes have been successfully deployed in the test field. Meanwhile, till now, the performance for NR channel coding schemes are mainly compared by simulations on ideal AWGN channel. However, in this contribution, the coded data pass through the real fading channel, and channel estimation also takes effect in the system as shown in Figure 1.

Figure 1. Simplified system diagram

For a fair comparison, we design a special sub-frame structure as shown in Figure 2. The first slot of one sub-frame is coded by Turbo codes, and the second one by Polar Codes. Both Polar slot and Turbo slot are subjected to the same code length, code rate code, and channel conditions.

Figure 2. Frame structure used for field tests

Next reports from the field test results are given.

# Field test environment

We implement a polar decoder (SCL32 decoder) at NR UE prototype test-bed and integrate it in our field trial test cell environment that would measure its real-time throughput performance at real radio environment. For comparison, LTE Turbo 6-iteration MLM decoder with early termination is tested in parallel for the performance reference. HARQ is not employed in the test.

Figure 3 shows a photo of the test environment, which is taken at the position of BTS (base-station). In this test case, one UE (UE#0) lies at the centre of the cell and another UE (UE#1) lies at the edge of the cell in order to test Polar Code’s performance at different SNR ranges. The system parameters are shown in Table 1.

Figure 3. Test environment (one central UE and one edge UE behind the trees)

Table 1 System settings

| Configuration Item | Value |
| --- | --- |
| Operating Band | 4.65GHz~4.75GHz |
| Data subband bandwidth | 20MHz (100RB) |
| BS TRx Number | 2T2R |
| UE TRx Number | 1T2R |
| UE Number | 1 |
| MIMO mode | open loop SFBC |
| Numerology | Subcarrier spacing:15KHz |
|  | CP length:5.2us/4.17us |
|  | TTI length : 0.5ms |
| CRS | Yes |
| CQI feedback | Yes |
| ACK/NACK feedback | Yes |
| AMC | OLLA AMC |
| Traffic | Full buffer |

A trial is designated to a downlink transmission from BTS and one targeted UE and lasts for approximately one minute to measure its throughput. By testing with different UEs (UE#0 or UE#1) and adjusting power control, we are able to test both Polar Code and LTE Turbo Code with the SNRs at the low, medium and high range in real time.

# Field test results

We show the average throughputs of the system when Polar or Turbo is used in this section.

Table 2. Throughput performance from field tests

|  |  | Turbo |  | Polar |  | Throughput Gain |
| --- | --- | --- | --- | --- | --- | --- |
| SNR range | RB | Successful decoding rate | Throughput(Mbps) | Successful decoding rate | Throughput(Mbps) |  |
| Low (around 10~11dB) | 100 | 89.891% | 31.918 | 99.706% | 35.563 | 11.420% |
|  | 50 | 90.188% | 17.198 | 98.139% | 18.901 | 9.902% |
| Medium (around 17dB) | 100 | 89.963% | 51.299 | 99.288% | 56.733 | 10.593% |
|  | 50 | 90.451% | 25.759 | 99.323% | 28.367 | 10.125% |
| High (around 27dB) | 100 | 89.527% | 58.830 | 97.561% | 65.134 | 10.716% |
|  | 50 | 90.244% | 34.159 | 99.899% | 37.844 | 10.788% |

Table 2 shows that the successful decoding rate and system throughput of Polar codes are higher than LTE Turbo in the field tests. According to the test results, Polar codes can be deployed in practical communication systems, and can provide high system throughput as LTE Turbo codes do. Meanwhile, there is a stable system throughput gain around 10% of Polar codes against LTE Turbo codes.

Observation 1: High throughput Polar codes can be deployed in real-time systems, and are better than LTE Turbo codes from the successful decoding rate and system throughput point of view.

# Conclusion

In test field, we have successfully deployed the high throughput Polar codes, whose performance is better than LTE Turbo codes.

Observation 1: High throughput Polar codes can be deployed in real-time systems, and are better than LTE Turbo codes from the successful decoding rate and system throughput point of view.

# References

[1] R1-164375, Evaluation of polar codes for eMBB scenario

[2] R1-164377, Performance of channel coding schemes for eMBB scenario