3GPP TSG RAN WG1 Meeting #86                                                                          R1-167348

Gothenburg, Sweden 22nd - 26th August 2016

Source:	NTT DOCOMO

Title:	Codebook Enhancement for eFD-MIMO

Agenda Item:	7.2.4.2.1

Document for: 	Discussion and Decision

# Introduction

At the RAN1 #85 meeting [1], non-precoded CSI-RS codebook enhancement was discussed and the following conclusion was made:

Conclusion:

- Class A codebook enhancement:

  - Starting from Rel-13 Class A codebook design structure by enabling different N1 and N2 combinations

- Reduce the number of combinations of codebook parameters (N1, N2, O1, and/or O2)

  - To be concluded in RAN1#86

  - Examples:

    - Down select from 19 (N1,N2) combinations; and/or

    - Reduce the number of (O1,O2) combinations

    - Other possibilities are not excluded

Codebook enhancement was also discussed in another agenda item, i.e., advanced CSI reporting and interfernece measurement, where comes one candidate for PMI enhacement, i.e., combination-based codebook enhancement. At the RAN1 #84b meeting [2], the following conclusion was made:

Candidates for PMI Enhancement:

- Codebook based (implicit feedback)

  - Linear combination codebook (enhanced W2) for Non-precoded CSI-RS and beamformed CSI-RS

  - MU CSI (e.g., additional i1, i1,1, or i1,2)

  - Other new or modified codebooks

- Explicit feedback

  - Channel quantization

  - Eigenvector quantization

  - Covariance matrix quantization

- Analog feedback

- Note: other schemes are not precluded

In this contribution, we discuss the remaining issues associated with the codebook design. We discuss the down-selection of the antenna port layouts and the oversampling factors; and candidates for codebook enhancement, including LTE Rel. 13 codebook based enhancement, as well as linear combination based codebook enhancement.

# Antenna ports layout and oversampling factor

The codebook enhancement aims to support newly added {20, 24, 28, 32} ports. We summarize the possible N1 and N2 combinations in Table 1.

Table 1: Possible (N1, N2) combinations for {20, 24, 28, 32} ports.

| Port Number | 20 | 24 | 28 | 32 |
| --- | --- | --- | --- | --- |
| Possible (N1, N2) combinations | (2, 5) (5, 2) (1, 10) (10, 1) | (2, 6) (6, 2) (3, 4) (4, 3) (1, 12) (12, 1) | (2, 7) (7, 2) (1, 14) (14, 1) | (2, 8) (8, 2) (4, 4) (1, 16) (16, 1) |

From the table above, there are 19 possibilities in total, including both 1D and 2D antenna ports layouts. In practice, it is not very feasible to mount extremely wide antenna array. On the other hand, it has been evaluated and observed that having a very tall antenna array is not very beneficial in terms of performance. So 2D antenna pattern is preferred when down selection is necessary.

For oversampling factors, Table 2 lists the possible antenna ports and oversampling factor combinations. Table 3 lists the combinations in another form. When all the possibilities are taken into consideration, that would be a lot of work. To minimize the standardization efforts, down selection would be a choice. We propose to evaluate the ports layouts and the over-sampling factor combinations to support down selection.

Table 2: Combinations of (N1, N2) and (O1, O2)

| Number of antenna ports | N1, N2 | O1, O2 |
| --- | --- | --- |
| 20 | (2,5) | (4, 4), (4, 8) |
|  | (5,2) | (4, 8), (8, 8) |
| 24 | (2,6) | (8, 4), (8, 8) |
|  | (6,2) | (4, 8), (4, 4) |
|  | (3,4) | (8, 4), (8, 8) |
|  | (4,3) | (4, 8), (4, 4) |
| 28 | (2,7) | (8, 4), (4, 4) |
|  | (7,2) | (4, 8), (8, 8) |
| 32 | (2,8) | (8, 4), (8, 8) |
|  | (8,2) | (4, 8), (4, 4) |
|  | (4,4) | (4, 4), (8, 8) |

Table 3: Combinations of (N1, N2) and (O1, O2)

| Number of antenna ports | N1, N2 | O1, O2 |
| --- | --- | --- |
| 20 | (2,5) | (4, 4), (4, 8), (8, 4), (8, 8) |
| 24 | (2,6) | (4, 4), (4, 8), (8, 4), (8, 8) |
|  | (3,4) | (4, 4), (4, 8), (8, 4), (8, 8) |
| 28 | (2,7) | (4, 4), (4, 8), (8, 4), (8, 8) |
|  | (3,4) | (4, 4), (4, 8), (8, 4), (8, 8) |
| 32 | (2,8) | (4, 4), (4, 8), (8, 4), (8, 8) |
|  | (4,4) | (4, 4), (4, 8), (8, 4), (8, 8) |

Proposal 1: Down selection from all the AP layouts possibilities. 2D layouts are preferred from perspectives of flexibility and performance. Conduct evaluation to support ports and oversampling factor combination down selection.

# Codebook enhancement considerations

## LTE Rel.13 codebook based enhancement

As the antenna port numbers are extended, the beam width becomes narrower. Based on this observation, there are some discussion on candidates for codebook enhancement. For example, extend beam group from 8 beams to 16 beams [3], or include more beams in W1 [4]; decrease the oversampling size to widen the spacing of beams [5]; extend Rel.13 codebook by introducing antenna grouping [6], etc. Some evaluation results were presented, from the simulation results it is observed that performance gain based on these alternatives are not delightful [3][6][7][8]. It is noticed that Rel. 13 codebook based enhancement does not incur significant increase of the feedback overhead.

Further investigation of the above alternatives is needed to improve the performance.

Proposal 2: LTE Rel. 13 codebook based enhancement need further study on performance improvement.

## Linear combination codebook based enhancement

In advanced CSI reporting topic, linear combination codebook scheme was being discussed. Before discussing this scheme, we want to review the Rel. 13 PMI feedback design.

In Rel. 13 class A feedback, PMI feedback is divided into two parts. One part feeds back wideband PMI which corresponds to W1, the other part feeds back wide-band/sub-band PMI which corresponds to W2. W1 is long term feedback and W2 is short term. The BS collects W1 and W2 feedback, fix the entire feedback precoder by W1*W2.

Linear combination codebook scheme proposes combining several beams to better reflect the multi-path effect. Comparing linear combination codebook with Rel. 13 codebook design, more information like beam scaling and phase coefficients are required. Scaling adjusts the relative beam amplitude and phases adjust the relative beam angles. This will bring significant changes to the codebook design and the feedback scheme.

In order to reuse the Rel. 13 codebook and feedback scheme as much as possible, linear combination codebook scheme can take Rel. 13 codebook and feedback scheme as a baseline.  That is, set one beam as basis beam which is selected following the Rel.13 beam selection procedure. Then, other beams are selected and combined with the basis beam. As showed in Figure 1, the basis beam b0 is selected by step 1, using the same mechanism with Rel.13’s beam selection. Linear combination is conducted in step 2. The combined beam, e.g., b8 is selected and the index should be informed to the BS. Besides the beam indices, the UE needs to feedback relative amplitude and phase for the latter selected beam.

Figure 1. Linear combination beam selection

Since the feedback content is extended, feedback design should take the overhead into consideration. Linear combination codebook feedback can also take the Rel.13 feedback as baseline: W1 feedback keeps unchanged, the W2 selection is the same as Rel. 13. To feedback coefficients, sub-band feedback can be divided into two steps: the first step reuses Rel. 13 feedback scheme; the second step feeds back extra information like combination coefficients and phases. The procedure is illustrated in Figure 2.

Figure 2. Linear combination based codebook feedback procedure

Proposal 3: For linear combination based codebook enhancement, consider reuse Rel. 13 codebook design and feedback scheme as much as possible.

# Summary

In this contribution, we discuss the ports layout and oversampling combinations down selection and the alternatives for codebook enhancement. We propose:

Proposal 1: Down selection from all the AP layouts possibilities. 2D layouts are preferred from perspectives of flexibility and performance. Conduct evaluation to support ports and oversampling factor combination down selection.

Proposal 2: LTE Rel. 13 codebook based enhancement need further study on performance improvement.

Proposal 3: For linear combination based codebook enhancement, consider reuse Rel. 13 codebook design and feedback scheme as much as possible.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #85 (v0.1.0)”, May,2016.

- 3GPP, R1-163961 “Final Report of 3GPP TSG RAN WG1 #84bis (v1.0.0)”, April,2016.

- 3GPP, R1-164525, LG Electronics ,“Discussion on codebook design for eFD-MIMO”,May,2016.

- 3GPP, R1-164776, Samsung, “View on class A codebook extension”,May,2016.

- 3GPP, R1-164145, Intel Corporation “Discussion on Class A codebook extension” ,May,2016.

- 3GPP, R1-164221, CATT,“Codebook design for {20,24,28,32} ports”, May,2016.

- 3GPP, R1-164312, ZTE, “Discussion on rank 1-2 codebook design for eFD-MIMO”, May,2016.

- 3GPP, R1-165097, Ericsson, “Class A Codebook Design for up to 32 Ports in Rel-14”, May,2016.