3GPP TSG RAN WG1 Meeting #86 	R1-167365

Gothenburg, Sweden 22nd - 26th August 2016

Source:	NTT DOCOMO

Title:	Evaluation for CRS-based MUST with 4 Tx antennas

Agenda Item:	7.2.5.3

Document for: 	Discussion and Decision

Introduction

At the RAN#71 meeting, a new work item regarding Downlink Multiuser Superposition Transmission (MUST) for LTE was agreed [1]. In the WI, up to 4 Tx CRS-based or 8 Tx DMRS-based transmission schemes can be used for MUST.

- For all three Cases using up to 4 Tx CRS-based or up to 8 Tx DMRS-based transmission schemes, evaluate the system-level performance based on the evaluation methodology and assumptions in TR36.859.

In this contribution, the system-level performance of DL MUST for Case 1 with 4 Tx CRS-based transmission scheme is evaluated and discussed based on the evaluation methodology and assumptions in TR36.859.

Simulation assumptions

The system-level simulations are conducted with MUST scenario 1 and evaluation assumptions in [2]. The detailed simulation parameters are shown in Table A-I in the Annex A. We assume 2x2 and 4x2 antenna configurations with SU-MIMO of TM4. MUST with CRS-based transmission scheme is considered. When MUST is applied, up to two superposed data layers from two co-scheduled UEs per spatial layer are supported and the maximum number of spatial layers in a cell is two. For the superposed UEs per spatial layer, the same precoder are applied. We assume multiple transmission power sets with 3 adaptive power ratios for non-uniform composite constellation and legacy power ratios for uniform composite constellation, as described in [3]. Transmission power alignment for different subbands is assumed. The scheduling SINRs of superposed users are approximated from LTE OFDMA implicit feedback and OLLA is further applied to compensate the CQI imperfections in the evaluations. The R-ML receiver is adopted for MUST-Near UE to deal with the inter-user interference.

In the evaluations, FTP traffic model 1 with resource utilization (RU) ranging from 70% to 90% are assumed. File dropping is modeled according to [4]. For performance metrics of FTP traffic, 5/50/95%, mean user perceived throughput (UPT) and mean UPT below 5% CDF are evaluated. In addition, ratio of served cell throughput over offered cell throughput is also provided.

For 2x2 antenna configurations, the size of codebook for precoding is 4. For 4x2 antenna configuration, the basic codebook size for precoding is 16, as the descriptions in [5].

Simulation results

Table I: Performance of MUST with 2x2 and 4x2 SU-MIMO for subband scheduling for FTP traffic (~70%, 80% RU)

| Throughput (Mbps) | 2x2 antennas |  |  | 4x2 antennas |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline | MUST | Gain | Baseline | MUST | Gain |
| Mean UPT | 11.8688 | 12.2950 | 3.59% | 13.7610 | 13.9363 | 1.27% |
| 95%ile UPT | 38.0952 | 38.0953 | 0.00% | 42.1053 | 42.1053 | 0.00% |
| 50%ile UPT | 7.6924 | 8.0808 | 5.05% | 9.3026 | 9.5239 | 2.38% |
| 5%ile UPT | 1.4626 | 1.5595 | 6.63% | 1.8736 | 1.9231 | 2.64% |
| Mean UPT below 5% | 1.0437 | 1.1238 | 7.67% | 1.3810 | 1.4207 | 2.87% |
| RU (%) | 71.75% | 70.85% | ---- | 70.03% | 69.72% | ---- |
| Served/Offered | 99.57% | 99.63% | ---- | 99.77% | 99.78% | ---- |
| λ / packet size | 12.0 / 100 Kbytes |  |  | 14.5 / 100 Kbytes |  |  |
| Throughput (Mbps) | 2x2 antennas |  |  | 4x2 antennas |  |  |
|  | Baseline | MUST | Gain | Baseline | MUST | Gain |
| Mean UPT | 8.7860 | 9.3872 | 6.84% | 9.1146 | 9.4538 | 3.72% |
| 95%ile UPT | 29.6297 | 30.7693 | 3.85% | 30.7692 | 30.7697 | 0.00% |
| 50%ile UPT | 5.2981 | 5.7556 | 8.64% | 5.4422 | 5.7555 | 5.76% |
| 5%ile UPT | 1.0001 | 1.1060 | 10.59% | 1.0382 | 1.1128 | 7.19% |
| Mean UPT below 5% | 0.6646 | 0.7592 | 14.23% | 0.7129 | 0.7805 | 9.48% |
| RU (%) | 82.60% | 81.51% | ---- | 84.87% | 84.24% | ---- |
| Served/Offered | 98.95% | 99.21% | ---- | 99.16% | 99.33% | ---- |
| λ / packet size | 13.5 / 100 Kbytes |  |  | 17.0 / 100 Kbytes |  |  |

Table I presents the system performance of MUST with 2x2 and 4x2 SU-MIMO for subband scheduling for FTP traffic, with RU around 70% and 80%. As shown in Table I, it can be observed that: MUST can achieve performance gains with FTP RU~70% and 80% traffic load for 2 Tx and 4 Tx antennas. For FTP traffic with RU~70%, MUST with 4 Tx antennas can achieve about 1.27% gains for mean UPT and 2.64% gain for 5%ile UPT. When high traffic load is assumed, i.e. RU~80%, performance gains of MUST with 4 Tx antennas are about 3.72% for mean UPT and 7.19% for 5%ile UPT. The performance gains of MUST with 4 Tx reduce compared to those with 2 Tx antennas.

Observation 1: MUST with 4 Tx antennas can achieve 3.72% for mean UPT 7.19% for 5%ile UPT with RU around 80%. Performance gains of MUST for 4 Tx are lower than those for 2 Tx.

Figure 1 shows the statistics of UE pairing for MUST with 2 Tx and 4 Tx antennas for FTP traffic (RU~80%). From the results, we can observe that the UE pairing probability for MUST decreases as number of antennas increases. For 4 Tx, the percentage of MUST UE pairing is about 7%, while 12% of UE pairing for 2 Tx. Since the codebook size in 4 Tx is larger than that in 2 Tx, the UE pairing probability for MUST with same precoder restriction becomes smaller with 4 Tx antennas. Therefore, the performance gains of MUST with 4 Tx reduce in respect to those with 2 Tx. To improve the MUST UE pairing probability, we need to consider the methods like reducing the codebook size for 4 Tx antennas.

Fig. 1. UE pairing probability for MUST with 2x2 and 4x2 antennas configurations (FTP traffic, RU~80%)

Observation 2: UE pairing probability for MUST in 4 Tx with same precoder restriction is smaller due to larger size of codebook in 4 Tx compared to 2 Tx

Observation 3: Methods to improve the MUST pairing probability for 4 Tx antennas need to be considered.

Conclusion

In this contribution, we discussed the evaluation of MUST with 4 Tx antennas CRS-based transmission scheme. According to the discussion, we have the following observations:

Observation 1: MUST with 4 Tx antennas can achieve 3.72% for mean UPT 7.19% for 5%ile UPT with RU around 80%. Performance gains of MUST for 4 Tx are lower than those for 2 Tx.

Observation 2: UE pairing probability for MUST in 4 Tx with same precoder restriction is smaller due to larger size of codebook in 4 Tx compared to 2 Tx.

Observation 3: Methods to improve the MUST pairing probability for 4 Tx antennas need to be considered.

- References

- 3GPP RP-160680, Mediatek, “New work item proposal: Downlink Multiuser Superposition Transmission for LTE”, Mar. 2016.

- 3GPP TR 36.859 V13.0.0, “Study on Downlink Multiuser Superposition Transmission (MUST) for LTE,” Dec. 2015.

- 3GPP R1-167363, NTT DOCOMO, “Discussion on power ratios for MUST scheme,” Aug. 2016.

- 3GPP TR 36.814 V9.0.0, “Further advancements for E-URRA physical layer aspects”, Mar. 2010.

- 3GPP TS 36.211 V12.5.0, “Physical channels and modulation,” Mar. 2015.

- Annex A: Simulation assumption

Table A-I: Simulation parameters

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2/4 Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| Traffic model | FTP: 0.1 MByte, RU=60%~90% |
| UE receiver | - MMSE-IRC is assumed for inter-cell interference suppression - R-ML for inter-spatial-layer - R-ML/ideal SIC for inter-user interference |
| Transmission mode | TM4 |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| Control delay (scheduling, AMC) | 5 msec |
| HARQ | Chase combining |
| Round trip delay (HARQ) | 8 msec |
| Granularity of CSI feedback | 5 msec |
| Granularity of rank adaptation | 100 msec |
| CQI quantization | Yes |
| Codebook | LTE Rel. 8 |
| Power ratio sets | Adaptive power ratios: 3 power sets, (α1, α2), α1={0.14, 0.17, 0.23}; Legacy power ratios: (α1, α2) for near and far UEs - (QPSK, QPSK), (1/3, 2/3) rank 2-1, (1/5, 4/5) rank 1-1 and rank 2-2 - (16QAM, QPSK), (5/13, 8/13) rank 2-1, (5/21, 16/21) rank 1-1 and rank 2-2 (64QAM, QPSK), (21/53, 32/53) rank 2-1, (21/85, 64/85) rank 1-1 and rank 2-2 |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Receiver impairment modeling for demodulation | Non-ideal CRS-based channel estimation |
| EVM | Tx EVM: 8%, UE Rx EVM: 4% |
| Duration of simulation | 50000 msec (5000 pre-run + 45000 simulation) |
| Maximum transfer time (T_drop) | 1.6 sec |