3GPP TSG RAN WG1 Meeting #86							R1-167371

Gothenburg, Sweden 22nd - 26th August 2016

Source:	NTT DOCOMO, INC.

Title:	sPUSCH for shortened TTI

Agenda Item:	7.2.12.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#72, the new Work Item on “shortened TTI and processing time for LTE” was approved [1]. The detailed objectives for shortened TTI include the following:

For Frame structure type 1: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 2-symbol sTTI and 1-slot sTTI for sPDSCH/sPDCCH

- Specify support for a transmission duration based on 2-symbol sTTI, 4-symbol sTTI, and 1-slot sTTI for sPUCCH/sPUSCH

  - Down-selection is not precluded

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

For Frame structure type 2: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 1-slot sTTI for sPDSCH/sPDCCH/sPUSCH/sPUCCH

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

In addition, following agreements on channel design for sPUSCH and UE behavior on uplink sTTI transmissions were made at RAN1 #85 meeting [2] and RAN1 #84bis meeting [3]. These consensuses achieved in RAN1 so far should be the baseline.

Agreement: - For DM-RS of sPUSCH, the followings are recommended to be supported: - For the case of 1-slot TTI length, reuse the current DM-RS - For the case of less than 1-slot TTI length, support DM-RS sharing/multiplexing of consecutive TTIs from one or multiple UEs - At least 2 contiguous TTIs can be shared/multiplexed. FFS on sharing /multiplexing of more than 2 contiguous TTIs - FFS on how to share/multiplex DM-RS (e.g., CDM, FDM, different BW between RS & data, and hybrid thereof) - FFS the DMRS position and whether it is indicated by eNB Agreement: - The following principles on sPUSCH are recommended to be supported: - UCI transmission on sPUSCH is supported - FFS UCI mapping rule, especially considering the impact of DM-RS design - FFS on which carrier to multiplex UCI on in case different sTTI lengths are used on different UL CA - Note: - The UCI herein refers to at least the ones for sTTI operations - FFS whether or not the UCI also includes the ones for legacy TTI operations Agreements: - A UE can be dynamically (with a subframe to subframe granularity) scheduled with PUSCH and/or sPUSCH - A UE is not expected to transmit PUSCH and short TTI sPUSCH simultaneously on the same REs, i.e. by superposition - FFS whether a UE may transmit PUSCH and short TTI sPUSCH in the same subframe on one carrier by puncturing PUSCH - FFS whether a UE may transmit PUSCH and short TTI sPUSCH in different PRBs on the same symbol(s) - Dropping/prioritization rules (if any) are FFS

In this contribution, we provide our views on the sPUSCH design for 2-symbol and 4-symbol sTTI length. Then, we propose UE behavior in case of being scheduled with legacy TTI unicast PUSCH and short TTI unicast PUSCH in the same subframe on the same carrier. PUCCH design for shortened TTI (sPUCCH) can be found in our companion contribution [4].

- PUSCH in a short-TTI (sPUSCH)

  - Channel and RS structure

According to the objectives for shortened TTI, the supported lengths of sTTI for uplink transmission could be 2-symbol, 4-symbol, and 1-slot. For 1-slot sTTI, it was agreed to re-use current DMRS which are mapped on the 3rd and 10th SC-FDMA symbols in a subframe; hence, the 1-slot sTTI pattern within one subframe is simply determined. However, for 2-symbol and 4-symbol sTTI length, actual sTTI pattern has not been clear yet and should be specified to fix UE procedure on transmitting sPUSCH. Following two cases need to be considered when specifying the sTTI pattern.

Case 1: UL sTTI patterns are designed without DM-RS multiplexing /sharing.

For Case 1, DMRS should be self-contained in each sPUSCH. Regarding the DMRS position, there are two options; (1) it is placed at the beginning of the sTTI to enable faster channel estimation or (2) it is placed at the same symbol position as in legacy PUSCH to reduce the interference. Note that the second option is not applicable to some 2-symbol sTTIs under the assumption of Case 1, since each sTTI should have its own DMRS.

For sPUSCH in 2-symbol sTTI, if single carrier property needs to be maintained, DMRS and data should be multiplexed between SC-FDMA symbols in every shortened TTI which accounts for large overhead, e.g., 50%. In addition, both symbols in a 2-symbol sTTI should be mapped on the same PRBs since one symbol must be DMRS and the other symbol must be data. This implies that intra-sTTI FH is not allowed in any cases. If a 2-symbol sTTI can span across the slot-boundary, it is difficult to handle coexistence between 2-symbol sTTI and 1ms TTI with intra-subframe FH on one carrier from scheduler point of view, so that they are not collided with each other. Especially, at least for legacy UEs, synchronous HARQ is applied to 1ms PUSCH and hence, eNB scheduler may need to handle 1ms PUSCH with synchronous HARQ and intra-subframe FH and sPUSCH with or without synchronous HARQ. Therefore, for 2-symbol sTTI adopting SC-FDMA, the sTTI pattern should be specified such that no cross slot boundary is allowed. However, there will be left one unpaired symbol within each slot as shown in Fig. 1. The unpaired symbol can be used for SRS or data transmission.

Fig. 1: Patterns without crossing slot boundary for 2-symbol sTTI using SC-FDMA

Another approach for 2-symbol sTTI is to adopt OFDMA waveform. In this case, it is preferable to multiplex DMRS and data within an OFDM symbol in every shortened TTI which is similar to DL DMRS design.  This way does not only reduce the DMRS overhead but also allow the intra-TTI FH. In addition, as shown in Fig. 2, the 2-symbol sTTI can be designed to span across slot boundary which is easier to co-exist with 1ms TTI with intra-subframe FH.

Fig. 2 Pattern with crossing slot boundary for 2-symbol sTTI using OFDMA (Pattern 3)

For sPUSCH in 4-symbol sTTI under the assumption of Case 1, it is straightforward to allow 3-symbol sTTI and split one subframe into 3/4/3/4-symbol sTTI or 4/3/4/3-symbol sTTI as shown in Fig. 3. For the 3/4/3/4-symbol sTTI pattern, the DMRS(s) for the 4-symbol sTTI is placed in the first symbol of the sTTI which is exactly the same symbol position as DMRS(s) in legacy PUSCH so that both fast channel estimation and the reduced/randomized inter-cell interference can be achievable.

Fig. 3: Patterns with 3/4/3/4-symbol per subframe (Pattern 4) and 4/3/4/3-symbol per subframe (Pattern 5)

Case 2: UL sTTI patterns are designed with DM-RS multiplexing /sharing.

For sPUSCH in 2-symbol sTTI, DMRS can be shared/multiplexed by contiguous TTIs to reduce the overhead. As shown in Fig. 4, depending on the number of shared/ multiplexed sTTI and the position of shared DMRS, different patterns can be designed.

Fig. 4: Patterns for 2-symbol sTTI with DMRS sharing/multiplexing by three consecutive sTTI per slot (Pattern 6) and by two consecutive sTTI per slot (Pattern 7)

For sPUSCH in 4-symbol sTTI, one DMRS symbol can be shared by two time-domain continuous short-TTIs within one slot. One way is to put the DMRS at the 1st symbol of each slot for faster channel estimation for both 4-symbol sTTIs within one slot, while there is delay between the DMRS and the second 4-symbol sTTI as shown in Fig. 5. The other way is to re-use current DMRS position, while the channel estimation for the first 4-symbol sTTI within one slot cannot start until the end of the sTTI.

Fig. 5: Pattern for 4-symbol sTTI with DMRS sharing/multiplexing by two consecutive sTTI per slot

For both sPUSCH in 2- and 4-symbol sTTI with DMRS sharing/multiplexing, in order to allow scheduling sPUSCH on different PRB sets between neighboring short-TTIs, DMRS multiplexing/de-multiplexing mechanism would be necessary.

Based on the specified UL sTTI pattern, sPUSCH structure can be realized by simply scaling TBS of the legacy PUSCH according to the length of TTI.

Proposal 1:

- For the case that UL sTTI patterns are designed without DM-RS multiplexing /sharing,

  - For 2-symbol sTTI,

    - If 2-symbol sTTI uses SC-FDMA, the sTTI pattern should be specified with the restriction that no cross slot boundary is allowed.

      - FFS the use of one unpaired SC-FDMA symbol

    - If 2-symbol sTTI uses OFDMA, the 2-symbol sTTI can be designed to span across slot boundary for efficient coexistence with 1ms TTI with intra-subframe FH.

  - For 4-symbol sTTI, it is preferable to split one subframe into 3/4/3/4-symbol sTTI and place the DMRS in the 1st symbol of each sTTI.

Proposal 2:

- PUSCH in a short-TTI (sPUSCH) can be realized by scaling TB size according to the given TTI length.

- UE behavior on UL sTTI operation

For shortened TTI operation, it is beneficial for a UE to support simultaneous transmission of legacy TTI PUSCH and short TTI sPUSCH in the same subframe in the same carrier. The first reason is that the traffic requiring low latency can break in at the middle of a subframe, even if the normal traffic using 1ms TTI is scheduled for the UE in the subframe. The second reason is that those two transmissions have different HARQ/scheduling timeline and therefore handling collisions between those two transmissions would be difficult. Therefore, it should be allowed to schedule unicast sPUSCH during the time-interval where unicast PUSCH is already scheduled.

Proposal 3:

- Scheduling unicast sPUSCH during the time-interval where unicast PUSCH is already scheduled should be allowed.

  - Advanced UEs may be able to perform simultaneous transmission of legacy TTI PUSCH and short TTI sPUSCH in the same subframe in the same carrier.

eNB can assume that the UE can support simultaneous transmission of legacy TTI PUSCH and short TTI PUSCH in the same subframe in the same carrier. If the UE is not able to transmit them simultaneously, a certain UE behavior should be defined. Following three alternatives can be considered.

Alt. 1: UE can encode and transmit both unicast PUSCH and sPUSCH in the same subframe in the same carrier under some conditions, except these conditions, UE always encodes and transmits sPUSCH.

Possible conditions can be the scheduled PUSCH is SPS PUSCH and/or the TBS/MCS of the PUSCH is under a threshold and/or the total summed TBS of the unicast PUSCH and sPUSCH is under a threshold. Additional factors that need to be taken into account are whether the scheduled PRBs for sPUSCH and PUSCH are overlapped and/or the UE is power limited. If these conditions are met, then UE can transmit both sPUSCH and PUSCH on their respective resources. If the scheduled PRBs for sPUSCH and PUSCH are overlapped and/or the UE is power limited, by puncturing PUSCH, the UE can transmit PUSCH and sPUSCH in the same subframe in the same carrier. One example is shown in Fig. 3.

Fig. 3:	Simultaneous transmission of PUSCH and sPUSCH by puncturing PUSCH

Except above conditions, UE should always prioritize encoding and transmitting sPUSCH once scheduled, and drop/abort the PUSCH transmission if necessary. With this UE behavior, low latency traffic can be scheduled to UEs flexibly regardless of their ability.

Alt. 2: UE always prioritize encoding and transmitting short TTI unicast PUSCH.

If the eNB wants to prioritize legacy TTI unicast PUSCH, it should not schedule short TTI unicast sPUSCH in the same subframe in the same carrier when the low latency traffic arrives; otherwise eNB should schedule short TTI unicast PUSCH for the UE transmitting legacy TTI unicast PUSCH. When the UE is aware of collision between legacy TTI unicast PUSCH and short TTI unicast sPUSCH, UE should drop/abort transmission of legacy TTI unicast PUSCH. This alternative also ensures the same understanding between eNB and UE and has small specification impact while lowers the UE capability especially for advanced UEs.

Alt. 3: left to UE implementation, UE can encode and transmit both of the legacy TTI unicast PUSCH and short TTI unicast PUSCH or select to encode and transmit one of them.

Different from Alt. 1 and Alt. 2, Alt. 3 gives the final say to the UE. UE can encode and transmit both of them or either one of them according to its capability. No common understanding between eNB and UE is ensured, blind detection at eNB side is needed. This alternative also keeps the benefits of supporting simultaneous transmission of legacy TTI unicast PUSCH and short TTI unicast sPUSCH in the same subframe in the same carrier.

Proposal 4:

- For non-advanced UEs, eNB can schedule simultaneous transmission of legacy TTI unicast PUSCH and short TTI unicast sPUSCH in the same subframe in the same carrier.

- UE behavior in case of being scheduled with legacy TTI unicast PUSCH and sPUSCH in the same carrier in the same subframe should be defined.

- Conclusion

In this contribution, we discussed PUSCH design for shortened TTI and made following proposals.

Proposal 1:

- For the case that UL sTTI patterns are designed without DM-RS multiplexing /sharing,

  - For 2-symbol sTTI,

    - If 2-symbol sTTI uses SC-FDMA, the sTTI pattern should be specified with the restriction that no cross slot boundary is allowed.

      - FFS the use of one unpaired SC-FDMA symbol

    - If 2-symbol sTTI uses OFDMA, the 2-symbol sTTI can be designed to span across slot boundary for efficient coexistence with 1ms TTI with intra-subframe FH.

  - For 4-symbol sTTI, it is preferable to split one subframe into 3/4/3/4-symbol sTTI and place the DMRS in the 1st symbol of each sTTI.

Proposal 2:

- PUSCH in a short-TTI (sPUSCH) can be realized by scaling TB size according to the given TTI length.

Proposal 3:

- Scheduling unicast sPUSCH during the time-interval where unicast PUSCH is already scheduled should be allowed.

  - Advanced UEs may be able to perform simultaneous transmission of legacy TTI PUSCH and short TTI sPUSCH in the same subframe in the same carrier.

Proposal 4:

- For non-advanced UEs, eNB can schedule simultaneous transmission of legacy TTI unicast PUSCH and short TTI unicast sPUSCH in the same subframe in the same carrier.

- UE behavior in case of being scheduled with legacy TTI unicast PUSCH and sPUSCH in the same carrier in the same subframe should be defined.

References

- RP-161299, “New Work Item on shortened TTI and processing time for LTEs,” Ericsson.

- 3GPP RAN1#85, Chairman’s note.

- 3GPP RAN1#84bis, Chairman’s note.

- R1-167370, ‘sPUCCH for shortened TTI,’ NTT DOCOMO, INC., August 2016