3GPP TSG-RAN WG1 Meeting #86	R1-167561

Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:   8.1.2.2

Source:	  InterDigital Communications

Title:	On the Performance of IDMA and RSMA Multiple Access Schemes

Document for:	  Discussion

# Introduction

In RAN1 #85 meeting, a wide range of multiple access (MA) schemes for uplink were proposed. The meeting agreements covers the following aspects related to the scope of the contribution [1];

- Autonomous/grant-free/contention based UL non-orthogonal multiple access has the following characteristics

  - A transmission from UE does not need the dynamic and explicit scheduling grant from eNB

  - Multiple UEs can share the same time and frequency resources

- For autonomous/grant-free/contention based UL non-orthogonal multiple access, the following should be studied

  - Collision of  time/frequency resources from different UEs, solutions potentially including

    - E.g., code, sequence, interleaver pattern

  - UL synchronization (DL synchronization assumed)

    - Case 1: Timing offsets between UEs are within a cyclic prefix

    - Case 2: Timing offsets between UEs can be greater than a cyclic prefix, FFS the exact model of timing offsets

  - Requirement for power control

    - Case 1: Perfect open-loop power control, i.e., equal average SNR between UEs for potentially link level calibration

    - Case 2: Realistic open-loop power control with certain alpha and P0 values

    - Case 3: Close-loop power control

  - Receiver impact

In a companion contribution, we have provided our views on categorization of multiple access systems [2]. At a high level, the proposed MA schemes can be categorized under two main clusters; 1) spreading- and 2) non-spreading. In turn, spreading schemes can be either based on a long or a short spreading mechanism.

IDMA [3] and RSMA [4] are considered the main candidates of schemes built on long spreading structure. In this contribution, we present our comparative analysis and simulation results for these two multiple access methods. The main focus of the contribution is on the collision performance and the receiver structure.

# Multiple Access Schemes with Very Low Code Rates

Earlier studies on spread spectrum multiple access systems indicated that the optimal capacity of a multiple access channel can be achieved if the spreading is entirely realized by employing a very low coding rate [5]. A common way to realize very low coding rate is by combing a low code rate with repetition coding. Since this does not result in an orthogonal set of channels, user-based randomization through interleaving and/or scrambling is introduced to assure discrimination of users at the eNB.

## Interleave Division Multiple Access

Figure 2 shows the basic structure of an IDMA-based multiple access system with K number of active users. At the transmitter, each UE payload is coded and then randomized prior to transmission in the shared channel. The randomization operation can be implemented through a combination of interleaving/scrambling functions. At the receiver side, the combined signal is jointly processed in a recursive manner through a two-step process of Elementary Signal Estimator (ESE) and decoding [6]. In brief, the ESE function provides updates on a-priori log likelihood ratio (LLR) estimates required for the decoding, and the decoder provides updated a-posteriori LLR estimates that are employed for ESE updates. The cycle of updates is iterated across the users several times prior to the detection of di, i=1,⋯,K (i.e., turbo processing).

The main feature of IDMA principle lies in that it does not treat the observed interference as an additive noise. The ESE function assist in enhancing the a-priori LLR’s by updating relevant statistics of the received signal and the interference.

Figure 1 Basic architecture of an IDMA system

## Resource Spread Multiple Access

Figure 2 shows the basic structure of an RSMA-based multiple access system with K number of active users. The transmitter structure is essentially the same as of an IDMA system where each UE payload is coded and then randomized prior to transmission in the shared channel. Similarly, the randomization operation may be realized through a combination of interleaving/scrambling functions. At the receiver side, the combined signal is processed in a disjoint manner such that each user’s signal is detected and decoded independent of other users’ signals. Hence, interference is treated as an additive noise and no attempt is made for enhancing a-priori LLR estimates of the demodulator.

The performance of an RSMA system can be improved by inclusion of successive cancellation capability. If a user’s payload is decoded correctly, its original signal can be regenerated and subsequently subtracted from the received signal facilitating the detection of other users. This operation is depicted in Figure 2 in red color.

Figure 2 Basic architecture of an RSMA system

# Operational Aspects

There are clear operational differences between an IDMA and an RSMA receiver that impact the complexity of the receiver processing. The main differences can be summarized as follows;

- IDMA receiver processing rests on completion of several iterations (N) of updates that cycles between ESE and the decoder (i.e., turbo processing). Hence, its receiver complexity and associated delay are directly impacted by the number of iterations. For example, consider a K-user system with LDPC coding, an IDMA receiver performs N number of LDPC decoding per user, while an RSMA receiver (without successive cancellation) requires only one LDPC decoding per user.

- An IDMA receiver is based on a joint processing mechanism that does not support detection of an arbitrary user. On the contrary, an RSMA receiver (without successive cancellation) is indeed capable of selectively processing and detection of a desired user.

- While IDMA relies on turbo decoding of all users jointly (i.e, N×K decoding), RSMA-SIC relies on detection of all or a subset of users in a joint/sequential manner. Therefore, a trade-off on the number of iteration can be made based on the required performance.

# Performance Evaluation

# Simulation Assumptions

An uplink multiple access system with K number of synchronous users is assumed. Table 1 summarizes the assumed simulation parameters. The low-rate coding is implemented by cascading an LDPC code 16 along with a repetition code rate (R=6) that yields an effective spreading factor of SF=36. With a K number of active users in the system and each using the BPSK modulation, total spectral efficiency of the system is K/36 bits/s/Hz.

Given the modulation format of BPSK and SF=36, per user spectral efficiency is 1/36 bits/s/Hz and the corresponding single user Shannon limit for an additive white Gaussian noise (AWGN) channel is at -17.11 dB SNR. We use this single user Shannon limit as a point of reference in our presented results for comparison. Moreover, to better reflect the interference-tolerance capability of the studied NOMA schemes, the performance of Orthogonal Multiple Access (OMA) is also considered. We consider an OMA system with the same LDPC code 16 without repetition code i.e., R=1 and thus, the total spectral efficiency achievable by OMA system for BPSK modulation is 1/6 bits/s/Hz. Therefore, comparing the same total spectral efficiency for both NOMA and OMA systems, at least 6 number of users should be supported by the NOMA scheme.

Table 1 Simulation Parameters

| Number of UL UEs | 1, 2, 4, 6, 8 and 10 |
| --- | --- |
| eNB-UE Configuration | 1TX-1RX |
| Modulation | BPSK |
| Channel Model | AWGN |
| Payload Size | 170 bits |
| Coding | LDPC (1/6) |
| Repetition Rate (R) | 6 |
| Per UE SE | 0.0278 bits/s/Hz |
| IDMA Iterations | 5 |
| Randomization | Random Interleaving |
| Power control | Perfect |

Figure 3 demonstrates the BER and BLER performance of IDMA, RSMA and RSMA-SIC multiple access systems. From the presented results, following observations can be made:

- The performance of the single user case for all schemes is about 2 dB away from the Shannon limit. The distance may be further reduced by improving the FEC coding scheme.

- IDMA receiver;

  - The number of users can go beyond the spectral efficiency of an OMA system.

  - As SNR increases, the average BLER performance of the system is not impacted by the number of interfering users.

  - Even with 10 active users and a target BLER=10-2, the average BER performance of the system is about 7.8 dB better than that of an OMA system.

- RSMA receiver;

  - Similar to IDMA, the number of users can go beyond the spectral efficiency of an OMA system.

  - The performance of the system is impacted as the number of interfering users increases.

  - Assuming 8 active users and a target BLER=10-2, the average BLER performance of the system is about 3 dB better than that of an OMA system. In case of 10 interfering users, the BLER results in only 1 dB worse than that of an OMA system.

- RSMA-SIC receiver;

  - Similar to previous cases, the number of users can go beyond the spectral efficiency of an OMA system.

  - The performance of the system is impacted as the number of interfering users increases but at a much slower rate than that of RSMA.

  - Assuming 10 active users and a target BLER=10-2, the average BLER performance of the system is about 4.8 dB better than that of a OMA system.

Figure 4 shows relative performance of the three studied NOMA schemes. For better clarity, the interference cases are limited to 4 and 8 users. The presented results indicate the effectiveness of the RSMA-SIC in reducing the performance gap between IDMA- and RSMA-based receivers. Assuming a target BLER=10-2, the performance gap is reduced from ~5 to ~2 dB and from ~1.6 to ~0.3 dB for 8 and 4 interfering user cases, respectively.

# IDMA

# RSMA

# RSMA+SIC

Figure 3 BER and BLER performance of IDMA, RSMA and RSMA-SIC

Figure 4 Comparative BER and BLER performance of IDMA, RSMA and RSMA-SIC

# Conclusion

In this contribution, we presented our evaluation results for three non-orthogonal multiple access schemes of IDMA, RSMA and RSMA-SIC. Based on our results, following observations can be made;

Observation 1 – IDMA-based multiple access system is the most robust scheme against increasing number of active users.

Observation 2 – Using successive interference cancellation, an RSMA receiver can reduce its performance gap with IDMA, at the price of an increase in the receiver complexity.

Proposal – RAN1 considers further study and evaluation of IDMA and RSMA-based multiple access systems.

# References

[1] Chairman’s Notes RAN1 #85, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China 23rd – 27th May 2016

[2] R1-167563, On categorization of multiple access candidates for NR, InterDigital Communications, 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden, 22nd – 26th August 2016

[3] R1-165021, Performance of Interleave Division Multiple Access (IDMA) in Combination with OFDM Family Waveforms, Nokia/Alcatel-Lucent, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China 23rd – 27th May 2016

[4] R1-164689, RSMA and SCMA comparison, Qualcomm, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China 23rd – 27th May 2016

[5] A. J. Viterbi, “Very low rate convolutional codes for maximum theoretical performance of spread spectrum multiple-access channels,” IEEE Selected Areas in Communications, Vol. 8, August 1999

[6] L. Ping, L. Liu, K. Wu and W.K. Leung, “Interleave-Division Multiple-Access,” IEEE Transaction on Wireless Communications, Vol. 5, April 2006.