3GPP TSG-RAN WG1 #86	R1-167635

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Ericsson

Title:	Support of CSI-RS port sharing with Rel-13 and Rel-12 UEs

Agenda Item:	7.2.4.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#84bis, the following agreements were made with regards to CSI-RS design for Class A eFD-MIMO and a number of alternatives for reducing per-port CSI-RS density were identified for further study:

Agreements:

- For {20, 24, 28, 32} ports, a CSI-RS resource for class A CSI reporting is composed as an aggregation of K CSI-RS configurations [i.e. RE patterns].

  - The number of REs in the kth configuration Nk ∈ {4, 8}

    - The same Nk = N can be used for all k

      - FFS whether the same Nk = N for all k is the only permitted configuration

      - FFS whether the set of values of Nk might be further restricted for some numbers of CSI-RS ports

    - FFS whether a different set of Nk might apply in case of CDM4

    - FFS on including Nk=2.

  - Aim to enable the support of CSI-RS port sharing with Rel-13 and Rel-12 UEs

  - The per-port CSI-RS density is FFS based on one or more of the following alternatives:

    - FDM

    - TDM

    - Partial port

    - Partial overlapping, e.g. for 32 ports, ports 15-38 in PRB#1, ports 23-46 in PRB#2

    - Aperiodic CSI-RS with partial bandwidth

    - Measurement restriction in frequency domain

    - CDM, e.g. 2 x Nk ports transmitted in a single Nk resource

    - Other schemes

    - Note that the following are not precluded:

      - per-port CSI-RS density per PRB = 1

      - different per-port CSI-RS densities for different CSI-RS ports is not precluded

In this contribution, we present our views on the issue of enabling support of CSI-RS port sharing with Rel-13 and Rel-12 UEs.

# Discussion

In a practical scenario, a notable proportion of the UEs being served by a Rel-14 eNodeB will be Rel-13 and Rel-12 UEs.  One approach to accommodating these pre-Rel-14 UEs in a Rel-14 eNodeB with {20, 24, 28, 32} ports is to assign different CSI-RS resources for the different types of UEs.  Some examples of this first approach are given below:

- Example 1:  A Rel-14 eNodeB with 32 ports is configured with one 32 port NZP CSI-RS resource intended for Rel-14 UEs and one separate 16 port NZP CSI-RS resource intended for Rel-13 UEs.  Both NZP CSI-RS resources have 1 CSI-RS RE/RB/port.

- Example 2:  A Rel-14 eNodeB with 32 ports is configured with one 32 port NZP CSI-RS resource intended for Rel-14 UEs and one separate 16 port NZP CSI-RS resource intended for Rel-13 UEs.  The 32-port NZP CSI-RS resource has 0.5 CSI-RS RE/RB/port and the 16-port NZP CSI-RS resource has 1 CSI-RS RE/RB/port.

An alternative approach is to aim for support of CSI-RS port sharing with Rel-13 and Rel-12 UEs as agreed in RAN1 #84bis.  With this approach one NZP CSI-RS resource is configured wherein some of the ports can be shared with pre Rel-14 UEs.  Some examples of the port sharing approach are given below:

- Example 3:  A Rel-14 eNodeB with 32 ports is configured with one 32 port NZP CSI-RS resource.  Ports 15-30 are shared with Rel-13 16 port UEs.  All ports have 1 CSI-RS RE/RB/port.

- Example 4:  A Rel-14 eNodeB with 32 ports is configured with one 32 port NZP CSI-RS resource.  Ports 15-30 are shared with Rel-13 16 port UEs and have 1 CSI-RS RE/RB/port.  Overhead reduction is applied to ports 31-46 which have 0.5 CSI-RS RE/RB/port.

The CSI-RS overheads associated with Examples 1-4 are quantified in Table 1.  The overhead reduction calculations in Table 1 assume a system with CSI-RS reuse 3, 2 CRS ports, 3 OFDM symbols for PDCCH, and 2 DMRS ports.  The CSI-RS periodicity is assumed to be 5 ms.

Table 1.  CSI-RS Overheads associated with Examples 1-4

| Example | CSI-RS overhead |
| --- | --- |
| 1 | 26.67% |
| 2 | 17.78% |
| 3 | 17.78% |
| 4 | 13.33% |

As can be seen from the table, useful overhead reductions can be achieved when CSI-RS port sharing is combined with overhead reduction on the non-shared CSI-RS ports.  In scenarios where full density CSI-RS is needed and Example 1 will be configured, port sharing using Example 3 provides about 9% reduction in CSI-RS overhead.  Comparing Examples 2 and 4, we see that gains of around 4.5% are possible in scenarios where half density CSI-RS is sufficient.

With a CSI-RS reuse factor of 3, useful overhead reductions (e.g. ~4-9%) can be achieved with CSI-RS port sharing.

# Simulation Results

In this section, we evaluate the performance of a 32-port CSI-RS configuration shown in Figure 1 which is suitable for port sharing with Rel-13 16 port UEs.  We performed system level simulations using a 32 port 8x4 dual polarized array with 2x1 subarray virtualization.  In the 32-port CSI-RS configuration of Figure 1, ports 15-30 can be shared with Rel-13 16 port UES and have 1 CSI-RS RE/RB/port and overhead reduction is applied to ports 31-46 which have 0.5 CSI-RS RE/RB/port (i.e., Example 4 of Section 2).  In addition, a 3 dB power boosting with CDM 4 (borrowing power from other CSI-RS REs) is assumed.  Detailed simulation parameters and assumptions are given in the Appendix.

Figure 1: A 32 CSI-RS configuration suitable for port sharing with Rel-13 16 port UEs

The performance of the 32 port CSI-RS configuration in Example 4 of Section 2 is shown in Table 2 (3D-UMi) and Table 3 (3D-UMa).  The performance gains shown in the tables are relative to a 32 port CSI-RS configuration with all ports having 1 CSI-RS RE/RB/port (i.e., Example 3 of Section 2).  As can be seen in Tables 2-3, the 32-port port sharing CSI-RS configuration combined with overhead reduction on non-shared ports (i.e., Example 4) outperforms the 32-port port sharing CSI-Rs configuration with no overhead reduction on the non-shared ports (i.e., Example 3).  Mean throughput gains of 13-15 % are observed at high load.

CSI-RS port sharing with overhead reduction on non-shared ports can yield notable (e.g. 13-15% mean throughput) performance gains over CSI-RS port sharing with no overhead reduction on non-shared ports.

Table 2.  Performance comparison between CSI-RS configurations in examples 3 and 4 for 3D-UMi

Table 3.  Performance comparison between CSI-RS configurations in examples 3 and 4 for 3D-UMa

Given the above two observations, a CSI-RS port sharing configuration that allows overhead reduction on the non-shared CSI-RS ports should be supported as part of Class A CSI-RS deign for eFD-MIMO.  As noted in our companion contribution [1], a measurement restriction mechanism in frequency domain can be used to enable port sharing with reduced overhead on the non-shared CSI-RS ports.  Hence, we make the following proposal:

For Class A eFD-MIMO, consider to support CSI-RS overhead reduction on a portion of CSI-RS ports to enable port sharing.

# Conclusions

In this contribution, we present our views on the issue of enabling support of CSI-RS port sharing with Rel-13 and Rel-12 UEs.  Based on the discussion and simulation results, we made the following observations:

Observation 1	With a CSI-RS reuse factor of 3, useful overhead reductions (e.g. ~4-9%) can be achieved with CSI-RS port sharing.

Observation 2	CSI-RS port sharing with overhead reduction on non-shared ports can yield notable (e.g. 13-15% mean throughput) performance gains over CSI-RS port sharing with no overhead reduction on non-shared ports.

Based on the discussion in Sections 2-3, we propose the following:

Proposal 1	For Class A eFD-MIMO, consider to support CSI-RS overhead reduction on a portion of CSI-RS ports to enable port sharing.

# References

R1-167633, “CSI-RS Design for Class A eFD-MIMO”, Ericsson, 3GPP TSG-RAN WG1 #86, Gothenburg, Sweden, August 22-26, 2016.

# Appendix A

| Simulation parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD, 3D UMa 500m ISD |
| Antenna Configurations | 8x4 with 2x1 virtualization tilt: 130° for 3D-UMi and 122° for 3D-UMa |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (3D-UMi), 46 dBm (3D-UMa) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| CRS interference | 2 CRS ports, Colliding CRS. |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modelled. Reuse factor 3, sectors with the same orientation (i.e., 0°, 120°, 240°) are grouped together. |
| Codebook | 2D Grid of Beams based on DFT |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |