3GPP TSG-RAN WG1#86	R1- 167643

Gothenburg, Sweden 22rd - 26th August 2016

Source:	Ericsson

Title:	Explicit versus implicit feedback for advanced CSI reporting

Agenda Item:	7.2.4.2.3

Document for:	Discussion and Decision

# Introduction

In RAN1#85, the following was agreed regarding advanced CSI reporting:

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

In this contribution, the pros and cons of the two categories of CSI reporting are discussed and schemes for each category are proposed and evaluated.

# Pros and cons with explicit and implicit feedback

The question of whether to formulate a new CSI feedback mechanism using implicit or explicit feedback methodologies has been discussed previously in RAN1, for instance when 8TX CSI feedback was introduced in Rel. 10. So far, RAN1 has decided to continue to formulate CSI feedback enhancements using an implicit feedback framework, but for the advanced CSI reporting enhancements considered in Rel. 14, this may not necessarily continue to be the case.

In an explicit feedback methodology, a UE strives for feeding back a representation of a channel state as observed by the UE on a number of antenna ports, without taking into account how the reported CSI is processed by the eNB when transmitting data to the UE. Similarly, eNB also does not know how such a hypothetical transmission is processed by the UE on the receiver side. The channel part of the feedback report could consist of quantized coefficients of the channel matrix H, the TX correlation matrix HHH, the RX correlation matrix HHH, the vectorized correlation matrix vecHvecHH or the eigenvector(s) of the said correlation matrices. The UE may also perform some additional processing of the observed channel matrix such as transforming it to a matrix space where the channel can be expressed more sparsely in order to reduce the number of channel coefficients. The experienced interference level would also be conveyed to the eNB in the form of some observed quantity, such as an RX antenna correlation matrix measured on an IMR.

In an implicit feedback methodology, on the other hand, a UE feeds back a desired transmission hypothesis as well as consequences of that transmission hypothesis. In LTE, the transmission hypothesis is indicated with a PMI and RI which maps to a precoder matrix in the configured codebook which is applied to the measured antenna ports to form the hypothetical transmission. The CQI correspondingly indicates the appropriate MCS given the indicated transmission hypothesis and thus takes receiver processing into account.

We qualitatively discuss and compare the two feedback methodologies around a number of criterions below:

- Feedback overhead: With implicit feedback, one has a better opportunity to optimize the feedback overhead consumed in the uplink by clever codebook design that matches the properties of the intended propagation channels. Explicit feedback naturally consumes larger overhead than its implicit counterpart. This speaks in favor of implicit feedback.

- Precoding performance: Explicit feedback has the potential to achieve better precoding performance than implicit feedback since the eNB has the full channel matrices at its disposal, and so, with sufficiently fine coefficient quantization, the performance with explicit feedback should theoretically approach the upper bound results. However, in order to make a fair comparison, precoding performance for different feedback schemes should be compared at the same level of feedback overhead. It is thus not obvious that explicit feedback has a precoding performance advantage over implicit feedback at the same level of overhead. We investigate this further in Section 4.

- MU/SU CQI mismatch: Since implicit feedback per definition is calculated assuming a certain transmission hypothesis, there is an inherent CQI mismatch if another transmission hypothesis than the assumed one is used. Thus, if CQI is calculated according to an SU hypothesis and the eNB chooses to perform MU transmission, it must try to estimate an appropriate CQI based on the co-scheduled UEs CSI reports. The eNB may of course instruct the UE to report implicit feedback assuming MU transmission, but this quickly becomes infeasible when there are many UEs in the cell as the number of MU hypotheses become vast. Further, it may not be optimal to apply the reported precoders directly, but instead apply additional SLNR/ZF/MMSE processing to suppress MU interference. Since advanced CSI reporting is mainly targeting increased MU performance, this speaks in favor of choosing explicit feedback. However, it should be noted that since RX processing is not taken into account in explicit feedback, the link adaptation will suffer from an inherent mismatch in that case as well, particularly if advanced receivers are used at the UE.

- RAN4 Testability: Performance testing in RAN4 is crucial in order to ensure interoperability between equipments from different vendors and indeed to assure we have a functional standard.  Currently, CSI reporting is validated with PMI tests where the ratio of the throughput when the PMI is selected by the UE over when the PMI is selected at random is tested. For explicit feedback, it is unclear how correct UE behavior should be enforced as the feedback report only includes observed quantities that are not directly related to any downlink transmission. It should not be taken for granted that different vendors will have a coherent definition of these quantities in their respective implementations. One option for testing explicit feedback could be to define a test case where e.g. single layer SVD precoding based on the reported channel is used. However, then we are essentially defining the transmission hypothesis the UE should assume when calculating the feedback report, so then one might just as well reframe it into an implicit report where the transmission hypothesis assumed is the same as the one in the RAN4 test case as that anyway will be the de facto transmission hypothesis UE vendors will optimize their implementation towards. Thus, for the sake of testability, it may be beneficial to phrase the feedback report as an implicit report even if the intention is to feed back explicit channel coefficients.

- Specification impact: As only a few RAN1 meetings remain in Rel. 14, the specification impact of advanced CSI enhancements should be considered. If explicit feedback is to be specified, this would require large changes to the spec as a new feedback methodology not previously used in LTE would have to be introduced. If implicit CSI reporting enhancements is chosen on the other hand, existing definitions such as CQI & RI and procedures for CSI reporting could be reused and the spec impact could boil down to only introducing a new PMI definition for the new advanced CSI codebook. We note though, that since the payload for advanced CSI reports is likely to be larger than the capacity of current feedback formats on PUSCH and PUCCH, regardless if explicit or implicit feedback is selected, new methods for carrying the CSI report on the physical channels may have to be introduced. Given the timeframe, it seems unlikely that there will be enough time to decide on and specify both new feedback formats for carrying the CSI report as well as a new feedback methodology for explicit feedback. Thus, from this perspective, implicit feedback enhancements seem like the better choice. With that in mind, we note that specification of new feedback formats for carrying the CSI report could (and should) be made forward compatible with Rel. 15, so that any new feedback formats specified in Rel. 14 could be reused for explicit feedback if it is introduced in Rel. 15.

Based on the above discussion, we make the following observations:

Observations:

- Precoding performance of explicit and implicit feedback should be compared at the same level of feedback overhead

- Implicit feedback suffers from inherent SU/MU CQI mismatch while explicit feedback suffers from increased specification impact and testability issues.

# Proposed explicit and implicit CSI schemes

In this section, we present our view on how advanced CSI enhancements should be specified, for both explicit and implicit feedback.

## Explicit feedback scheme

It is clear that the explicit channel feedback has to be compressed in some way in order to have a reasonable overhead, as feeding back the full channel matrix for each subband would lead to intolerable amounts of overhead. The traditional approach of most compression algorithms is to utilize correlation between samples in order to compress them. One such way is to find a basis where the sample vector can be expressed sparsely, perform a basis change by multiplying the sample vector with a basis transformation matrix, and finally discard everything but a couple of the strongest coefficients of the sample vector expressed in that basis. If the sample vector is sparse enough, the compression loss is small.

Such a method could of course be applied to the explicit channel feedback problem, and a natural basis that would sparsely express the channel matrix H from a 2D antenna array is a 2D DFT basis. Given that the propagation channel between the 2D antenna array and the UE antenna is spatially correlated, the channel matrix, expressed in 2D-DFT beam space, could be significantly sparser than expressed in antenna space. The equivalent 2D-DFT beam space channel matrix can be derived by multiplying the antenna space channel matrix H with an orthonormal transformation matrix BN1,N2 =DN1⨂DN200DN1⨂DN2 as

HDFT=HBN1,N2 ,

where DN is a size N×N DFT matrix, i.e. the elements of DN  are defined as  DNk,l=1Nej2πklN and N1, N2 are the number of antenna ports in the horizontal and vertical dimension, respectively. The columns dii=12N1N2 of  BN1,N2 constitutes an orthonormal basis of the vector space C2N1N2 and each column corresponds to a DFT beam transmitted on one polarization.

An explicit feedback scheme could thus work by selecting the Nbeams strongest beams (which could be selected once for the entire bandwidth) and only feeding back the 2NbeamsNRX beam space channel coefficients of the reduced beam space channel matrix HDFT,Nbeams obtained by selecting Nbeams columns from HDFT, where NRX  is the number of receive antennas at the UE. Each beam space channel coefficient is then uniformly quantized in gain and phase with Nbits/coeff bits per coefficient. It is noted that reduced beam space feedback is applicable also to eigenvector feedback or correlation feedback.

One can further reduce the number of beams required to express the channel matrix by allowing the beam space to be rotated, that is, multiplying the DFT matrix for each dimension with a size N×N rotation matrix RNq=diag(ej2π⋅0⋅qNej2π⋅1⋅qN⋯ej2π⋅(N-1)⋅qN) , defined for 0≤q<1. Multiplying DN with RNq from the left creates a rotated DFT matrix with entries RNqDNk,l=1Nej2πk(l+q)N.  That is, a rotated 2D beam space transformation matrix may be expressed as

BNV,NH qV,qH=RNHqHDNH⨂RNVqVDNV00RNHqHDNH⨂RNVqVDNV=d10d20……dNVNH00d10d2……0dNVNH=b1b2…b2N1N2   .

We assume that the rotation factors q are uniformly quantized, i.e. q1=iQ1, i=0,1,…,Q1-1. q2=iQ2, i=0,1,…,Q2-1 and that 4 rotation hypothesis per dimension is used, i.e. Q1=Q2=4.

Rotating the beam space basis has an effect similar to oversampling a codebook, for example, if the channel is a pure LOS channel and the angle of the LOS ray if perfectly aligned with a constituent beam in the beam space, the channel matrix can be described by only one beam coefficient. However, if the angle of the LOS ray lies in between two beams in the beam space, two beam coefficients are required to express the channel, doubling the amount of overhead needed.

Depending on the number of bits used to quantize the channel coefficients Nbits/coeff and how many columns of the beam space channel matrix Nbeams, i.e. how many beams, are fed back, the overhead is as follows.

Overhead calculation:

- Beam selection + rotation: Nbeams⋅log2N1N2+log2Q1Q2 bits (wideband)

  - 4Nbeams+4 bits assuming N1=N2=Q1=Q2=4

- Coefficient quantization: 4NbeamsNbits/coeff bits per subband assuming channel matrix feedback with 2RX (or rank 2 eigenvector feedback)

- Assuming 9 subbands, the total overhead: Nbeams⋅log2N1N2+log2Q1Q2+9⋅4NbeamsNbits/coeff bits

## Implicit feedback scheme

Our proposal for implicit advanced CSI enhancements is presented at greater detail in our companion contribution [2]but is summarized here for convenience.

We propose that a new advanced CSI codebook should keep the familiar factorized codebook structure, where the precoding matrices W are decomposed into a wideband W1 matrix factor and subband W2 matrix factor as

W=W1W2

but where new codebooks for W1 and W2 are designed.

The W1 design for advanced CSI reporting should include:

- Beam selection: Provide an unconstrained selection of Nbeams orthogonal (2D) DFT beams in order to ensure that the selected beams correspond to the multi-path components of the channel. Hence, beam selection is not limited to select beams from a subset of beams.

- Beam power allocation: Setting the relative power level of the beams in the precoder so that the relative strength of the different multi-path components of the channel are taken into account.

- Beam space rotation: Rotating the orthogonal DFT basis from where beam selection is done in order to maximize the channel energy captured by the precoder, similar to oversampling.

The W2 design for advanced CSI reporting should include:

- Beam co-phasing: Co-phasing of the selected beams in W1.

One may thus express the W1 matrix as the multiplication of a beam selection matrix  BIS and a beam power matrix P as

W1= BISP.

The selected beam matrix consists of columns from BNV,NH qV,qH, as defined in the previous Section, where Nbeams beams are selected, as

BIS=dIS(1)0dIS(2)0……dIS(Nbeams)00dIS(1)0dIS(2)……0dIS(Nbeams),

where Is denotes the selected beam indices

IS=i1,i2,…,iNbeams=N2n1(1)+n2(1),N2n1(2)+n2(2),…,N2n1(Nbeams)+n2(Nbeams),

where n1(m),n2(m) denotes the horizontal and vertical orthogonal DFT beam indices of the mth selected beam, respectively. The beam power matrix can be expressed as

P=diag1,p2,…,pNbeams,1p2,…,pNbeams=I2⨂100p2⋱⋱⋱00pNbeams,

where 0≤pi≤1 ,i=2,…,Nbeams denotes the relative power allocation for each selected beam.

The enhanced W2 matrix may then be expressed as

W2=1ejα21⋮ejα2Nbeams1………1ejα2r⋮ejα2Nbeamsr,

where r is the transmission rank, Nbeams is the number of DFT beams in W1 per polarization and ejαkl is a co-phasing factor from a PSK alphabet intended for the kth column of W1 and the lth layer.

Given that W1 comprises Nbeams beams, that the relative power allocations pi may be selected from 2Nbits,power different values and that the co-phasing factors may be selected from a 2Nbits,phase-PSK constellation, the resulting feedback overhead will be as follows.

Overhead calculation:

- For W1 (wideband):

  - Beam selection + rotation: Nbeams⋅log2N1N2+log2Q1Q2 bits

    - 4Nbeams+4 bits assuming N1=N2=Q1=Q2=4

  - Beam power selection: Nbeams-1⋅Nbits,power bits

- For W2 (subband):  22Nbeams-1⋅Nbits,phase per subband, assuming rank-2 report

- Assuming 9 subbands, the total overhead: Nbeams⋅log2N1N2+log2Q1Q2+Nbeams-1⋅Nbits,power+ 9⋅ 22Nbeams-1⋅Nbits,phase

# Evaluation results

As per our observation in Section 2, we compare the proposed explicit and implicit feedback schemes as a function of their respective feedback overhead. For each scheme, we simulate a number of systems with different parameterizations and thus a different level of performance and feedback overhead. For the explicit feedback scheme, systems with different numbers of beams Nbeams and different number of bits for coefficient quantization Nbits/coeff have been simulated. For the implicit feedback scheme, systems with different numbers of beams Nbeams, different number of bits for allocating beam power Nbits,power and different PSK alphabets for co-phasing in W2 have been simulated.

We have simulated the 3GPP 3D UMi scenario with an 8x4 antenna array with 2x1 virtualization, so a 32 port system with N1=N2=4 is assumed. The advanced CSI systems perform MU-MIMO transmission and utilizes SLNR precoding. Relative gains are calculated against an SU-MIMO baseline using a Rel. 13 style codebook. Other simulation parameters and the full set of simulation results may be found in the appendix.

In order to compare the performance and feedback overhead of the simulated systems, we plot the results as a scatter plot with the cell edge UTP gain at 70% RU on the y-axis and the associated feedback overhead for channel/PMI feedback on the x-axis in Figure 1. As seen, up to ~150% CE gain is achievable with the implicit CSI scheme while up to ~250% CE gain is achievable with explicit CSI scheme. Though, to reach that level of CE gain, several thousand bits of overhead is required.

It can be observed that the performance does not monotonically increase with feedback overhead. This can be expected since a CSI report is based on a combination of parameters, and some can be excessively quantized while others have too coarse resolution.  For example, the explicit CSI result at 1500 bits uses a quantization where many beams are reported, but each beam’s gain and phase is coarsely quantized.

In order to more clearly investigate the performance of the different systems, we remove, for each scheme, the systems that have larger feedback overhead but worse performance than another system so as to only keep the best performing systems for each scheme. We present the result as a line plot in Figure 2, where we have also removed systems with an overhead larger than 1500 bits. As seen in the Figure, the implicit feedback scheme clearly outperforms the explicit feedback scheme when the feedback overhead is smaller than around 500 bits. For larger feedback overheads though, the performance of the implicit feedback scheme saturates and the explicit feedback scheme performs best. Thus, which scheme to choose from a performance versus overhead perspective depends on how much feedback overhead can be tolerated. As a reference, regular Rel. 13 style CSI feedback with 4 bits for beam selection and polarization co-phasing in W2 and 6 bits for W1 would consume 42 bits, assuming 9 subbands. While the feedback overhead tolerated for advanced CSI enhancements should be substantially larger than for regular CSI reporting, it makes sense to try to limit it to only an order of magnitude larger, i.e. 420 bits.

Figure 1: Scatter plot of different explicit and implicit CSI systems, comparing cell edge gain over SU-MIMO Rel.13 CB baseline at 70%RU as a function of feedback overhead for quantizing the channels/precoders.

Figure 2:  Plot of best performing systems of Figure 1.

Observations:

- The implicit feedback scheme clearly outperforms the explicit feedback scheme when the feedback overhead is smaller than ~500 bits.

- For feedback overheads larger than ~500 bits, the gains from implicit advanced CSI feedback saturates and the explicit feedback scheme has a clear advantage.

Since the implicit feedback scheme has an advantage in the lower overhead range, it would not make sense to only specify explicit CSI enhancements and not implicit CSI enhancements since all UE categories and capabilities may not be able to support calculating and feeding back a large payload CSI report. Given this and the discussion in Section 2, and considering the timeline for Rel.14, we make the following proposal:

Proposal:

- For advanced CSI reporting in Rel. 14, specify implicit feedback enhancements consisting of a new codebook for rich CSI with both new W1 and W2 designs.

# Conclusions

In this paper, we have discussed the pros and cons of implicit and explicit feedback based schemes for advanced CSI reporting enhancements. We have further discussed and proposed schemes for both implicit and explicit CSI feedback and presented evaluation results comparing their respective performance as a function of the associated feedback overhead. The following observations have been made:

Observations:

- Precoding performance of explicit and implicit feedback should be compared at the same level of feedback overhead

- Implicit feedback suffers from inherent SU/MU CQI mismatch while explicit feedback suffers from increased specification impact and testability issues.

- The implicit feedback scheme clearly outperforms the explicit feedback scheme when the feedback overhead is smaller than ~500 bits.

- For feedback overheads larger than ~500 bits, the gains from implicit advanced CSI feedback saturates and the explicit feedback scheme has a clear advantage.

Based on these observations, we have made the following proposal:

Proposal:

- For advanced CSI reporting in Rel. 14, specify implicit feedback enhancements consisting of a new codebook for rich CSI with both new W1 and W2 designs.

# References

RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia, CATT, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, March 7 - 10, 2016

R1-167644, “On the need for W1 enhancements for advanced CSI”,  Ericsson, RAN1#86

# Appendix

## Numerical results

Table 1: Simulation results comparing reduced beam space explicit feedback

| 50% RU |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell edge UTP gain [%] |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  | 28 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 9 |  |
| Reduced beam space explicit feedback MU-MIMO |  |  |  |  | Reduced beam space explicit feedback MU-MIMO |  |  |  |  |  |
| Number of Beams | Coefficient Quantization |  |  |  | Number of Beams | Coefficient Quantization |  |  |  |  |
|  | Unquantized | 4 bits | 6 bits | 10 bits |  | Unquantized | 4 bits | 6 bits |  | 10 bits |
| 1 | 36 | 19 | 33 | 40 | 1 | 13 | 3 | 11 |  | 12 |
| 2 | 75 | 40 | 73 | 73 | 2 | 26 | 13 | 25 |  | 27 |
| 3 | 102 | 45 | 77 | 99 | 3 | 35 | 15 | 29 |  | 35 |
| 5 | 110 | 39 | 95 | 111 | 5 | 43 | 12 | 36 |  | 43 |
| 10 | 103 | 1 | 79 | 122 | 10 | 45 | -4 | 33 |  | 48 |
| 16 (Full channel) | 114 | -19 | 73 | 106 | 16 (Full channel) | 47 | -17 | 29 |  | 46 |
| 70% RU |  |  |  |  |  |  |  |  |  |  |
| Cell edge UTP gain [%] |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  | 67 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 26 |  |
| Reduced beam space explicit feedback MU-MIMO |  |  |  |  | Reduced beam space explicit feedback MU-MIMO |  |  |  |  |  |
| Number of Beams | Coefficient Quantization |  |  |  | Number of Beams | Coefficient Quantization |  |  |  |  |
|  | Unquantized | 4 bits | 6 bits | 10 bits |  | Unquantized | 4 bits | 6 bits |  | 10 bits |
| 1 | 99 | 82 | 93 | 87 | 1 | 27 | 21 | 27 |  | 26 |
| 2 | 161 | 107 | 149 | 155 | 2 | 55 | 31 | 50 |  | 53 |
| 3 | 200 | 106 | 181 | 195 | 3 | 66 | 35 | 62 |  | 63 |
| 5 | 226 | 92 | 198 | 227 | 5 | 79 | 27 | 66 |  | 78 |
| 10 | 244 | 36 | 184 | 251 | 10 | 89 | 5 | 67 |  | 87 |
| 16 (Full channel) | 242 | *** | 154 | 207 | 16 (Full channel) | 87 | *** | 58 |  | 81 |

Table 2: Evaluation results for multi-beam codebook implicit feedback with 3 bits power quantization

| 50% RU |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  |  | 28 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 9 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 42 | 37 | 30 |  | 38 | 1 | 15 | 14 | 12 |  | 13 |
| 2 | 77 | 44 | 59 |  | 59 | 2 | 27 | 16 | 20 |  | 23 |
| 3 | 105 | 56 | 69 |  | 70 | 3 | 35 | 21 | 24 |  | 26 |
| 5 | 110 | 55 | 73 |  | 79 | 5 | 36 | 21 | 28 |  | 29 |
| 10 | 109 | 62 | 76 |  | 82 | 10 | 36 | 23 | 29 |  | 30 |
| 70% RU |  |  |  |  |  |  |  |  |  |  |  |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  |  | 67 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 26 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 105 | 89 | 85 |  | 82 | 1 | 35 | 31 | 30 |  | 31 |
| 2 | 151 | 107 | 121 |  | 116 | 2 | 53 | 37 | 46 |  | 44 |
| 3 | 180 | 114 | 132 |  | 167 | 3 | 61 | 41 | 48 |  | 56 |
| 5 | 218 | 119 | 143 |  | 152 | 5 | 69 | 46 | 53 |  | 56 |
| 10 | 225 | 121 | 162 |  | 163 | 10 | 73 | 46 | 58 |  | 59 |

Table 3: Evaluation results for multi-beam codebook implicit feedback with 6 bits power quantization

| 50% RU |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  |  | 28 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 9 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 42 | 37 | 40 |  | 38 | 1 | 15 | 14 | 14 |  | 13 |
| 2 | 69 | 42 | 52 |  | 67 | 2 | 25 | 16 | 20 |  | 24 |
| 3 | 95 | 54 | 73 |  | 69 | 3 | 32 | 19 | 25 |  | 26 |
| 5 | 100 | 51 | 82 |  | 87 | 5 | 36 | 21 | 29 |  | 31 |
| 10 | 122 | 55 | 76 |  | 88 | 10 | 40 | 23 | 30 |  | 32 |
| 70% RU |  |  |  |  |  |  |  |  |  |  |  |
| Cell edge UTP gain [%] |  |  |  |  |  | Average UTP gain [%] |  |  |  |  |  |
| Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  | Rel. 13 Style Codebook SU-MIMO |  |  |  | 0 |  |
| Rel. 13 Style Codebook MU-MIMO |  |  |  | 67 |  | Rel. 13 Style Codebook MU-MIMO |  |  |  | 26 |  |
| Multi-beam codebook MU-MIMO |  |  |  |  |  | Multi-beam codebook MU-MIMO |  |  |  |  |  |
| Number of Beams | Phase Quantization |  |  |  |  | Number of Beams | Phase Quantization |  |  |  |  |
|  | Unquantized per RB | PSK per subband |  |  |  |  | Unquantized per RB | PSK per subband |  |  |  |
|  |  | 2 bits | 3 bits |  | 10 bits |  |  | 2 bits | 3 bits |  | 10 bits |
| 1 | 105 | 89 | 85 |  | 82 | 1 | 35 | 31 | 30 |  | 31 |
| 2 | 153 | 106 | 119 |  | 131 | 2 | 52 | 39 | 43 |  | 47 |
| 3 | 191 | 110 | 153 |  | 151 | 3 | 65 | 42 | 52 |  | 54 |
| 5 | 230 | 133 | 159 |  | 175 | 5 | 75 | 48 | 55 |  | 62 |
| 10 | 226 | 116 | 152 |  | 180 | 10 | 74 | 46 | 57 |  | 64 |

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead not accounted for. Channel estimation error modeled. |
| Codebook | 2D Grid of Beams based on DFT |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |