3GPP TSG-RAN WG1#86	R1-167649

Gothenburg, Sweden, 22-26 August, 2016

Source:	Ericsson

Title:	Preliminary FeCoMP Results for Indoor Hotspot Scenario

Agenda Item:	7.2.14.3

Document for:	Discussion and Decision

# Introduction

In RAN1#85, one of the three scenarios agreed for feCOMP evaluation is Indoor Hotspot [1][2][3]. In this contribution, we present some preliminary system simulation results on CS/CB under the Indoor Hotspot scenario.

# Simulation Setup

The simulation setup is based on the agreed Indoor Hotspot scenario where UEs are dropped in an 10mx50m floor area. 8 TPs each with 2Tx antennas are located uniformly within the area as specified in [2]. The TPs are 6m above the floor.  CS/CB with explicit CSI feedback is assumed. Two different cluster sizes are studied including  4TPs and 8TPs per cluster.  Figure 1 shows the clusters with 4TPs.

4TPs /cluster

4TPs /cluster

Figure 1: 4 TPs per cluster arrangement

# Simulation Results

The simulation results are summarized in Table 1 below. The baseline scheme is per TP SU-MIMO without coordination between TPs.  It can be seen that there are some large cell edge gains with CS/CB CoMP.  The largest gains are achieved when a cluster size of 8TPs is used, i.e. transmissions over all 8 TPs are coordinated.  The cell edge throughput gains are 63% and 170% at resource utilizations of 50% and 70%, respectively.  The mean throughput suffered some losses at 50% of resource utilization.  For instance, mean throughput losses of -3% and -6% are observed for cluster sizes of 4TPs and 8TPs, respectively.  At 70% resource utilization, however, an 8% mean throughput gain is observed for the case with a cluster size of 4TPs and a 20% mean throughput gain is seen for the case with a cluster size of 8TPs.

Table 1:  Performance gains with CS/CB under the indoor  hotspot scenario

| Baseline RU = 50 % | Mean user throughput | Cell-edge user throughput |
| --- | --- | --- |
| NO COMP (baseline) | 0% | 0% |
| 4TPs per cluster | -3% | 16% |
| 8TPs per cluster | -6% | 63% |
| Baseline RU = 70 % | Mean user throughput | Cell-edge user throughput |
| NO COMP (baseline) | 0% | 0% |
| 4TPs per cluster | 8% | 42% |
| 8TPs per cluster | 20% | 170% |

Observation:

- Potential for large gains are observed with CoMP CS/CB under the indoor hotspot scenario

# Conclusion

In this contribution, we have presented some preliminary CoMP results under the indoor hotspot scenario.  The results show some potential for large performance gains with CoMP CS/CB.

# References

- Chairman’s notes RAN1#85

- R1-165845, “ Way forward on scenarios for FeCoMP evaluations”

- R1-165852, “Way forward on Evaluation Assumptions for FeCoMP”

- R1-165104, “feCoMP Motivations and Techniques”, Ericsson, RAN1#85

# Appendix

| Simulation parameters |  |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| Bandwidth | 10 MHz |
| Scenarios | Indoor Hotspot |
| TP antenna Configuration | (M,N,P) =(1,1,2) , 2D Omni with 5dBi gain |
| UE antenna | 2Rx |
| Cell layout | According to [2][3] |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | Explicit CSI feedback |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| TP Tx power | 24dBm |
| Traffic model | FTP Model 1, 100 kB packet size |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead not accounted for. Channel estimation error modeled. |
| CoMP method | CS/CB |
| CoMP cluster size | 4TPs, 8TPs |
| HARQ | Max 5 retransmissions |
| Handover margin | 3 dB |