3GPP TSG-RAN WG1 Meeting #86	                                   R1-167690

Gothenburg, Sweden, 22nd - 26th May 2016

Agenda item:		7.2.5.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	System-level Evaluation of CASE 3 MUST

Document for:		Discussion and Decision

# 1	Introduction

In the previous 3GPP TSG RAN Meeting #71 a MUST WID [1] has been approved. According to the WID, a MUST UE receiver is assumed to be capable to cancel or suppress intra-cell interference between co-scheduled MUST users for the following cases:

CASE-1: Superposed PDSCHs are transmitted using the same transmission scheme and the same spatial precoding vector

CASE-2: Superposed PDSCHs are transmitted using the same transmit diversity scheme.

CASE-3: Superposed PDSCHs are transmitted using the same transmission scheme, but their spatial precoding vectors are different.

In this contribution we investigate the system level performance of MUST CASE 3 with dynamic switching between SU-MIMO, semi-orthogonal and orthogonal pairing for 2Tx and 4Tx antennas using two-layer RML for reception. Together with the results we provide corresponding L2S methodology.

# 2	L2S for two-layer ML

Similarly to L2S for MUST CASE 1, the L2S for MUST CASE 3 system simulations has not been specified by 3GPP. Therefore, we will describe herein, the L2S we used for evaluation of CASE 3.

We start from definition of mutual information between transmitted symbol xown from a vector of transmitted symbols x=xownxcoT and received signal y.

Iy,xown,mML=log2(Xb)-1Xb∈Xbx'∈Xxownm=bEnlog2x''∈Xexp-1σn2n+Hx'-x''2x''∈Xxown''(m) =xown'(m)exp-1σn2n+Hx'-x''2,

where X=Xown×Xco is a joint constellation of symbols xown and xco,  m is the position of a bit within symbol xown assuming Gray bit to symbol mapping of Xown and Xcoconstellations.  Xb=0,1 is a bit set and σn2 is the noise-plus-interference variance, which is assumed to be white. Mutual information (MI) of bits m in constellation Xown can be summed together.

Unlike with single layer L2S, we deal with two layers, each transmitted over different equivalent channel H=hownhco. In this contribution we model MI as function of 1) Own layer SINR γown and modulation order 2) Co layer SINR γco and modulation order 2) correlation between channels ρ=hcoHhown|hown||hco|. By Monte-Carlo simulations we obtain  Iγown,γco,ρ for modulation combinations summarized in Table 1.

| Case | UE1 MOD | UE2 MOD |
| --- | --- | --- |
| TL1 | 2 | 2 |
| TL2 | 2 | 4 |
| TL3 | 4 | 2 |
| TL4 | 4 | 4 |

Table 1 Two-layer ML cases

Given mutual information and MCS N, we obtain BLER estimate. In addition, we take into account channel estimation error, namely self-interference as well as interference from the co-layer due to erroneous channel estimate.

# 3	System performance results

This section presents the results obtained by LTE system-level simulator. We perform wideband scheduling and we restrict the CSI feedback to rank-1 only. The RML L2S for both single-layer and two-layer operation has been described in previous two sections. We simulate the full-buffer traffic with 10UEs per sector for simplicity. Similarly to MUST CASE 1 investigations, gains in FTP1 were visible only in high load cases were RU is close to 90%, these high load conditions are not so different to full-buffer conditions.

Observation-1: High arrival rate FTP1 traffic conditions are not so different to full-buffer traffic conditions.

The goal of these simulations is to investigate performance of MUST CASE 3 with respect to SU-MIMO baseline. With MUST CASE 3, we assume three power splits 0.2/0.8, 0.8/0.2 and 0.5/0.5. In Table 2 and Table 3, it can be observed that MUST CASE 3 in TM4 delivers sufficient average gain in 2Tx as well as 4Tx scenario.

Proposal-1: Proceed with standardization of MUST CASE 3.

| Case | User Throughput (bps) | Baseline |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | MUST Case 3 | Gain |
| 2Tx | Cell average | 1.272E+7 | 1.590E+7 | 25.0% |
|  | Cell edge | 2.396E+5 | 2.477E+5 | 3.4% |
|  | Note: rank1 only wideband feedback |  |  |  |

Table 2 MUST 2Tx with ML receiver

| Case | User Throughput (bps) | Baseline |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | MUST Case 3 | Gain |
| 4Tx | Cell average | 1.525E+7 | 1.878E+7 | 23.2% |
|  | Cell edge | 3.053E+5 | 3.198E+5 | 4.75% |
|  | Note: rank1 only wideband feedback |  |  |  |

Table 3 MUST 4Tx with ML receiver

Last meeting it has been discussed whether only equal split or also unequal split should be supported in MUST CASE 3. In Table 4 we show the statistics of power split selection. It seems that in MUST CASE 3 unequal power split is often selected with 2Tx, and even more with 4Tx. This is because with semi-orthogonal pairing, unequal power split helps RML in decoupling of layers.

| Power split | 0.2/0.8 | 0.5/0.5 | 0.8/0.2 |
| --- | --- | --- | --- |
| 2Tx | 33.3% | 31.5% | 35.2% |
| 4Tx | 39.2 | 23.9 | 36.9% |

Table 4 Selection statistics of power scaler in MUST CASE 3

Proposal-2: Consider support of unequal power split in addition to equal power split for MUST CASE 3.

# 4	Conclusions

In this contribution we have been presenting system-level results of MUST performance for CASE-3. The following observations and proposals can be summarized:

Observation-1: High arrival rate FTP1 traffic conditions are not so different to full-buffer traffic conditions.

Proposal-1: Proceed with standardization of MUST CASE 3.

Proposal-2: Consider support of unequal power split in addition to equal power split for MUST CASE 3.

# References

- RP-160680, “DL Multiuser Superposition Transmission for LTE”, Mediatek, CMCC,  RAN #71, Gothenburg, Sweden, 7 - 10 March 2016

- R1-154458, “Link-abstraction Method for ML Receiver in MUST”, Mediatek.

# Appendix

Table 5 Simulation Assumptions

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| Number of UEs per cell | 10 |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| Minimum distance from macro-cell to UEs | 35 m |
| Traffic model | Full buffer |
| UE receiver | MMSE-diagonal covariance knowledge + RML per layer |
| Transmission mode | 2(4)x2 TM4 (rank1 only) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Redundancy Version |
| Feedback | WB rank1 only |
| CQI quantization | Yes |
| Codebook | 2Tx/4Tx LTE Rel. 8 |
| Power ratio sets | According to super-constellation |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Channel Estimation | Realistic |
| EVM | Tx/Rx 8/4% |