3GPP TSG RAN WG1 Meeting #86                                             R1-167845

Gothenburg, Sweden, 22nd-26th, August 2016

Agenda Item:		   7.1.2.2

Source:		   National Taiwan Un

Title:		  Design issues of MUST with HARQ

Document for:	   Discussion

Introduction

In this contribution, further considerations on MUST with CQI and ACK/NACK feedback delay. HARQ retransmission is one of important scheme to guarantee the reliability of transmission and/or coverage. With HARQ, the MCS/power ratio will be aggressively selected such that the first transmission may not succeed (but the overall throughput will be higher). In this case, SIC’s near UE will suffer due to unsuccessful cancellation of interference. This is not only for the first transmission, because the near UE may be paired with a different far UE at each (re)transmission, and will most likely not be able to accumulate signals to successfully decode and cancel any of the far UEs at different (re)transmissions.

Considerations on evaluation method

Block diagram

We use e exhaustive search to find two user pair can maximum the PF scheduling.

Step 1: 1. select one of the pair of users as (UE1, UE2) and selected transmission power set (α1, α2)

Step2 . Calculate the scheduling SINRs for user using the reported CQI assuming OMA and the power set (α1, α2)

    - Scheduling SINR of NOMA for near user: α1 × CQI1

    - Scheduling SINR of NOMA for far user: α2 /(α1 + 1/ CQI2)

Step3. calculate the multi-user PF metric ι∈U(τ(l|U,P)T(l))

where U denotes the candidate user set. Terms 𝑇(𝑙) and τ(l│U, P) denote the average throughput and instantaneous throughput of UEl, respectively. P denotes the allocated power sets. Instantaneous throughput can be approximated by comparing scheduling  SINR with initial MCS threshold.

Step4. repeat Steps 1-4 with all the combinations of candidate transmission power sets (α1, α2) and user sets (UE1, UE2)

Step5. Choose the pair which has highest multi-user PF metric to be temporary near user and far user

Step6. Decide who is final near user and far user: Base on temporary near user and far user.

Step7: Check temporary (near/far) user needs retransmit or not

Step8:If needs retransmit check last time was (near/far) user and its partner(near or far) needs retransmit or not

Step9: If its partner also need retransmit decide them be final near and far user

Step10:If  not both of them(near/far+ partner last time) need to retransmit  decide temporary near user and far user be final user pair

Ste p 11: Approximate the real post-processing SINR based the upper bound.

Conclusion

In this contribution, we present MUST with HARQ. By using HARQ the MCS/power ratio can be more aggressively selected such that higher MCS and lower power ratio can be utilized.

- References

- [1] YuyaSaito, Yoshihisa Kishiyama, AnassBenjebbour, TakehiroNakamura, AnxinLi, and Kenichi Higuchi, ”Non-Orthogonal Multiple Access (NOMA) for Cellular Future Radio Access”, IEEE 77th Vehicular Technology Conference (VTC Spring), p.p1 –5, 2013.

- [2] Daniele Tarchi, Romano Fantacci, Cagri Tanriove, Novel link adaptation for TETRA cellular systems, INTERNATIONAL JOURNAL OF COMMUNICATION SYSTEMS, 2009

- [3] Fan Chen, Chen Meiya, Su Lijun, Zhang Xin, Yang Dacheng, RESEARCH ON THRESHOLD ADJUSTMENT ALGORITHM IN ADAPTIVE MODULATION AND CODING, The 17th Annual IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC'06).

- Appendix

- A.1 Evaluation assumptions

| Attributes | Values or assumptions |
| --- | --- |
| Number of bean | 4 |
| Cell radius | 500 m |
| Number of user | 120 |
| Range of path loss | 90db~110dB |
| Path loss | 129.1+37.6log10(r)dB |
| AWGN | -90dB |
| Channel model | Rayleigh fading |
| Power ratio | (0.05,0.95),(0.1,0.9)…(0.45,0.55) |
| ACK feedback delay | 3 frame |
| CQI feedback delay | 5 frame |
| MCS | QPSK(1/2),QPSK(3/4),16QAM(1/2),16QAM(2/3), 16QAM(3/4),16QAM(7/8),64QAM(2/3),64QAM(3/4),64QAM(2/3) |
| UE antenna elements | 1 Rx |
| BS antenna elements | 4 Rx |
| UE antenna gain | -10 dBi |
| HARQ | Case combining |
| Channel estimation | Ideal |

- A.2 SIC Receiver of near user

- A.3 SIC Receiver of far user