3GPP TSG RAN WG1 Meeting #86                                                     R1-167870

Gothenburg, Sweden, August 22nd - 26th 2016

Source:	CATT

Title:	Update of LLS results of PDMA

Agenda Item:	8.1.2.2

Document for: 	Discussion and Decision

Introduction

In the RAN #84bis, pattern division multiple access (PDMA) was proposed as a novel multiple access (MA) Scheme for NR [1]. In the RAN #85, the initial views and preliminary LLS performances of both UL and DL were provided [2].

In this contribution, we further present LLS results to verify the gain of PDMA over orthogonal multiple access (OMA) and the robustness of PDMA in typical overloading scenarios. The PDMA detail transmission mechanisms are described in [1, 3].

Discussion

LLS of PDMA Uplink

Simulation parameters for PDMA uplink are shown in Appendix A.1.

PDMA vs. OMA

For OMA, there are two types of subcarrier allocation: localized and distributed. And the detail description is shown in Appendix A.2. In this simulation, both two types are adopted as baseline. And the MCS table of PDMA and OMA is given in Appendix A.3.

- 4 RB in TDL-A 30ns 3km/h

For 4RB in channel TDL-A 30ns 3km/h, the BLER curves of PDMA and OMA with 4UEs/8UEs/12UEs (corresponding to overloading factor of 100%/ 200%/ 300%) are shown in Figure 1, Figure 2 and Figure 3, respectively.

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 1. BLER curve of PDMA and OMA (4UEs, 4RB, and in TDL-A 30ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 2. BLER curve of PDMA and OMA (8UEs, 4 RB, and in TDL-A 30ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 3. BLER curve of PDMA and OMA (12UEs, 4RB, and in TDL-A 30ns 3km/h)

- 12 RB in TDL-A 30ns 3km/h

For 12RB in channel TDL-A 30ns 3km/h, the BLER curves of PDMA and OMA with 4UEs/8UEs/12UEs are shown in Figure 4, Figure 5 and Figure 6, respectively.

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 4. BLER curve of PDMA and OMA (4UEs, 12RB, and in TDL-A 30ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 5. BLER curve of PDMA and OMA (8UEs, 12RB, and in TDL-A 30ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 6. BLER curve of PDMA and OMA (12UEs, 12RB, and in TDL-A 30ns 3km/h)

- 4 RB in TDL-C 300ns 3km/h

For 4RB in channel TDL-C 300ns 3km/h, the BLER curves of PDMA and OMA with 4UEs/8UEs/12UEs are shown in Figure 7, Figure 8 and Figure 9, respectively.

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 7. BLER curve of PDMA and OMA (4UEs, 4RB, and in TDL-C 300ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 8. BLER curve of PDMA and OMA (8UEs, 4RB, and in TDL-C 300ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 9. BLER curve of PDMA and OMA (12UEs, 4RB, and in TDL-C 300ns 3km/h)

- 12 RB in TDL-C 300ns 3km/h

For 12RB in channel TDL-C 300ns 3km/h, the BLER curves of PDMA and OMA with 4UEs/8UEs/12UEs are shown in Figure 10, Figure 11 and Figure 12, respectively.

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 10. BLER curve of PDMA and OMA (4UEs, 12RB, and in TDL-C 300ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 11. BLER curve of PDMA and OMA (8UEs, 12RB, and in TDL-C 300ns 3km/h)

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 12. BLER curve of PDMA and OMA (12UEs, 12RB, and in TDL-C 300ns 3km/h)

The SNR gains of PDMA over OMA are summarized in Table I. From the Table I we can see, for scenarios with different overloading factor and total target SE, the performance gain of PDMA over OMA can be different. The results show that PDMA can have significant SNR gains over OMA under the same total target SE.

When the overloading factor from 100%~300%, the SNR gains of PDMA over OMA with distributed subcarrier allocation grows from 0.1~0.3 dB to 4.1~4.5 dB at the same target SE = 0.25 bps/Hz per UE and from 0.2~0.8 dB to 4.1~5.0dB at the same target SE = 0.375 bps/Hz per UE, respectively. The gap is mainly derived from non-orthogonal multiplexing and diversity.

Table I: SNR Gain (dB @ BLER = 0.1) of PDMA over OMA

| Allocated bandwidth | Channel model | SE per UE | SNR Gain (distributed/localized) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 100% | 200% | 300% |
| 4PRB | TDL-A 30ns | 0.25 | 0.2/0.2 | 2.5/2.6 | 4.1/4.1 |
|  |  | 0.375 | 0.0/0.0 | 3.2/3.2 | 5.0/5.1 |
| 12PRB |  | 0.25 | 0.1/0.2 | 2.6/2.8 | 4.1/4.3 |
|  |  | 0.375 | 0.1/0.3 | 3.1/3.3 | 4.1/4.3 |
| 4PRB | TDL-C 300ns | 0.25 | 0.1/0.4 | 2.2/2.3 | 4.1/4.2 |
|  |  | 0.375 | 0.4/0.5 | 3.2/3.2 | 4.2/4.2 |
| 12PRB |  | 0.25 | 0.3/1.1 | 2.4/3.3 | 4.5/4.9 |
|  |  | 0.375 | 0.8/1.3 | 3.1/3.7 | 4.5/5.0 |

The following observations can be made from above evaluation results:

- Observation 1: The significant gain of PDMA over OMA is up to 5.1 dB;

- Observation 2: The gain of PDMA over OMA increases as the overloading factor and target spectrum efficiency increases.

- Observation 3: Owing to frequency diversity gain by distributed subcarriers allocation, the gain of PDMA over OMA increases as the number of PRB increases.

Robustness of PDMA in overloading scenarios

In this subsection, the robustness of PDMA is investigated. SNR is defined as transmitted signal power per UE over noise power, in order to compare per UE performance under different overloading factor.

For 4RB in TDL-A 30ns, 12PRB in TDL-A 30ns, 4RB in TDL-C 300ns, and 12PRB in TDL-C 300ns, the comparison of BLER curves of overloading factor 100%/ 200%/ 300% are shown in Figure 13, Figure 14, Figure 15 and Figure 16, respectively. The target BLER is 0.1. As shown in Figure 13, the dynamic range of SNR is 1.1 dB @ (SE=0.25)/2.8 dB @ (SE=0.375) among overloading factor of 100%, 200% and 300%. As shown in Figure 14, the dynamic range of SNR is 1.2 dB @ (SE=0.25)/2.8 dB @ (SE=0.375) among overloading factor of 100%, 200% and 300%. As shown in Figure 15, the dynamic range of SNR is 1.2 dB @ (SE=0.25)/2.4 dB @ (SE=0.375) among overloading factor of 100%, 200% and 300%. As shown in Figure 16, the dynamic range of SNR is 1.1 dB @ (SE=0.25)/2.4 dB @ (SE=0.375) among overloading factor of 100%, 200% and 300%.

- Observation 4: With non-orthogonal multiplexing and diversity gain, the performance of PDMA under different overloading factors enables robust grant-free transmission.

- 4 RB in TDL-A 30ns

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 13. BLER curves of PDMA with different overloading factor

- 12 RB in TDL-A 30ns

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 14. BLER curves of PDMA with different overloading factor

- 4 RB in TDL-C 300ns

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 15. BLER curves of PDMA with different overloading factor

- 12 RB in TDL-C 300ns

(a)SE = 0.25 bps/Hz per UE                                                    (b)SE = 0.375 bps/Hz per UE

Figure 16. BLER curves of PDMA with different overloading factor

LLS of PDMA Downlink

Simulation parameters for PDMA downlink are shown in Appendix A.4.

For PDMA scheme with 4RB shared resource in ETU channel with 3km/h, Figure 17 shows the throughput region of far UE and near UE. For the power ratio {0.6, 0.4}, {0.7, 0.3}, {0.8, 0.2} and {0.9, 0.1} (between far UE and near UE) and the dynamic MCS selection of QPSK and code rate set {0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8}, PDMA can get higher throughput for both far UE and near UE compared with OMA.

Figure 17 far UE throughput vs. near UE throughput

Observation 5: Compared with OMA, PDMA can get higher throughput for both UEs, with reasonable power ratio.

Conclusion

In this contribution, LLS results for PDMA uplink and downlink are presented. We have following observations:

For PDMA uplink

- Observation 1: The significant gain of PDMA over OMA is up to 5.1 dB;

- Observation 2: The gain of PDMA over OMA increases as the overloading factor and target spectrum efficiency increases.

- Observation 3: Owing to frequency diversity gain by distributed subcarriers allocation, the gain of PDMA over OMA increases as the number of PRB increases.

- Observation 4: With non-orthogonal multiplexing and diversity gain, the performance of PDMA under different overloading factors enables robust grant-free transmission.

For PDMA downlink

- Observation 5: Compared with OMA, PDMA can get higher throughput for both UEs, with reasonable power ratio.

- References

- R1-163383, “Candidate Solution for New Multiple Access”, CATT.

- R1-164247, “Performance of LLS of PDMA”, CATT.

- X. Dai, S. Chen, S. Sun, S. Kang, Y. Wang, Z. Shen, and J. Xu, “Successive interference cancelation amenable multiple access (SAMA) for future wireless communications,” in Proc. IEEE ICCS 2014, Nov. 2014, pp. 1–5.

- 36.213, “Physical layer procedures”, v13.1.1.

- 38.900, “Channel model for frequency spectrum above 6 GHz (Release 14)”, V0.2.0.

- Appendix

- A.1 simulation parameters for uplink

Table II: simulation parameters for uplink

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM |
| Channel coding | LTE Turbo |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | 4RB (0.72MHz), 12RB (2.16MHz) |
| Overhead | 2 DMRS symbols, no SRS, 144 available RE per RB for data transmission |
| Target SE per UE (bps/Hz) | 0.25 0.375 |
| TB size per UE and total number of resource elements shared for data transmission | For 4RB case (without CRC): 120 bits, 192 bits For 12RB case (without CRC): 408 bits, 624 bits |
| Overloading Scenarios | 100%: 200%: 200%: |
| BS antenna configuration | 2Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213[4]) |
| Pattern allocation | Fixed |
| Receiver type(the time of iteration) | Turbo-MPA (Inner iteration:3, Outer iteration:3) |
| SNR distribution of multiple UEs | Multi-user LLS with equal average SNR |
| Propagation channel | TDL-A(30ns), TDL-C (300ns) in TR38.900 [5] |
| UE velocity | 3km/h |
| Max number of HARQ transmission | 1 |
| Channel estimation | Ideal |

- A.2 Two types of OMA subcarrier allocation

There are two types of subcarrier allocation for OMA, as shown in figure 18. One is localized and the other is distributed.

As for localized, the subcarrier allocation of each UE is in turn continuous over the whole transmit bandwidth.

As for distributed, the subcarrier allocation of each UE is discrete by interval which is the number of total UEs over the whole transmit bandwidth.

Figure 18 Localized and distributed subcarrier allocation for OMA

- A.3 MCS selection of PDMA and OMA

Table III: MCS Selection of PDMA and OMA

| PDMA |  | OMA |  | PRB | TB size | SE per UE |
| --- | --- | --- | --- | --- | --- | --- |
| Overloading factor | MCS | Number of UEs | MCS |  |  |  |
| 100% | QPSK0.5 | 4 | QPSK0.5 | 4PRB | 120 | 0.25 |
|  | QPSK0.75 | 4 | QPSK0.75 |  | 192 | 0.375 |
| 200% | QPSK0.5 | 8 | 16QAM0.5 |  | 120 | 0.25 |
|  | QPSK0.75 | 8 | 16QAM0.75 |  | 192 | 0.375 |
| 300% | QPSK0.5 | 12 | 64QAM0.5 |  | 120 | 0.25 |
|  | QPSK0.75 | 12 | 64QAM0.75 |  | 192 | 0.375 |
| 100% | QPSK0.5 | 4 | QPSK0.5 | 12PRB | 408 | 0.25 |
|  | QPSK0.75 | 4 | QPSK0.75 |  | 624 | 0.375 |
| 200% | QPSK0.5 | 8 | 16QAM0.5 |  | 408 | 0.25 |
|  | QPSK0.75 | 8 | 16QAM0.75 |  | 624 | 0.375 |
| 300% | QPSK0.5 | 12 | 64QAM0.5 |  | 408 | 0.25 |
|  | QPSK0.75 | 12 | 64QAM0.75 |  | 624 | 0.375 |

- A.4 simulation parameters for downlink

Table IV: simulation parameters for downlink

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM |
| Channel coding | LTE Turbo |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | 4RB (0.72MHz) |
| Overhead | 2 PDCCH symbols, 1 CRS ports for TM1, 138 REs per RB for data transmission |
| Target spectral efficiency | MCS: dynamic, QPSK and code rate set {0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8} Overloading factor: 2 UEs with PDMA pattern matrix in Table II Far UE occupies the first 4 columns and near UE occupies the last 4 columns. |
| BS antenna configuration | 2 Tx |
| UE antenna configuration | 2 Rx |
| Transmission mode | SFBC(TM2) |
| SNR distribution of multiple UEs | Fixed gap {0, 10} dB between UEs |
| Number of UEs | 2 UEs |
| SNR of the reference UE | 0dB |
| Power allocation between UEs | Power ratio: {0.6, 0.4}{0.7, 0.3}{0.8, 0.2}{0.9, 0.1} (between far UE and near UE) |
| Receiver type(the time of iteration) | Turbo-MPA (Inner iteration:3, Outer iteration:3) |
| Propagation channel & UE velocity | ETU 3km/h |
| Max number of HARQ transmission | 1 |
| Given BLER level (to calculate sum rate region) | 0.1 for 1 transmission |
| Channel estimation | Ideal |