3GPP TSG RAN WG1 Meeting #86                                             R1- 168072

Gothenburg, Sweden, 22nd-26th, August 2016

Agenda Item:		   7.2.5.1

Source:		   National Taiwan University

Title:		 Design Issues of MUST with HARQ

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss and present the effects of HARQ (for both the near and the far UEs) on MUST. Both error detection for HARQ and code word level SIC are considered. It is well known that HARQ can mitigate the errors caused by delayed and inaccurate CQI feedback. With HARQ, the MCS/power ratio may be aggressively selected such that even though the error probability of the first transmission of a packet may be high, the overall throughput can be improved. In this case, SIC at the near UE may fail due to aggressive MCS selection of the far UE. As a result, both the near and the far UEs may need retransmissions, and their pairing will become complicated. For example, we may need to consider the situation that the near UE may be paired with a different far UE at retransmissions, and may not be able to accumulate signals to successfully decode and cancel any of the far UEs paired with it.

- System Design Issues

In this section, we describe the design issues when HARQ is considered for MUST.

- Transmission modes for MUST

In this study, we focus on the code level SIC, i.e., the near UE decodes its paired far UE’s signal, and, if the decoding is successful, re-encodes the far UE’s signal and subtracts it from the received signal. The Block diagram is shown in Fig. 1.
If the near UE cannot decoder its paired far UE’s signal, it will not perform SIC, and will store the signal in its memory and wait for the next retransmission. Depending on whether SIC can be done, there will be many cases. Fig. 4 and Fig. 5 show the block diagrams of different situations. At a retransmission, the near UE will first check if it has successfully decoded the paired far UE before. If so, it can use the decoded data in its memory to re-encode and perform SIC. If not, it will have to combine (using Chase combining) and decode the paired far UE and perform SIC.

- Scheduling methodology

Once the CSI reported from the UEs are available, the eNB conducts resource allocation to its served UEs by using the proportional fair (PF) scheduling algorithm for MUST, as follows. For simplicity, the multiplexing order of superposition coding is assumed to be two with only one layer per UE. For step 1 to step 6 see [3]

- Start from the first subband

- Select one of the pair of users as (UE1, UE2) from the serving cell.

- Select one of the transmission power sets from the predetermined multiple transmission power sets for paired NOMA UEs where the predetermined transmission power sets (α1, α2) are, for example, (0.05, 0.95), (0.1, 0.9), …, (0.45,0.55), with a constraint of the total transmission power of 1 (α1+α2 =1.). Far UEs have lower channel gain needs higher power.

- Calculate the scheduling SINRs for NOMA using the reported CQI assuming OMA and the power set of (α1, α2). Computation of scheduling SINR for NOMA UEs is based on an approximation of the received SINR after power-domain multiplexing as follows.

  - Scheduling SINR of NOMA for UE1 with IC: α1×CQIUE1

  - Scheduling SINR of NOMA for UE2 without IC:  α2 /(α1 + 1/ CQIUE2)

We note that UE1 and UE2 are selected such that CQIUE1 is always higher than CQIUE2. We also assume that transmission power, α1, for UE1 is always lower than that, α2, for UE2. Then, UE1 is assumed to apply the perfect interference cancellation to remove the interference from the other user, while UE2 directly decodes its own signal.

- Calculate the multi-user PF metric by l∈Uτ(l|U, P)Tα(l)l∈Uτ(l|U,P)Tα(l)

l∈Uτ(l|U, P)T(l)l∈Uτ(l|U,P)T(l) where U denotes the candidate user set. Terms (𝑙) and (l |U, P) denote the average throughput and instantaneous throughput of UEl, respectively. P denotes the allocated power sets.

- Repeat Steps 2-5 with all of the combinations of candidate transmission power sets (α1, α2) and user sets (UE1, UE2).

- Select one of the MCS sets from MCS table for (UE1, UE2). (MCSUE1, MCSUE2) are, for example, (8, 2), (7, 2), …, (0,0). See the MCS table in Fig.2.

- Repeat Steps 2-7 with all of the MCS sets (MCSUE1, MCSUE2) and user sets (UE1, UE2).

- Select the transmission power, user pairs and MCS sets with the highest PF metric in Step 8. This pair become temporary near/ far user pair.

- Decide who is final near user and far user:  Base on temporary near user and far user
Step11: Check temporary (near/far) user needs retransmit or not.

Step12: If needs retransmit check last time was (near/far) user and its partner (near or far) needs retransmit or not.

Step13: If its partner also need retransmit decide them be final near and far user.

Step14: If not both of them (near/far+ partner last time) need to retransmit decide temporary near user and far user be final user pair

- Conduct the Steps from 1 to 14 for the remaining subbands.

When step 9 is finished, transmission power sets are optimized in each subband. But for code level SIC near UEs need to store far UEs’ signals to wait for next retransmission, if the near UE is not able to decode the far UE’s signal this time. We want reduce the memory loading in UEs. When near UE and far UE are both nearby cell edge, they may fail in high probability. If near UE SIC failed in first transmission round and its partner (far UE) also failed, by step 11 to 14 we can make more use of the information left in first round. Then near user can complete SIC with higher SNR after chase combining.

- Simulation Evaluation

The simulation parameters are shown in Table B-I in the Appendix B.

Fig 1: Performance comparison between fixed near-far UE pairing and flexible pairing (which can be changed during retransmissions).

Although our scheduling may have some loss by fixing the UE pairs when both of them need to retransmit. But in our way when near user and far user are both nearby cell edge, they will have better performance than the scheduling without fixing UE pairs. As shown in Fig I, we can observe that the performance of fixed pairing and flexible pairing are similar. We suppose that this fixed pairing scheduling will improve cell edge UE performance.

- Conclusion

In this contribution, we presented MUST with HARQ used for both the near the far UEs. The effects of HARQ on MUST and the related design issues were discussed.

- References

- 3GPP TR 36.866 v12.0.1

- 3GPP, R1-157191,  MediaTek, “Further System-Level Analysis for MUST,” Anaheim, USA, Nov. 5 – 9 2015

- 3GPP, R1-155931, NTT DOCOMO, “System-level evaluation results for downlink multiuser superposition schemes,” Malmö, Sweden, Oct. 5 – 9 2015

- 3GPP, R1-157352, NTT DOCOMO, “System-level evaluation results for downlink multiuser superposition schemes,” Anaheim, USA, Nov. 5 – 9 2015

- YuyaSaito, Yoshihisa Kishiyama, AnassBenjebbour, TakehiroNakamura, AnxinLi, and Kenichi Higuchi, ”Non-Orthogonal Multiple Access (NOMA) for Cellular Future Radio Access”, IEEE 77th Vehicular Technology Conference (VTC Spring), p.p1 –5, 2013.

- Daniele Tarchi, Romano Fantacci, Cagri Tanriove, Novel link adaptation for TETRA cellular systems, INTERNATIONAL JOURNAL OF COMMUNICATION SYSTEMS, 2009

- Fan Chen, Chen Meiya, Su Lijun, Zhang Xin, Yang Dacheng, RESEARCH ON THRESHOLD ADJUSTMENT ALGORITHM IN ADAPTIVE MODULATION AND CODING, The 17th Annual IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC'06).

- Appendix A:

Figure 1 –Code Level SIC Block diagram

Figure 2 –MCS table

- Appendix B: Simulation assumption

Table B-I. Simulation parameters

| Layout | Hexagonal grid, 3 sectors per site, 1 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | 128.1+37.6log10(r)dB |
| Penetration loss | 0dB |
| Fading | Rayleigh fading |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 15 m |
| UE antenna Height | 1.5 m |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 4Tx UE: 1Rx |
| Number of UEs per cell | 30 |
| minimum distance from macro-cell to UEs | 35 m |
| Traffic model | Full buffer (30 UEs/cell) |
| UE receiver | - R-ML/ideal SIC for inter-user interference |
| UE noise figure | 10 dB |
| UE speed | 3 km/h |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Chase combining |
| Maximum number of retransmission | 4 |
| Granularity of CSI feedback | 5 msec |
| Round trip delay (HARQ) | 8 msec |
| Codebook | LTE Rel. 8 |
| CQI quantization | Yes |
| Power ratio sets | 10 power sets, i.e. (α1, α2), where α1=0.05,0.1,….,0.45; power ratios: (α1, α2) for near and far UEs |
| MCS sets | Near UEs: QPSK(1/2),QPSK(3/4),16QAM(1/2),16QAM(2/3), 16QAM(3/4),16QAM(7/8),64QAM(2/3),64QAM(3/4),64QAM(5/6) Far UEs: QPSK(1/2),QPSK(3/4),16QAM(1/2),16QAM(2/3) |
| Number of superposed signals in superposition transmission | 2 |
| Receiver impairment modeling for demodulation | ideal CRS-based channel estimation |

Figure 3 – Simulation system block diagram

- Appendix C: Near and Far receiver

Figure 4 – Near user receiver data flow diagram

Figure 5 – far user receiver data flow diagram