3GPP TSG RAN WG1 Meeting #86	R1-168485

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	8.1.7

Source: 	Sony,  u-blox, Sierra Wireless

Title:	Battery Lifetime Model for NR

Document for:	Discussion / Decision

# Introduction

Battery modelling / Traffic characteristics

Battery models and traffic characteristics are defined for battery lifetime evaluation in link level study. The different characteristics relate to different use cases, where each use case has an appropriate battery model (e.g. utility meters are assumed to have a battery that is reasonably unconstrained, whereas wearable devices have a battery that is constrained in terms of maximum pulse and continuous current).

# Battery Lifetime Text Proposal

A discharge-unconstrained battery is modelled as having an energy capacity. After self-discharge, the battery is considered to have a nominal energy capacity of 5Wh at 3.6V.

The power consumption of a proposal shall be stated. A backoff to battery lifetime, based on Peukert’s Law (k=1.1, rated discharge time, H=10 hours) needs to be applied if the peak current used is greater than 140mA.

Discharge-constrained batteries are modelled in Table 1.

Table 1 – Battery Models for Discharge-constrained Batteries

| Characteristic | Battery Model A(Coin cell) | Battery Model B (Rechargeable) |
| --- | --- | --- |
| Nominal voltage | 3.0V | 3.6V |
| Nominal capacity | 1000mAh (ambient temperature) | 200mAh (ambient temperature) |
| Maximum continuous current | 6mA | 100mA |
| Pulse capability | 30mA | 100mA |

The power consumption model used for mMTC evaluation is provided in Table 2

Table 2 UE power consumption model for mMTC battery life evaluation

| Operating mode |  | Value | Notes |
| --- | --- | --- | --- |
| Transmit | Loss from PA to radiated power | FD-FDD: 3dB HD-FDD / TDD: 1.5dB | Accounts for switches, duplexers, insertion loss |
|  | PA efficiency | Integrated PA: 45% External PA: 50% |  |
|  |  | 60mW | Other RF and baseband circuitry |
| Receive |  | 60mW | RF and baseband circuitry |
| Sleep |  | 3mW | Corresponds to maintaining accurate timing by keeping RF frequency reference active. |
| Standby |  | 0.015mW | Common assumption. |

The protocol flow assumed for the proposal should be stated.

MAR-based traffic characteristics

MAR characteristics are defined in Table 3. For MAR traffic characteristics discharge-unconstrained batteries are used for battery lifetime evaluation purposes. Battery lifetimes are reported in the form of Table 3.

Table 3 – Traffic characteristics for unconstrained batteries

|  | Battery life/years (1 year = 365 days) for three coverage levels |  |  |
| --- | --- | --- | --- |
| Packet size, reporting interval combination | MCL = 144dB | MCL = 154dB | MCL = 164dB |
| 50 bytes, 2 hours |  |  |  |
| 200bytes, 2 hours |  |  |  |
| 50 bytes, 24 hours |  |  |  |
| 200 bytes, 24 hours |  |  |  |

General mMTC Traffic Characteristics

General mMTC traffic characteristics are defined in Table 4. For each traffic characteristic, battery lifetime is calculated based on a discharge-constrained battery model. The battery models and target KPIs to be used for the traffic characteristics are defined in Table 5.

Table 4 – Traffic characteristics for general mMTC traffic

| Traffic Characteristic | Traffic Characteristic Description |
| --- | --- |
| Wearable | MT 100 bytes + 20 byte ACK. 1 message per 2 hours. |
| Precision Tracking | MO 100 bytes + 20 byte ACK. 1 message per minute. |
| Real-time data | MO 100 bytes + MT 100 bytes every 40ms. |
| Bulk data | MT 500K byte + 20K bytes every 4 seconds |

Table 5 – Battery models and target KPIs for general mMTC traffic characteristics

| Traffic Characteristic | Battery model | Battery life | MCL | Latency |
| --- | --- | --- | --- | --- |
| Wearable | A | 1 year | 144dB | 5 sec |
|  | B | 2 weeks | 144dB | 5 sec |
| Precision tracking | B | 24 hours | 144dB | 100 ms |
| Real time data | B | 1 hour | 144dB | 40 ms |
| Bulk data | B | 1 hour | 134dB | 4 sec |