3GPP TSG-RAN WG1 #86  													                           R1-166527

Gothenburg, Sweden, 22 - 26 Aug 2016

Source: 	Intel Corporation

Title:	Remaining Details of open-loop techniques

Agenda item:	    7.2.4.3

Document for:	Discussion and Decision

# Introduction

In the RAN 1 #85 meeting the specification enhancement on DMRS based semi-open-loop transmission was agreed [1], the following areas are included.

- CSI reporting scheme with partial PMI feedback, and/or

- Transmission method with transmit diversity and/or precoder cycling, with possible enhancements to precoder bundling

- Associated enhancements to DMRS if required

In this contribution we provide our views on the above specification enhancements.

# Discussion

Since Rel-8, the open-loop technique has been adopted. In TM3, when rank v is greater than 1, the precoder for PDSCH with large delay CDD can be changed every v subcarriers. For every v contiguous subcarriers, the same random precoders can be used to form v beams. Within these subcarriers, the CDD matrix can change the mapping of v intermixed data symbols to the v beams, which makes the channel qualities of the TB close to each other. When rank v is equal to 1, the transmit diversity is used, where the SFBC is applied to a pair of 2 or 4 consecutive REs. As the TM3 is CRS based, it cannot support open-loop scheme with more than 4 antenna ports. Also DMRS based demodulation becomes more efficient with larger number of antennas. Therefore the DMRS based open loop scheme can be one way to implement the open-loop technique for more antenna parts case, such as eFD-MIMO.

## 2.1 Transmission Method

For rank 1 transmission, either transmit diversity or precoder cycling can be utilized to implement the beam sweeping. However for transmit diversity, there may be the orphan RE issue. The SFBC can be applied to a pair of 2 or 4 consecutive REs. Hence if there are some orphan RE(s), the performance may be impacted. Therefore compared to transmit diversity, the precoder cycling may be more robust, which is slightly preferred.

For rank>1 transmission, the precoder cycling can be applied to enable the beam sweeping. As observed in [2], compared to RB level precoder cycling, the performance for the RE level precoder cycling with large delay CDD can bring in better performance, especially for lower SINR case. Further from [3] it can be observed that significant performance gain can be achieved from RB level precoder cycling with per subcarrier large delay CDD, compared to that without large delay CDD. The link level simulation results are also shown in the appendix. Hence generally precoder cycling can be a promising method to enhance the DMRS. For rank 1 transmission, RE level precoder cycling can be applied. For rank>1 transmission, RB level precoder cycling with large delay CDD can be used.

Proposal 1: for rank 1 transmission, compared to transmit diversity, per RE precoder cycling based transmission is slightly preferred as there would be no orphan RE issue; for rank>1 transmission, to obtain better performance, RB level precoder cycling with large delay CDD based data transmission should be applied.

## 2.2 CSI reporting scheme

For the semi-open-loop based DMRS, the precoder can be given as W=W1W2, where W1 indicates the long-term and wideband precoder and W2 denotes the short-term precoder which can be one beam selected from a set of beams. With the help of W1, the long-term transmission direction can be determined, so that the beam accuracy can be confirmed. With regard to W2, as it could represent the short-term sub-space. Based on beam sweeping operation, it would be possible to cover most or all sub-space. As the channel may change very fast as a result of UE’s mobility, the accuracy of W2 feedback cannot be confirmed. Meanwhile to feedback all of the swept matrix W2, the overhead of the uplink control information can get increased. Therefore the feedback of W2 is not necessary.

When deciding RI and CQI, after determining the long-term transmission direction, which is indicated by W1, the UE could assume the precoder cycling is applied for W2. Then the reported RI and CQI can reflect the channel state with diversity gain. Thus the RI and CQI can be achieved from averaging of different sub-space of long-term transmission direction.

Proposal 2: with regard to accuracy and overhead of W2 feedback, the CSI reporting can include feedback of the long-term transmission direction as well as the RI and CQI estimated based on precoder cycling.

# Conclusions

In this contribution we have provided our views on specification enhancements for DMRS based semi-open loop transmission. From the discussion, we have the following proposals.

Proposal 1: for rank 1 transmission, compared to transmit diversity, per RE precoder cycling based transmission is slightly preferred as there would be no orphan RE issue; for rank>1 transmission, to obtain better performance, RB level precoder cycling with large delay CDD based data transmission should be applied.

Proposal 2: with regard to accuracy and overhead of W2 feedback, the CSI reporting can include feedback of the long-term transmission direction as well as the RI and CQI estimated based on precoder cycling.

# References

- 3GPP Chairman's Notes RAN1#85

- R1-164433, “Discussion on DMRS based OL and Semi-OL Transmission”, Qualcomm

- R1-164151, “Evaluation of open loop schemes”, Intel

# Appendix – link level simulation results

Table 1 Simulation Assumptions

| Parameters | Value |
| --- | --- |
| System bandwidth | 10MHz (50RBs) |
| Antenna configuration | 4/8/16 ports (other values of the number of ports are not precluded) 2 Rx antennas (cross-polarization) |
| Channel model | 3D UMa 500m |
| UE velocity | 30 km/h, 60 km/h, and/or 120km/h |
| Carrier Frequency | 2GHz |
| Overhead | 2 PDCCH symbols per sub-frame |
| Modulation, code rates | Based link adaptation |
| HARQ | HARQ with max 4 transmissions |
| Number of allocated PRBs | 4 PRBs |
| CSI feedback | Open loop with CDD: TM3 like feedback Open loop without CDD: TM9 like feedback Close loop: TM9 Semi-open loop: TM9 like feedback; Only feedback i1,1 |
| Feedback delay | 5 msec |
| Feedback periodicity | 5 and/or 10 msec |
| Channel estimation for demodulation and CSI estimation | Non-ideal |
| Receiver | MMSE-IRC |
| Rank adaptation | ON |
| Baseline | TM 9 based closed loop transmission |

Figure 1: 4Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

Figure 2: 8Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

Figure 3: 16Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)