3GPP TSG RAN WG1 Meeting #86							R1-167369

Gothenburg, Sweden 22nd - 26th August 2016

Source:	NTT DOCOMO, INC.

Title:	sPDSCH for shortened TTI

Agenda Item:	7.2.12.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#72, the new Work Item on “shortened TTI and processing time for LTE” was approved [1]. The detailed objectives for shortened TTI are:

For Frame structure type 1: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 2-symbol sTTI and 1-slot sTTI for sPDSCH/sPDCCH

- Specify support for a transmission duration based on 2-symbol sTTI, 4-symbol sTTI, and 1-slot sTTI for sPUCCH/sPUSCH

  - Down-selection is not precluded

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

For Frame structure type 2: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 1-slot sTTI for sPDSCH/sPDCCH/sPUSCH/sPUCCH

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

In addition, following agreements on channel design for sPDSCH and UE behavior in case of being scheduled with legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same subframe on the same carrier were achieved at the RAN1 #85 meeting [2] and RAN1 #84bis meeting [3].

Agreements: - For sPDSCH based on a CRS based transmission scheme the maximum number of supported layers is 4 - For sPDSCH based on a DM-RS based transmission scheme shall be down-selected among the following options - the maximum number of supported layers is 2 - the maximum number of supported layers is 4 - the maximum number of supported layers is 8 - FFS for sPDSCH based on a DM-RS based transmission scheme it is recommended to increased PRB bundling size compared to PDSCH for at least sTTI lengths shorter than 1-slot Agreements: - A UE is expected to handle the following cases in the same carrier in a subframe - Receiving legacy TTI non-unicast PDSCH (except FFS for SC-PTM) and short TTI unicast PDSCH - Receiving legacy TTI non-unicast PDSCH (except FFS for SC-PTM) and legacy TTI unicast PDSCH(s) - FFS between: - Alt 1: A UE is not expected to receive legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier - Alt 2: If the UE is scheduled with legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier, then it may skip the decoding of one of them (FFS rules for determining which one) - Alt 3: A UE is expected to receive legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier - FFS UE behaviour in case of being scheduled with legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously with legacy TTI non-unicast PDSCH (except FFS for SC-PTM) on the same carrier - A UE can be dynamically (with a subframe to subframe granularity) scheduled with legacy TTI unicast PDSCH and/or (depends on outcome of FFS above) short TTI PDSCH unicast

In this contribution, we provide our views on the above aspects. sPDCCH design for shortened TTI can be found in our companion contribution [4].

- PDSCH in a short-TTI (sPDSCH)

  - Channel structure

According to the WID, sTTI lengths for downlink of 2 symbols and 1 slot are to be supported. Depending on the number of OFDM symbols occupied by legacy PDCCH and whether CRS-based sPDCCH can be transmitted in the legacy PDCCH region, different sTTI patterns/positions within one subframe exist. Fig. 1 gives one example for the 2-symbol and 1-slot sTTI pattern for downlink transmission assuming that sPDCCH can be transmitted in legacy PDCCH region. Analysis on the sTTI patterns/positions can be found in our companion contribution [4].

Fig. 1: 2-symbol and 1-slot sTTI pattern within one subframe

sPDSCH can be realized by scaling TB size according to the given TTI length. So, large part of the PDSCH framework except for RS can be kept as it is.

Proposal 1:

- sPDSCH can be realized by scaling TB size according to the given TTI length.

  - RS structure

For sTTI transmission, it was agreed to support both CRS based TMs and DMRS based TMs. For CRS based TM, existing CRS port #0-#3 can be re-used to support the maximum number of 4-layer transmissions for both 2-symbol and 1-slot sPDSCH.

For DMRS based TM, the maximum number of supported layers should be 4, at least for 1-slot sPDSCH, in order to achieve the comparable performance with CRS based TM. In the following, DMRS design for 1-slot and 2-symbol sPDSCH will be discussed.

For 1-slot sTTI, existing DMRS on antenna port #7-#10 located in last two symbols per slot can simply be re-used. However, it requires waiting time for eNB to start the channel estimation procedure. Therefore, it is beneficial to place DMRS in the earlier symbol(s) of one sTTI while avoiding the collision with CRS REs and legacy PDCCH REs. Figure 2 illustrates one possible example of new DMRS mapping. The 4 layers can be achieved by CDM in either time and/or frequency domain.

Fig 2: Example of DMRS design for 1-slot sPDSCH

For 2-symbol sTTI, to support 4-layer transmissions, the DMRS overhead becomes higher which will restrict the throughput performance. Two solutions can be considered to reduce the DMRS overhead. One is to allow DMRS sharing between sPDCCH and sPDSCH. This requires aligning the number of layers and precoder for sPDSCH and sPDCCH. The other solution is to define DMRS per PRB group, which consists of multiple PRBs. The flexibility in the number of PRBs per scheduling is limited in this case.

In summary, as the TTI length becomes shorter, the DMRS overhead becomes higher. If DMRS overhead is reduced, either the number of MIMO layers is limited, or the channel estimation performance is degraded. Therefore, a certain kind of restriction or performance degradation may be unavoidable. In order to alleviate this, it is important to support dynamic adaptation of TTI length between 1ms TTI and sTTI, i.e., the TTI length is dynamically adapted, e.g., per subframe/scheduling basis. Then, for TCP slow-start phase, sPDSCH is scheduled so that TCP congestion window rapidly extends. After the TCP slow-start phase, PDSCH is scheduled on a 1ms TTI so that higher spectrum efficiency is achievable by optimizing RS overhead and/or by utilizing higher-order MIMO. With the dynamic adaptation between 1ms TTI and sTTI, DMRS overhead would not be a critical concern. Otherwise, it is quite essential to optimize DMRS including higher-order MIMO to achieve high data rate using only sTTI.

Proposal 2:

- For CRS-based TMs, current CRS port #0-#3 can be re-used to support up to 4 layers for sPDSCH.

- For DMRS-based TMs, 4 layers should be supported at least for 1-slot sPDSCH.

  - It is beneficial to locate DMRS in the earlier symbols of sTTI.

  - More layers should be applicable by dynamic change of TTI length between sTTI and 1ms TTI.

- UE behavior on DL sTTI operation

In the current LTE, a UE does not support to receive more than one unicast PDSCH in the same subframe on one carrier, while a UE can support simultaneous reception of one unicast PDSCH and one non-unicast PDSCH with small message. This implies that the UE may be capable of simultaneous reception of one unicast sPDSCH and one unicast PDSCH in the same subframe on the same carrier. It is beneficial for a UE to enable simultaneous reception of these two channels. The first reason is that the traffic requiring low latency can break in at the middle of a subframe by this, even if the normal traffic using 1ms TTI is scheduled for the UE in the subframe. The second reason is that the two transmissions have different HARQ/scheduling timeline and therefore, from eNB scheduler point of view, handling those two transmissions so that they do not collide each other would be non-efficient. Therefore, it should be allowed to schedule unicast sPDSCH during the time-interval where unicast PDSCH is already scheduled.

It is expected that at least the advanced UEs have this capability. Therefore, a per UE capability signaling of simultaneous reception of legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same subframe in the same carrier needs to be introduced to help eNB make the scheduling decision. For example, if the UE indicates to eNB that it is able to receive legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier, then the eNB can schedule these two traffic in the same carrier in a subframe to the UE. Else, eNB should not schedule these two traffic in the same carrier in a subframe to the UE.

Proposal 3:

- It is necessary to introduce a per UE capability signaling of simultaneous reception of legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same subframe in the same carrier.

If the UE does not indicate this capability to the eNB, eNB can assume that the UE can support simultaneous reception of legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same carrier in a subframe. In this case, the UE behavior should be well defined. Following three alternatives can be considered.

Alt. 1: UE can decode both legacy TTI unicast PDSCH and short TTI unicast sPDSCH in the same subframe in the same carrier under some conditions, except these conditions, UE should prioritize decoding short TTI unicast sPDSCH.

Possible conditions can be the received legacy TTI unicast PDSCH is SPS PDSCH and/or the unicast PDSCH is part of a random access procedure and/or the TBS/MCS of the unicast PDSCH is under a threshold and/or the total summed TBS of the unicast PDSCH and sPDSCH is under a threshold, etc. This alternative can be viewed as an extension of UE behavior on simultaneous reception of legacy TTI non-unicast PDSCH and legacy or short TTI unicast sPDSCH in one subframe on one carrier. Except above conditions, UE should always prioritize decoding short TTI unicast sPDSCH and can skip decoding legacy TTI unicast PDSCH. With this UE behavior, low latency traffic can be transmitted to UEs flexibly regardless of their capability.

Alt. 2: UE always prioritize decoding short TTI unicast sPDSCH.

If the eNB wants to prioritize legacy TTI unicast PDSCH, it should not schedule short TTI unicast sPDSCH in the same subframe in the same carrier when the low latency traffic arrives; Otherwise eNB should schedule short TTI unicast sPDSCH for the UE being served with legacy TTI unicast PDSCH. When the UE is aware of collision between legacy TTI unicast PDSCH and short TTI unicast sPDSCH, UE should skip decoding legacy TTI unicast PDSCH. This alternative also ensures the same understanding between eNB and UE on what traffic type (e.g., legacy-TTI traffic or low latency traffic) UE would decode and has small specification impact while lowers the UE capability especially for advanced UEs.

Alt. 3: left to UE implementation, UE can decide to decode both of the legacy TTI unicast PDSCH and short TTI unicast sPDSCH or decode one of them in the same subframe in the same carrier.

Unlike alternative 1 and 2, this alternative gives the final say to the UE. UE can decode both of them or either one of them. No common understanding between eNB and UE regarding which TTI is/are decoded is ensured, but eNB can defer which one UE must decode based on the ‘ACK’ feedback to the related sPDSCH and/or PDSCH. This alternative also keeps the benefits of supporting simultaneous reception of legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same subframe in the same carrier.

Proposal 4:

- If UE does not indicate the capability, eNB can schedule legacy TTI unicast PDSCH and short TTI unicast sPDSCH simultaneously in the same subframe in the same carrier.

- UE behavior in case of being scheduled with legacy TTI unicast PDSCH and sPDSCH in the same carrier in the same subframe should be defined.

- Conclusion

In this contribution we showed our views on PDSCH design for shortened TTI and made the following proposals.

Proposal 1:

- sPDSCH can be realized by scaling TB size according to the given TTI length.

Proposal 2:

- For CRS-based TMs, current CRS port #0-#3 can be re-used to support up to 4 layers for sPDSCH.

- For DMRS-based TMs, 4 layers should be supported at least for 1-slot sPDSCH.

  - It is beneficial to locate DMRS in the earlier symbols of sTTI.

  - More layers should be applicable by dynamic change of TTI length between sTTI and 1ms TTI.

Proposal 3:

- It is necessary to introduce a per UE capability signaling of simultaneous reception of legacy TTI unicast PDSCH and short TTI unicast PDSCH in the same subframe in the same carrier.

Proposal 4:

- If UE does not indicate the capability, eNB can schedule legacy TTI unicast PDSCH and short TTI unicast sPDSCH simultaneously in the same subframe in the same carrier.

- UE behavior in case of being scheduled with legacy TTI unicast PDSCH and sPDSCH in the same carrier in the same subframe should be defined.

References

- RP-161299, “New Work Item on shortened TTI and processing time for LTEs,” Ericsson.

- 3GPP RAN1#85, Chairman’s note.

- 3GPP RAN1#84bis, Chairman’s note.

- R1-167368, ‘sPDCCH for shortened TTI,’ NTT DOCOMO, INC., August 2016.