TSG-RAN WG1 #86	R1-167451

Göteborg, Sweden, Aug 22 – 26, 2016

Source:	Ericsson, Telstra

Title:	Channel model for extreme rural scenario

Agenda Item:	8.1.7

Document for:	Discussion and Decision

# Introduction

TR 38.913 on "Scenarios and Requirements for Next Generation Access Technologies" defined several deployment scenarios and evaluations assumptions for the three families of usage scenarios, namely, eMBB, mMTC and URLLC. This contribution refers to the deployment scenario 6.1.7 and listed attributes therein. Specifically it aims to provide a simple channel model for evaluating coverage and throughput performance. A companion paper [2] addresses the path loss model.

# Channel modelling for extreme ranges

The radio channel in rural scenarios has been well studied through measurements and captured in channel models such as the IMT-Advanced channel model [3] or the channel model in 3GPP TR 38.900 [4]. However the measurements behind these models have not been done at extreme ranges. To increase the confidence of channel modelling for extreme ranges a set of new measurements for remote rural sites were performed and reported in [1]. These measurements addressed the path loss and rms delay spread.

One of the learnings from [1] is that the rms delay spread does not seem to change as a function of propagation distance, rather an average delay spread of around 100 ns was found for the full distance range. This gives some confidence in extrapolating the channel behaviour experienced at shorter ranges to long distance.

To support link level simulations and simpler single- and multi-cell system simulations, it would be beneficial to combine the path loss model proposal in [2] with a low complexity channel model such as the TDLs or CDLs specified in [4]. The TDL/CDLs can be adjusted to the measurement data through the delay and angular scaling procedure defined in [4]. It is reasonable to use a 100 ns delay scaling as this was observed in the measurements. No angular spread measurements are available at extreme ranges but it seems likely that the angular spreads will be quite small in such a scenario due to the long range and high effective antenna height in combination. Therefore, the following is proposed:

Proposal 1: For link level modelling and simple system-level modelling, use TDL-A and CDL-A with the following delay and angle scaling:

- DS = [100 ns]

- ASD = [1°]

- ASA = [30°]

- ZSD = [0.1°]

- ZSA = [1°]

- The absolute propagation delay τ0=c0d is added to all tap delays

- For system-level simulations, the mean AOD,ZOD,AOA,ZOD for each link are determined from the geometry assuming LOS propagation

# Conclusion

The following proposals are made:

Proposal 1: For link level modelling and simple system-level modelling, use TDL-A and CDL-A with the following delay and angle scaling:

- DS = [100 ns]

- ASD = [1°]

- ASA = [30°]

- ZSD = [0.1°]

- ZSA = [1°]

- The absolute propagation delay τ0=c0d is added to all tap delays

For system-level simulations, the mean AOD,ZOD,AOA,ZOD for each link are determined from the geometry assuming LOS propagation

# References

- R1-166559, “Measurements of extreme rural scenarios”, Telstra, Ericsson, Göteborg, Sweden, 22-26 Aug 2016.

- R1-167452, “Path loss model for extreme rural scenario”, Ericsson, Telstra, Göteborg, Sweden, 22-26 Aug 2016.

- Report ITU-R M.2135-1 “Guidelines for evaluation of radio interface technologies for IMT-Advanced” (12/2009).

- 3GPP TR 38.900 “Study on channel model for frequency spectrum above 6 GHz”, v14.0.0.