TSG-RAN WG1 #86	R1-167454

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Ericsson, Telstra

Title:	Link budget for extreme rural scenario

Agenda Item:	8.1.7

Document for:	Discussion and Decision

# Introduction

In RAN1#85, the following was agreed

- Recommended requirements in TR 38.913 on extreme long range:

  - [Traffic density: [380-500kbps/km²]

  - User Experienced Data Rate: up to [2]Mbps while stationary and [384kbps] while moving]

- Aiming to have link budget analysis in RAN1#86:

  - Channel model for extreme rural scenario

  - Target data rate DL 2 Mbps (stationary) and 384 kbps (moving) with corresponding BS Tx power, BS antenna height, BS antenna gain, BS receiver NF

  - Alternative DL data rate with corresponding BS Tx power, BS antenna height, BS antenna gain, BS receiver NF

  - UL data rate with corresponding BS Tx power, BS antenna height, BS antenna gain, BS receiver NF

  - Decision on remaining parameters in the SLS evaluation table

In [1], we provide a channel model for the extreme range scenarios together with a path loss model in [2] based on measurements reported in [5]. Additionally, [3] gives a model for traffic. In this contribution we provide link budget analysis for the extreme long range scenario.

# Modeling assumptions

The proposal in [4] contains some remaining undecided parameters and to be able to make an link budget analysis, we make the following assumptions and parameter proposal on top of those listed in [4]:

Table 1 Modeling parameters for link budget

| Parameter |  | BS |  | UE |
| --- | --- | --- | --- | --- |
| TX power |  | 49 dBm (80 W) |  | 23 dBm |
| Number of antenna elements |  | 256 Tx/Rx |  | 2 Tx/Rx |
| Antenna element gain |  | 8 dBi |  | 0 dBi |
| Noise figure |  | 3 dB |  | 9 dB |

The path gain is obtained from the measurement analysis in [2], which results in free space propagation plus a clutter term and can be described as:

GpdB=-92.45+20log10f+20log10d+20

Where f  is the carrier frequency in GHz and d is the distance in kilometers. The term 20 dB is the measurement fitting term [4], which models the loss due to environment clutter. Note that the path gain equation is not explicitly dependent on the BS antenna height, however, for the equation to hold a certain BS antenna height is required and it is also required that the UE is positioned adequately with respect to the BS. In the measurements used to confirm the model, the relative height difference between BS and UE ranged from 60 to 600 meters.

# Link budget results

In table 2, the link budget using default parameter settings from Table 1 are shown.

Table 2 Link budget for extreme rural scenario – default (maximal) parameter values

The experienced downlink and uplink user data rate at 400 km distance is 31.2 and 0.58 MBps respectively which exceeds the downlink target data rate of 2 MBps.

Observation: From a link budget perspective, the requirement on the downlink data rate for the extreme rural with extreme long range can be met with the assumptions (e.g antenna size, TX power,..) that are tentatively agreed in the TR.

In Table 2 results, two omni antennas have been used at the UE. If more advanced antennas are used at the UE that also allows for directional transmission (beamforming) using 4 directional antenna elements, then the uplink can also sustain more than 2 MBps at 400 km range.

Even if the BS use a moderate size array with 8 antenna elements per polarization, and with smaller TX power than in Table 2 (46 dBm), the requirement of 2 MBps is met in the downlink at 400 km range.

Table 3 Link budget for extreme rural scenario –small BS array and lower TX power

In Table 4, we also studied the 3.4 GHz case. To meet the requirements at 400 km, there is a need for more antenna gain and it can be seen that 64 BS antenna elements per polarization and 8 UE antenna elements is sufficient.

Table 4 Link budget for extreme rural scenario for 3.4 GHz case

# Discussion

It has been shown with link budget analysis that the requirements of throughput at extreme range can be achieved with current NR assumptions at both 0.7 and 3.4 GHz carrier frequency. Note that even with very high sites there will be Fresnel zone obstruction by the curvature of the earth and to obtain coverage beyond that will depend upon how much refraction loss can be tolerated given the MAPL.  Ducting is the other possibility for overcoming the radio horizon and of course airborne devices will potentially operate at very long distances.

It should also be noted that while it is possible to connect to the NR system at extreme ranges,  95% of the user population is within 50 km distance from the BS site [3] and hence, this should thus not be seen as an area covering solution.

Proposal: NR should be designed to be able to operate over extreme ranges (~400 km)

# References

[1] R1-167451, “Channel model for extreme rural scenario”, Ericsson, Telstra, Göteborg, Sweden, 22-26 Aug 2016.

[2] R1-167452, “Path loss model for extreme rural scenario”, Ericsson, Telstra, Göteborg, Sweden, 22-26 Aug 2016.

[3] R1-167453, “User and traffic distribution for extreme rural scenario”, Ericsson, Telstra, Göteborg, Sweden, 22-26 Aug 2016.

[4] R1-165485, “WF on Extreme Long Range KPI Evaluation”, Qualcomm

[5] R1-166559, “Measurements of extreme rural scenarios”, Telstra, Ericsson, Göteborg, Sweden, 22-26 Aug 2016.