3GPP TSG RAN WG1 Meeting #86	R1-167566

Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:   8.1.4.1

Source:	  InterDigital Communications

Title:	          Performance Evaluation of Puncturing Schemes for Polar Codes

Document for:	  Discussion

# 1	Introduction

In the RAN1 meeting #85, the BLER performance of major candidate channel codes (TBCC, Turbo, LDPC, Polar) was examined. This effort will continue along with channel coding simulation data sharing [1]. Nevertheless, it is observed [2] that at least in AWGN channels, all candidate channel coding schemes show comparable link performance at large information block length.

Besides the BLER performance, the implementation complexity, latency and flexibility are other aspects of evaluating channel coding schemes [3].

As a major candidate channel code, polar code shows very good BLER performance, especially at small information block lengths. However, the coded block length of polar code is a power of 2 due to its structured encoding nature. Hence, the puncturing on the polar-encoded block is necessary to achieve flexible coding rates.

In this contribution, we investigate two puncturing schemes of polar codes. The performance evaluation of both puncturing schemes are provided.

# 2	Discussion

## 2.1 	Polar Codes

We consider a N,K polar code, where K is the information block length and N is the coded block length. Here, the value N is a set as a power of 2, i.e., N=2n for some integer n. The generator matrix of the polar code can be expressed by GN=BNF2⨂n, where BN is the bit-reversal permutation matrix, (∙)⨂n denotes the n-th Kronecker power and F2=1011.

With the well-defined polar encoder, the main design (or construction) of polar code is the mapping of the K information bits to the N input bits of polar encoder. In principle, the K information bits should be put on the K best (or most reliable) bit channels, and the remaining N-K input bits not mapped from the information bits are called frozen bits. There are several ways to determine the reliability of bit channels [5], e.g., the Bhattacharyya bounds, the Monte-Carlo estimation, the full transition probability matrices estimation, and the Gaussian approximation, etc.

The decoding algorithms of polar codes include Successive Cancellation (SC) decoding [4], Successive Cancellation List (SCL) decoding [6] and CRC-Aided SCL decoding [7].

## 2.2 	Two Puncturing Schemes of Polar Codes

The length N of polar encoded block is a power of 2 due to the nature of polar encoding structure. On the other hand, for a given information block length K and coding rate R, the resulting code word length is determined as KR, which is not always a power of 2. Hence, the puncturing of the polar encoded block is needed to achieve the required coding rate. Suppose M bits are punctured from the N polar encoded bits. The resulting coding rate is KN-M. Hence, the number M is set as N-KR to match the desired coding rate. In the remaining of this contribution, we use (N,K,M) to denote a (N,K) polar code with M bits punctured.

The puncturing of polar codes can be represented by a puncturing vector P=(p1, …,pN), where pi∈{0,1}, and “0” indicates the punctured positions. Here, we have M zeros in the puncturing vector.

The Quasi-Uniform Puncturing (QUP) scheme was introduced in [8]. By the QUP scheme, the puncturing vector P is bit-reversal permutation on the N-bit vector with the first M bits zeros and the remaining N-M bits ones. The design criterion of QUP scheme is to maximize the minimum Hamming distance. It is shown [8]  that the minimum Hamming distance resulting from QUP scheme is larger than that resulting from random puncturing.

Another puncturing scheme was introduced in [9], which we call it Weight-1 Column Reduction Puncturing scheme (WCRP). By this scheme, the puncturing vector is determined from the generator matrix GN of polar code. Specifically, the index of a weight-1 column in GN is selected as a puncturing position (i.e., pi=0). The column and row corresponding to that 1’s location are removed from generator matrix GN. This reduces the matrix dimension from N×N to (N-1)×(N-1). Continue on the new matrix for the remaining M-1 puncturing positions in the same manner.

It should be mentioned that a weight-1 column of GN implies the one-to-one mapping between the corresponding input bit and output bit of polar encoder. In other words, if the input bit corresponding to a weight-1 column of GN is set as 0, its associated output bit is also equal to 0.

The code construction for the WCRP scheme is adjusted such that the input bit corresponding to a weight-1 column is set as a frozen bit (i.e., 0). This implies that the associated output bit is equal to 0. Therefore, puncturing on that output position does not lose information to decoder, as decoder knows the punctured bit is always 0. This a priori information improves the polar decoding performance. One advantage of the WCRP scheme over the QUP scheme is that its code construction does not need to be repeated for different number of punctured bits.

Note that the WCRP scheme does not lead to a unique puncturing vector, as there may exist more than one weight-1 column within each loop of the matrix reduction process. Different choices could be made on the weight-1 column selection.

Here, we use a different selection of the weight-1 columns from the exemplary selection in [9]. By this scheme, the puncturing vector P is the N-bit vector with the last M bits zeros and the remaining N-M bits ones. Figure 1 illustrates the example of N=8, M=4.

Figure 1: WCRP scheme for the case of N=8, M=4

## 2.3 	Performance Evaluations of the Puncturing Schemes of Polar Codes

In this section, we simulate the performance of the two puncturing schemes: QUP scheme and WCRP scheme. In our simulations, the polar code based on the Bhattacharyya bounds with design-SNR 0 dB is used. We use QPSK modulation and AWGN channel in the simulations.

Figure 2 shows the simulation results of a (1024, 256, 50) polar code and a (1024, 256, 250) polar code, where the CRC-aided SCL-4 decoding algorithm is used. It is seen from the figure that the QUP scheme outperforms the WCRP scheme in the case of 50 punctured bits, while the WCRP scheme outperforms the QUP scheme in the case of 250 punctured bits.

Observation 1: The performance of QUP scheme and WCRP scheme is related to the number of punctured bits.

Figure 2: Puncturing schemes comparison under different number of punctured bits

In Figure 3, we compare the performance of the two puncturing schemes under different block lengths at identical coding rate. Here, a (1024, 256, 50) polar code and a (2048, 512, 100) polar code with CRC-aided SCL-4 decoding algorithm are used. It is seen from the figure that both schemes have similar performance for the (2048, 512, 100) polar code, while the QUP scheme has better performance for the (1024, 256, 50) polar code.

Observation 2: The performance of QUP scheme and WCRP scheme is related to the block length of polar code.

Figure 3: Puncturing schemes comparison under different block lengths

In Figure 4, we compare the performance of the two puncturing schemes under different decoding algorithms. Here, a (2048, 512, 100) polar code is used. It is seen from the figure that both schemes have similar performance with the CRC-aided SCL-4 decoding algorithm, while the WCRP scheme outperforms the QUP scheme with the CRC-aided SCL-32 decoding algorithm.

Observation 3: The performance of the QUP scheme and the WCRP scheme is related to the polar decoding algorithm.

Figure 4: Puncturing schemes comparison under different decoding algorithms

As seen from the simulations, the performance of WCRP scheme and QUP scheme may crossover, depending on coding rate, block length and decoding algorithm. Hence, we propose to further study these two puncturing schemes for polar codes.

Proposal: For polar codes, both the WCRP scheme and the QUP scheme could be further studied.

# 3	Conclusion

In this contribution, we discussed two different puncturing schemes for polar codes, and compared their performance under different coding rates, block lengths, and decoding algorithms. Our simulation results show that:

Observation 1: The performance of the QUP scheme and the WCRP scheme is related to the number of punctured bits.

Observation 2: The performance of the QUP scheme and the WCRP scheme is related to the block length of polar code.

Observation 3: The performance of the QUP scheme and the WCRP scheme is related to the polar decoding algorithm.

Hence, we have the following proposal:

Proposal: For polar codes, both the WCRP scheme and the QUP scheme could be further studied.

# References

- R1-165959, “WF on channel coding simulation data sharing,” May 2016.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China, May 2016.

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #84bis, Busan, Korea, Apr. 2016.

- E. Arikan, “Channel polarization: A method for constructing capacity-achieving codes for symmetric binary-input memoryless channels,” IEEE Transactions on Information Theory, vol. 55, July 2009.

- H. Vangala, E. Viterbo and Y. Hong, “A comparative study of polar code constructions for the AWGN channel,” arXiv.1501.02473v1, Jan. 2015.

- I.Tai and A. Vardy, “List decoding of polar codes,” arXiv:1206.0050v1, May 2012.

- K. Niu and K. Chen, “CRC-aided decoding of polar codes,” IEEE Communications letters, Oct. 2012.

- K. Niu, K. Chen and J. Lin, “Beyond turbo codes: rate-compatible punctured polar codes,” IEEE ICC 2013.

- R. Wang and R. Liu, “A novel puncturing scheme for polar codes,” IEEE Communications letters, Dec. 2014.

- R1-162927, “Design considerations on channel coding with very large blocklength,” Apr. 2016.