3GPP TSG-RAN WG1#86	R1-167644

Gothenburg, Sweden 22rd - 27th August 2016

Source:	Ericsson

Title:	On the need for W1 enhancements for advanced CSI

Agenda Item:	[7.2.4.2.3]

Document for:	Discussion and Decision

# Introduction

In RAN1#85, the following was agreed regarding advanced CSI reporting:

- Specify enhancement on CSI reporting to improve eNB precoding. The specified enhancement is to be selected from the following categories:

  - Enhancements to Rel-13 feedback codebooks (FFS which numbers of antenna ports from the set {8, 12, 16, 20, 24, 28, 32}) that increase CSI resolution through improved beam selection / construction in W1 and/or improved beam/port selection / combining / weighting mechanisms in W2

  - Parameters representing channel coefficients, or some reduced space representation thereof including beam combining / weighting with coefficient quantisation or channel quantisation or channel covariance matrix quantisation

    - Uplink physical channel enhancements to carry the representation of channel coefficients can be included if selected

In our companion contribution [2], we discuss and evaluate the two categories of CSI enhancements. In our other companion contribution [3], we present our proposal of a new W1 design for implicit advanced CSI reporting. In this contribution, we conduct system simulations comparing our proposed enhanced W1 design with W1 from legacy codebooks.

# Desired properties for W1 enhancements

Most companies seem to be aligned regarding the structure of an enhanced W2 codebook, namely, that instead of selecting columns (i.e. beams) from the W1 matrix and only co-phasing (or combining) the different polarizations, the W2 matrix should perform co-phasing between different beams on the same polarization as well (i.e. linearly combining columns of W1 by weighting each column with a co-phasing factor ejα).  An enhanced W2 matrix may then be expressed as

W2=1ejα21⋮ejα2Nbeams1………1ejα2r⋮ejα2Nbeamsr,

where r is the transmission rank, Nbeams is the number of DFT beams in W1 per polarization and ejαkl is a co-phasing factor from a PSK alphabet intended for the k:th column of W1 and the l:th layer. Note that co-phasing between different polarizations of the same beam is not differentiated from the co-phasing between different beams on the same polarization with this formulation. One may of course separate the two into a “beam combination co-phasing coefficient” ck and a “polarization co-phasing coefficient” dk so that W2, for Nbeams=3 may be expressed as

W2=1c2(1)c3(1)d1(1)c2(1)d2(1)c3(1)d3(1)………1c2(r)c3(r)d1(r)c2(r)d2(r)c3(r)d3(r).

However, if both dk and ck are chosen from the same PSK alphabet, the multiplication of dk and ck will also be a PSK symbol in the same alphabet and a single co-phasing coefficient may be used for simplicity. In our view, there is no need to use different PSK alphabets for dk and ck and so, a single co-phasing parameter ejαk should be used.

While most companies are aligned on the need for W2 enhancements for advanced CSI reporting, some companies have proposed that W1 from the legacy codebook, consisting of non-orthogonal – overlapping – DFT beams, can be reused in the context of advanced CSI so that only a new W2 codebook needs to be specified. It is our view, however, that it is crucial that enhancements are also made for W1 and that a new W1 codebook should consist of the following components:

- Beam selection: Provide an unconstrained selection of Nbeams orthogonal (2D) DFT beams in order to ensure that the selected beams correspond to the multi-path components of the channel. Hence, beam selection is not limited to select beams from a subset of beams.

- Beam power allocation: Setting the relative power level of the beams in the precoder so that the relative strength of the different multi-path components of the channel are taken into account.

- Beam space rotation: Rotating the orthogonal DFT basis from where beam selection is done in order to maximize the channel energy captured by the precoder, similar to oversampling.

Further details on the proposed W1 codebook for rich CSI is given in [3],along with evaluation results motivating the W1 design.

# Evaluation results and motivation for W1 enhancements

To motivate the need for W1 enhancements, we present simulation results comparing rich CSI systems which all use an enhanced W2 linear combination codebook as described above, which selects co-phasing coefficients from an 8-PSK constellation on a subband level.

We simulate the following systems:

- A system performing SU-MIMO with W1 and W2 from a Release 13 Config 2 style codebook, extended to the N1=N2=4 case, with oversampling factors O1=O2=4.

- A system performing MU-MIMO with W1 and W2 from the Release 13 Config 2 codebook, as above.

- A system performing MU-MIMO with W1 from the Release 13 Config 2 codebook and using an enhanced W2 codebook as defined in Section 2.

- A system performing MU-MIMO with W1 from the Release 13 Config 4 codebook and using an enhanced W2 codebook as defined in Section 2.

  - That is, four adjacent non-orthogonal beams along the horizontal dimension is grouped in W1.

- A system performing MU-MIMO with our proposed enhanced W1 and W2 codebook where Nbeams=4 freely chosen orthogonal beams are selected in W1, along with a respective power level for each beam. Four beam space rotation hypotheses per dimension is used, to correspond to the 4x oversampling used in the legacy W1 codebook. Beam powers are quantized using 3 bits per beam.

The simulations are performed in 3GPP 3D UMi with an 8x4 cross-polarized antenna array with 2x1 subarray virtualization using the 3GPP FTP traffic model-1 with 500 kB packet size. The UEs are equipped with 2 RX antennas and dynamic rank adaptation as well as dynamic SU/MU-switching is used. For the MU-MIMO cases, additional SLNR processing of the reported precoders are used to suppress MU interference.

Simulation results are presented in Table 1. Our evaluations show that the advanced CSI schemes with an enhanced W2 codebook but legacy W1 codebooks give only marginal MU-MIMO gains for Config 4, a few percent, and no gains for Config 2, compared to using the legacy codebooks for both W1 & W2. However, if the proposed enhanced W1 codebook is used together with an enhanced W2 codebook, the gains are substantial, more than doubling the MU gain over SU compared to the legacy codebook. It is therefore a necessity to also specify a new W1 codebook when specifying advanced CSI enhancements in Rel. 14.

Table 1: Comparison between legacy and enhanced W1

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| Legacy Rel.13 style Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| Legacy Rel. 13 style DFT Codebook MU-MIMO | 28 | 9 | 67 | 26 |
| Legacy (Config 2) W1 + enhanced W2 MU-MIMO | 24 | 10 | 66 | 24 |
| Legacy (Config 4) W1 + enhanced W2 MU-MIMO | 36 | 13 | 84 | 33 |
| Enhanced (multi-beam) W1 + enhanced W2 MU-MIMO | 68 | 26 | 146 | 54 |

Observation:

- Enhanced W2 (linear combination codebook) without enhanced W1 gives only a marginal increase in MU-MIMO performance over Rel. 13 style codebook. However, if combined with enhanced (“multi-beam”) W1, substantial gains are seen.

Proposal:

- Specify both a new W1 and W2 codebook for advanced CSI reporting.

In RAN1#85, some companies provided evaluation results for advanced CSI where only rank 1 transmission where simulated. For completeness, we have simulated this case as well and present the results in Table 2. We note that when the rank is fixed to one, the schemes using legacy W1 codebook with enhanced W2 show a significant performance gain over the legacy Rel. 13 CB, which was not seen in the full rank case. This indicates that the legacy W1 codebook may not handle dual layer rich CSI precoding as effective as the proposed enhanced W1 codebook, or, the gains of the legacy W1 systems may be inflated since when all transmissions are rank one, the IRC receiver is much more effective and suppression of MU interference may be done sufficiently at the UE side, reducing the need for MU interference suppression by accurate precoding at the eNB side.

Table 2: Comparison between legacy and enhanced W1 where rank is fixed to 1

| Scheme | 50% RU |  | 70% RU |  |
| --- | --- | --- | --- | --- |
|  | CE UTP gain [%] | Mean UTP gain [%] | CE UTP gain [%] | Mean UTP gain [%] |
| Rel.13 style Codebook SU-MIMO | 0 | 0 | 0 | 0 |
| Legacy Rel. 13 style DFT Codebook MU-MIMO | 70 | 27 | 133 | 52 |
| Legacy (Config 2) W1 + enhanced W2 MU-MIMO | 117 | 36 | 217 | 71 |
| Legacy (Config 4) W1 + enhanced W2 MU-MIMO | 128 | 37 | 237 | 73 |
| Enhanced (multi-beam) W1 + enhanced W2 MU-MIMO | 151 | 39 | 286 | 78 |

Observation:

- When transmission rank is artificially fixed to one, enhanced W2 together with W1 from legacy codebook can give significant gains overusing both W1 and W2 from the legacy codebook, but the gains disappear in regular simulations when full rank selection with dynamic rank adaptation is turned on. It is thus crucial to evaluate advanced CSI codebook enhancements using full rank simulations with dynamic rank adaptation in order to not draw erroneous conclusions.

# Conclusions

In this contribution we have discussed and presented evaluation results with advanced CSI codebooks. A proposed enhanced W1 design have been compared to reusing the legacy W1 codebook designed for regular CSI reporting in the context of advanced CSI and only using an enhanced W2 “linear combination” codebook. We have made the following observations and proposals:

Observations:

- Enhanced W2 (linear combination codebook) without enhanced W1 gives only a marginal increase in MU-MIMO performance over Rel. 13 style codebook. However, if combined with enhanced (“multi-beam”) W1, substantial gains are seen.

- When transmission rank is artificially fixed to one, enhanced W2 together with W1 from legacy codebook can give significant gains over using both W1 and W2 from the legacy codebook, but the gains disappear in regular simulations when full rank selection with dynamic rank adaptation is turned on. It is thus crucial to evaluate advanced CSI codebook enhancements using full rank simulations with dynamic rank adaptation in order to not draw erroneous conclusions.

Proposal:

- Specify both a new W1 and W2 codebook for advanced CSI reporting.

# References

RP-160623, “New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE”, Samsung, Nokia, CATT, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, March 7 - 10, 2016

R1-167643, “Explicit versus implicit feedback for advanced CSI reporting”, Ericsson, RAN1#86

R1-167645, “W1 design for implicit advanced CSI reporting”, Ericsson, RAN1#86

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 8x4 with 2x1 virt., UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead not accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |