TSG-RAN WG1 #86	R1-167746

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Ericsson

Title:	Guardband for mixed numerologies

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

# Discussion

It has been proposed that NR should support the simultaneous use of different numerologies (different sub-carrier spacing and/or different CP lengths) on the same carrier, also referred to as mixed numerology. Mixed numerology is to be enabled by the use of spectral confinement on a subband basis.

In [1] we propose that spectral confinement on a subband basis should be specified as (RAN4) requirements on out-of-subband emissions (transmitter side) and adjacent subband interference suppression (receiver side) with no specification on exactly how the spectral confinement is achieved. These requirements should not be a single value but should rather specify the out-of-subband emissions and adjacent-suband interference suppression as a function of the distance to the subband of interest.

In [2] we further state that, in case of mixed numerologies, sub-carriers of acertain numerology should still lie on their “normal” grid, i.e. the sub-carrier grid and resource-block grid applicable for a certain numerology should not be impacted by the fact that mixed numerologies are used on a carrier.

However, there is still a question what guardbands one should assume between two subbands using different numerologies. What guard band is needed depends on, at least

- The out-of-subband emissions and  adjacent-subband interference suppression

- The acceptable inter-subband interference

The acceptable inter-subband interference heavily depends on the deployment

- In some cases a deployment is targeting high SINR implying the need for very low inter-subband interference and thus a high degree of inter-subband interference suppression. As a consequence, a larger guards band between subbands using different numerologies is needed.

- In other cases, for example in wide-area deployments or deployments with a high traffic density per node, the achievable SINR will anyway be limited and even a relatively high level of inter-subband interference will have limited impact on the overall performance. As a consequence, a smaller guardband would be sufficient. In some cases operation with zero-guardband may even be possible.

# Proposal

In case of mixed numerologies within one carrier, the guardband between subbands with different numerologies should not be specified but should be a deployment decision.

# References

[1]	R1-167029 “On NR spectrum confinement”

[2]	R1-167033 “On resource allocation and guard subcarriers for mixed numerologies”