3GPP TSG RAN WG1 Meeting #86		R1-167871

Gothenburg, Sweden, 22nd - 26th August 2016

Agenda Item: 8.1.4.1

Source: MediaTek Inc.

Title: Examination of NR Coding Candidates for Low-Rate Applications

Document for: Discussion/Decision

# Overview

In addition to the coding enhancement to achieve multi-Gbps data rates for NR, there exist equally important demand for NR low-rate applications, including at least control channels and massive MTC. Regarding a cost-effective multi-mode NR-LTE UE implementation, it is very useful if there are up to two NR decoders analogous to LTE low-rate TBCC Viterbi decoder and high-rate Turbo decoder. In addition, the NR low-rate decoder shall be subject to a practical cost constraint, say 1/16 w.r.t. the high-rate one, as in a commercial LTE chip. Keeping the UE demand in mind, this contribution is to examine and compare the NR coding candidates aiming at low-rate applications. In particular, Turbo code, TBCC, and Polar code will be considered.

From the agreement in #84b meeting:

- Selection of 5G new RAT channel coding scheme(s) will consider, - Performance - Implementation complexity - Latency (Decoding/Encoding) - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

and with the observations provided in #85 meeting t-docs, we can deduce the following analysis:

Table 1. Examination on NR coding candidates for low-rate applications (at least control and mMTC)

| Criterion | Requirement | Turbo | TBCC | Polar |
| --- | --- | --- | --- | --- |
| Performance | Superior performance with small codeblock sizes - Shall avoid tailing bit overhead | ∆ | O | O |
| Complexity | 1/16 area w.r.t. the high-rate decoder - Scaling codeblock size from 8192 to 1000 is not sufficient to shrink Turbo decoder | ∆ | O | ∆ O (constraint on SCL decoder L∙N complexity) |
| Latency | Low latency with small control codeblocks - Iterative Turbo decoding and successive Polar SC/SCL decoding are subject to longer latency | ∆ | O | ∆ |
| Flexibility | Support full rate-matching granularity at O(N) complexity (N: code bit length) - Effective O(N)-complexity rate-matching | O | O | ∆ O (O(N) complexity rate- matching) |

Based on the above analysis, one can reach a similar conclusion as for LTE MTC applications:

Proposal 1: Turbo code shall be de-prioritized for NR low-rate applications (at least control and mMTC) due to the tailing bit overhead, higher MAP iterative decoding complexity and latency.

For Polar code, the latency due to successive decoding is concerned, particularly for control channels with large number of blind decoding. Besides it, further technical investigation, however, clarifies:

Observation 1: There exists O(N) complexity rate-matching design for Polar code that can support full rate-matching granularity at linear complexity w.r.t. the code bit length as TBCC

Observation 2: With intended constraint on Polar SCL decoder complexity, 1/16 area w.r.t. the high-rate decoder is feasible.

Observation 3: With the O(N)-complexity rat-matching and the strict cost constraint, Polar code can still outperform TBCC by near 1 dB or more.

With the above observations, the following proposal is suggested:

Proposal 2: For NR low-rate applications, the complexity issues on Polar code rate-matching and decoder cost can be resolved while still achieving better performance than TBCC. However, there remains to investigate the impact of the successive decoding latency, critical to control channels with many blind decoding attempts, before selecting the coding for NR low rate applications.

This contribution is organized as follows: In Section 2, one O(N)-complexity rate-matching scheme for Polar code is introduced. In Section 3, it is identified that, by constraining the product of Polar SCL decoder list size and code bit length, 1/16 area w.r.t. the high-rate decoder can be achieved. However, adaptive Polar SCL decoder list size w.r.t. the code bit length is suggested to best balance the performance and complexity. In Section 4, the performances of TBCC with Viterbi decoder and Polar code with adaptive SCL decoder will be compared. Finally, conclusions are drawn in Section 5.

# Achieving Full Rate-Matching Granularity at Linear Complexity

Define the following notations:

- B: Information bit length

- K: Input bit length which equals (I + CRC length)

- N: Code bit length before rate matching; N = 3K for TBCC and being a power of 2 for Polar code

- N’: Code bit length after rate matching

- R = B/N’: Targeted code rate

By full rate-matching granularity at linear complexity, we mean, for every K and R of interest, one can produce N’ code bits at a processing complexity O(N). For LTE TBCC, the circular buffer rate-matching design can achieve this feature as only simple repetition and puncturing are involved. However, for conventional Polar code rate matching, there require good bit re-estimation after code bit puncturing. Gaussian approximation scheme of complexity O(N∙log2(N)) is commonly used with the SNR assumptions for different code rates assumed known in both transmitter and receiver [1]. In [2], it is observed that QUP and the design based on compound Polar code will suffer significant performance loss without good-bit re-estimation, and thus low-complexity design is still demanded.

To fulfill the demand, the following scheme consisting 3 simple steps is identified:

- Rate-dependent code bit puncturing:

Apply different deterministic code bit puncturing designs according to the mother code rate, K/N. In particular, one design is to preserve the polarization effect for the good bits when K/N <= 1/4, and the other design apply the known bit puncturing [3] for K/N > 1/4. For more details of the puncturing design, please refer to the Appendix.

- Input bit loading w.r.t. predetermined good-bit-order list(s):

A good-bit-order list is a sequence of input bit indices where the order indicate the possibility to be selected as a good bit. Such list can be obtained by off-line counting the occurrence for each input bit index over the lists of selected good bit indices under different targeted code rates and corresponding operating SNRs. For each N of interest, one can store one good-bit-order list. But, there exist a useful nested property so that we can store only the good-bit-order list of largest N of interest:

For N1 < N2, the good-bit-order list of N1 can be well approximated by excluding the entries with index value >= N1 (or > N1 if the smallest index is 1) from the good-bit-order list of N2.

In the Appendix, the good-bit-order list of N = 4096 is provided, and those for smaller N can be derived with the above nested property.

- Good bit skipping along the input bit loading:

With the indication on the indices of punctured code bits, one can finally load the K input bits according to the first K indices from the good-bit-order list that has no puncture indication. Effectively, we skip the good bit indices where the directly connected code bits are punctured.

Below is an illustration with K = 3, N = 8, N’ = 4. First, {c0, c1, c2, c5} are to be punctured by the decided puncturing scheme according to K/N. A predetermined good-bit-order list of the sequence 7, 6, 5, 3, 4, 2, 1, 0 is then applied to load the K input bits. In the third step, u5 is skipped since the code bit c5 of direct connection will be punctured. It can be checked that the proposed 3-step processing can be applied to arbitrary K and N’ with complexity O(N), thus achieving full rate-matching granularity at linear complexity. In Section 4, there will be simulation results showing the effectiveness of the low-complexity rate-matching.

Fig. 1 Illustration of O(N)-complexity rate-matching scheme for Polar code

# Confining the Complexity of the Low-Rate Decoder

In a high-throughput LTE chip, the decoder related area is dominated by the high-rate turbo decoder. On the other hand, the area of the low-rate TBCC decoder can only occupy 1/16 area of the high-rate decoder. For a MTC device, such a stringent area requirement is more important to realize compact cost. For cost-effective NR devices, we shall expect similar area target on NR low-rate decoder, i.e., 1/16 area of NR high-rate decoder.

According to Ericsson’s analysis [4], quoted in Table 2 below, Polar code exhibits large memory complexity. Since both computational and memory complexities of Polar code are related to L ∙ N, i.e., the produce of SCL decoder list size and the code bit number before puncturing, we propose to pose a stringent constraint on L ∙ N value, say 4096 or 16384, for NR low-rate applications so as to significantly scale down the arithmetic and memory complexities. Although mapping the analytical complexity to real chip area depends on many factors, the significantly small complexity values can still ensure the 1/16 area target by a certain margin.

Table 2. Complexity comparison among NR coding candidates (part of the data is quoted from [4])

| Complexity comparison per decode | Description | Nr adders [adder equivalents] | Memory [bits] | Note |
| --- | --- | --- | --- | --- |
| LDPC | 20 iterations | 18,620,000 | 958,000 | K = 8,192 Code rate = 1/3 Polar N = 32,768 SCL list size L = 32 Complexity ∝ L⋅N =1,048,576 |
| Turbo | 8 iterations | 12,176,000 | 925,000 |  |
| Polar | 1 decode, list 32 | 9,000,000 | 6,300,000 |  |
| Suggested Polar (for low rate) | 1 decode, N-dependent list size | 37,000 (146,000) | 33,000 (132,000) | Constrain L⋅N≤4096 (16384) |

To optimize the performance under the constraint, we further propose N-dependent SCL decoder list size regarding that Polar code with larger N can perform better and require a smaller list size. Since the SCL decoding can be implemented by recursively scheduling an atomic unit processing a fraction of N and L [5], the adaptive SCL list size can be realized by switching the scheduling control. Regarding the control and mMTC settings of interest, the following setting:

- L = min{32, (L ∙ N constraint, say 4096 or 16384) / N}

- N <= 4096 (and do repetition if more code bits can be accommodated)

can be effective for optimizing the performance under the complexity constraint.

# Performance Comparison

In this section, simulation results are provided for:

- Checking if the suggested O(N)-complexity rate-matching scheme can work well by comparing QUP of good bit re-estimation post puncturing

- Examining the performance of Polar code with the proposed complexity reductions by comparing TBCC performance

For the comparisons, all possible specifications on (N’, N, L) for information bit length and code rates of interest are listed below with L ∙ N constrained to 4096. For the more relaxed constraint, 16384, one can simply set 4 times of list size but no larger than 32. Note that we assume CRC length is 24 bits for comparing the NB-IOT like settings.

| (N’, N, L) L∙N ≤ 4096 |  | Information bit length I (Input bit length K = I + 24-bit CRC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 (K=44) | 40 (K = 64) | 100 (K = 124) | 200 (K = 224) | 600 (K = 624) | 1000 (K = 1024) |
| Code rate Code rate | 1/12 | (240, 256, 16) | (480, 512, 8) | (1200, 2048, 2) | (2400, 4096, 1) | (36007200*, 4096, 1) | (400012000*, 4096, 1) |
|  | 1/6 | (120, 128, 32) | (240, 256, 16) | (600, 1024, 4) | (1200, 2048, 2) | (3600, 4096, 1) | (30006000*, 4096, 1) |
|  | 1/3 | (60, 64, 32) | (120, 128, 32) | (300, 512, 8) | (600, 1024, 4) | (1800, 2048, 2) | (3000, 4096, 1) |
|  | 1/2 | N/A | (80, 128, 32) | (200, 256, 16) | (400, 512, 8) | (1200, 2048, 2) | (2000, 2048, 2) |
|  | 2/3 | N/A | N/A | (150, 256, 16) | (300, 512, 8) | (900, 1024, 4) | (1500, 2048, 2) |
|  | 5/6 | N/A | N/A | N/A | (240, 256, 16) | (720, 1024, 4) | (1200, 2048, 2) |

* Red-colored code bit number is generated via repetition after Polar code encoding and rate-matching.

In Fig. 2, the performance of O(N)-complexity rate-matching (in blue color) is compared  with that of QUP rate-matching (in red color). It can checked that the performance difference is small for various codeblock sizes and code rates, indicating the effectiveness of the O(N)-complexity rate-matching scheme. Note that L ∙ N constraint is 16384 for the comparison, and, due to reduced SCL list size and simple repetition scheme for the large codeblock sizes, their performance can become slightly worse.

Fig. 2 O(N)-complexity rate-matching (blue curves) v.s. QUP with good-bit re-estimation (red ones)

In Fig. 3, the performance of Polar code (in blue color) and that of TBCC (in red color) are compared, where Polar code is equipped with O(N)-complexity rate-matching and subject to the stricter constraint L ∙ N ≤ 4096 for the solid blue curves (and remaining 16384 for the blue dash curves). Yet, by virtue of N-dependent SCL decoder list size, there still exhibits near 1 dB or more gain universally over various code rates and codeblock sizes.

Fig. 3: Performance of Polar code (L∙N = 4096 [16384] for blue solid [dash] curves) v.s. TBCC (red ones)

Since for small codeblock sizes, CRC overhead is usually reduced, the following settings are further compared with 16-bit CRC, where codeblock sizes of 60 bits and 80 bits are further added:

| (N’, N, L) L∙N ≤ 4096 |  | Information bit length I (Input bit length K = I + 16-bit CRC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 (K=36) | 40 (K = 56) | 60 (K = 76) | 80 (K = 96) | 100 (K = 216) | 200 (K = 216) |
| Code rate | 1/12 | (240, 256, 16) | (480, 512, 8) | (720, 1024, 4) | (960, 1024, 4) | (1200, 2048, 2) | (2400, 4096, 1) |
|  | 1/6 | (120, 128, 32) | (240, 256, 16) | (360, 512, 8) | (480, 512, 8) | (600, 1024, 4) | (1200, 2048, 2) |
|  | 1/3 | (60, 64, 32) | (120, 128, 32) | (180, 256, 16) | (240, 256, 16) | (300, 512, 8) | (600, 1024, 4) |
|  | 1/2 | (40, 64, 32) | (80, 128, 32) | (120, 128, 32) | (160, 256, 16) | (200, 256, 16) | (400, 512, 8) |
|  | 2/3 | N/A | (60, 64, 32) | (90, 128, 32) | (120, 128, 32) | (150, 256, 16) | (300, 512, 8) |
|  | 5/6 | N/A | N/A | N/A | N/A | (120, 128, 32) | (240, 256, 16) |

In Fig. 4, the performance of Polar code (in blue color) and that of TBCC (in red color) are further compared, and it can be checked that 0.8 dB or more gains are realized even with the strict constraint L ∙ N ≤ 4096.  Overall, we can conclude that better performance than TBCC is feasible with the low-complexity Polar code utilizing O(N)-complexity rate-matching and small product of SCL list size and the code bit length. Note that the detail simulation results can be found in [6].

Fig. 4: Performance of Polar code (L∙N = 4096 for blue solid curves) v.s. TBCC (red ones)

# Summary

In this contribution, the coding candidates for NR low rate applications, particularly control channels and mMTC, are examined. In particular, we first have

Proposal 1: Turbo code shall be de-prioritized for NR low-rate applications (at least control and mMTC) due to the tailing bit overhead, higher iterative decoding complexity and latency.

For Polar code, the latency due to successive decoding is concerned, particularly for control channels with large number of blind decoding. Besides it, further technical investigation, however, clarifies:

Observation 1: There exists O(N) complexity rate-matching design for Polar code that can support full rate-matching granularity at linear complexity w.r.t. the code bit length as TBCC

Observation 2: With intended constraint on Polar SCL decoder complexity, 1/16 area w.r.t. the high-rate decoder is feasible.

Observation 3: With the O(N)-complexity rat-matching and the strict cost constraint, Polar code can still outperform TBCC by near 1 dB or more.

With the above observations, the following proposal is suggested:

Proposal 2: For NR low-rate applications, the complexity issues on Polar code rate-matching and decoder cost can be resolved while still achieving better performance than TBCC. However, there remains to investigate the impact of the successive decoding latency, critical to control channels with many blind decoding attempts, before selecting the coding for NR low rate applications.

References

- R1-164039  “Polar codes - encoding and decoding”, Huawei, HiSilicon

- R1-165454  “Discussion on Polar code design and performance”, Mediatek

- R. Wang and R. Liu, "A Novel Puncturing Scheme for Polar Codes",  IEEE Communications Letters, vol. 18, no. 12, pp. 2081-2084, Dec. 2014.

- R1-164360  “Analysis of candidate code types for long block length”, Ericsson

- R1-164040  “On latency and complexity”, Huawei, HiSilicon

[6]    R1-167544 “Data sharing on NR Channel Coding Simulation Result”, Mediatek

# Appendix – Supplement for O(N)-Complexity Rate-Matching Scheme

In the Appendix, more specification will be supplied so that companies interested in the scheme can easily reproduce the simulations. First, we assume the following type of input-output connection for Polar encoding. Let P = N – N’ be the targeted number of punctured code bits, then we can specify:

- Puncturing scheme for low mother code rate, i.e., K/N ≤ 1/4:

  - Puncture cj for j = 0, …, min(P, N/4) – 1.

  - If P > N/4, further puncture cj for both

    - j = N/4, …, N/4 +  ceil((P – N/4) / 2) – 1

    - j = N/2, …, N/2 + floor((P – N/4) / 2) – 1

  - The following is an example with K = 1, N = 8, N’ = 4, P = 4, and it can be checked that the puncturing design is to preserve the polarization effect for the input bit(s).

  - Note that, for the decoding, the LLR(s) of the punctured code bit(s) will be set to zero before feeding into the decoder.

- Puncturing scheme for higher mother code rate, i.e., K/N > 1/4:

  - For the case, we apply the known-bit puncturing in [3], where cj with j = (N – P), …, N-1 will be punctured. Note that since the P punctured code bit(s) will be all zero with their directly connected input bit(s) set to zero, the LLR(s) of the punctured code bits will be set to non-zero value(s) of a large amplitude for the high confidence of receiving a known zero on the code bit(s) before feeding into the decoder.

  - The following is an example with K = 3, N = 8, N’=6, P=2. Since setting a large LLR for the known code bits will effectively break those shadowed connections, there also reduce interference for the selected input bits (marked by green color).

For the predetermined good-bit-order list, below we provide the list for N = 4096. As noted previously, the list for N < 4096 can be extracted from the list via the following nested property:

For N1 < N2, the good-bit-order list of N1 can be well approximated by excluding the entries with index value >= N1 (or > N1 if the smallest index is 1) from the good-bit-order list of N2.

The list is not unique, and there shall exist more systematic design(s) to generate the list efficiently.

| Row 0: | 4095 | 4094 | 4093 | 4091 | 4087 | 4079 | 4092 | 4063 | 4090 | 4089 | 4086 | 4031 | 4085 | 4078 | 4083 | 4077 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Row 1: | 3967 | 4062 | 4075 | 4061 | 4088 | 4071 | 4030 | 4059 | 4084 | 3839 | 4029 | 4082 | 4055 | 4076 | 4081 | 4027 |
| Row 2: | 3966 | 4047 | 4074 | 3965 | 4023 | 4060 | 4073 | 3583 | 4070 | 3963 | 4058 | 4015 | 4069 | 3838 | 4057 | 3959 |
| Row 3: | 4028 | 4054 | 3837 | 4067 | 3999 | 4080 | 4053 | 4026 | 3835 | 3951 | 4046 | 4025 | 4051 | 3071 | 4072 | 4022 |
| Row 4: | 3964 | 4045 | 3582 | 3831 | 4021 | 3935 | 3581 | 3962 | 4043 | 4068 | 4014 | 4019 | 3823 | 4056 | 3961 | 4013 |
| Row 5: | 3579 | 4066 | 4039 | 3958 | 3836 | 3903 | 3998 | 4065 | 4052 | 4011 | 3957 | 3807 | 3575 | 4024 | 3834 | 3997 |
| Row 6: | 2047 | 3950 | 4050 | 3070 | 3955 | 4007 | 4044 | 3949 | 3833 | 4049 | 3567 | 3995 | 3069 | 4020 | 3830 | 3775 |
| Row 7: | 4042 | 3934 | 3947 | 3580 | 3829 | 3067 | 4018 | 3991 | 3960 | 4041 | 3933 | 3551 | 4012 | 4038 | 3822 | 3943 |
| Row 8: | 3578 | 4017 | 3827 | 3063 | 3902 | 3931 | 4037 | 3956 | 3821 | 4064 | 3983 | 4010 | 3711 | 3577 | 3574 | 3901 |
| Row 9: | 3996 | 3806 | 3519 | 3954 | 4035 | 4009 | 3927 | 3819 | 3055 | 4006 | 3948 | 2046 | 4048 | 3573 | 3832 | 3953 |
| Row 10: | 3899 | 3805 | 3994 | 3068 | 4005 | 2045 | 3815 | 3919 | 3566 | 3946 | 3993 | 3039 | 3571 | 4040 | 3774 | 3803 |
| Row 11: | 3990 | 3828 | 3932 | 3895 | 4003 | 3066 | 3565 | 2043 | 3945 | 3455 | 4016 | 3942 | 3989 | 3773 | 3065 | 3826 |
| Row 12: | 3799 | 4036 | 3550 | 3930 | 3563 | 3062 | 2039 | 3982 | 3887 | 3941 | 3820 | 3007 | 3987 | 3825 | 3771 | 3576 |
| Row 13: | 4008 | 3929 | 3710 | 3549 | 3981 | 4034 | 3061 | 3900 | 3926 | 3791 | 3939 | 3559 | 3818 | 2031 | 3709 | 3054 |
| Row 14: | 3871 | 4033 | 3952 | 3979 | 3804 | 3767 | 4004 | 3925 | 3547 | 3518 | 3572 | 3817 | 3327 | 3059 | 3898 | 2044 |
| Row 15: | 3814 | 3053 | 3992 | 3707 | 3918 | 2943 | 3517 | 3802 | 3897 | 4002 | 2015 | 3570 | 3759 | 3975 | 3813 | 3543 |
| Row 16: | 3923 | 3894 | 3944 | 2042 | 3038 | 3917 | 3564 | 3051 | 3801 | 4001 | 3988 | 3772 | 3515 | 3569 | 3798 | 3703 |
| Row 17: | 3893 | 3811 | 3454 | 2041 | 3064 | 3037 | 3535 | 3743 | 3915 | 3940 | 2038 | 3824 | 3886 | 3562 | 3986 | 3797 |
| Row 18: | 1983 | 3047 | 3770 | 3453 | 3891 | 3980 | 3928 | 3511 | 3548 | 3695 | 2037 | 3885 | 3985 | 3790 | 3561 | 3035 |
| Row 19: | 3006 | 3938 | 3769 | 3060 | 3911 | 2815 | 3558 | 3795 | 3766 | 3816 | 2030 | 3708 | 3978 | 3451 | 3789 | 3937 |
| Row 20: | 3924 | 3870 | 3546 | 3883 | 4032 | 2035 | 3005 | 3503 | 3557 | 3058 | 3765 | 3326 | 3031 | 2029 | 3679 | 3977 |
| Row 21: | 3896 | 3869 | 1919 | 3545 | 3974 | 3787 | 3052 | 3812 | 3922 | 3706 | 3758 | 3516 | 3542 | 3447 | 3879 | 3003 |
| Row 22: | 3057 | 3555 | 3325 | 3916 | 3763 | 2942 | 2014 | 3800 | 2027 | 3921 | 3973 | 4000 | 3705 | 3867 | 3487 | 3757 |
| Row 23: | 3568 | 3892 | 3023 | 3810 | 3541 | 3783 | 3702 | 3050 | 3514 | 2040 | 3914 | 2013 | 3323 | 3647 | 2941 | 3439 |
| Row 24: | 3971 | 2999 | 3036 | 3809 | 3796 | 3534 | 3742 | 3755 | 3890 | 3513 | 2023 | 3863 | 3701 | 3049 | 3913 | 2559 |
| Row 25: | 3539 | 3510 | 3984 | 3560 | 3884 | 3046 | 3910 | 3452 | 3768 | 2011 | 3533 | 1982 | 2036 | 3741 | 3694 | 3889 |
| Row 26: | 2939 | 3794 | 3319 | 1791 | 3034 | 3509 | 3699 | 3751 | 3909 | 3936 | 3788 | 3855 | 3423 | 2991 | 3045 | 2814 |
| Row 27: | 3693 | 3882 | 1981 | 3793 | 3531 | 3556 | 3450 | 3976 | 3739 | 3502 | 2034 | 3764 | 2007 | 3033 | 3868 | 3004 |
| Row 28: | 3507 | 3907 | 3030 | 3544 | 3881 | 2935 | 3311 | 2028 | 3043 | 2813 | 3449 | 3786 | 3501 | 3678 | 3554 | 3691 |
| Row 29: | 3878 | 3527 | 2033 | 1979 | 3446 | 2975 | 3920 | 3056 | 1918 | 3029 | 3391 | 3324 | 3762 | 3866 | 3735 | 3704 |
| Row 30: | 3972 | 3002 | 3785 | 1999 | 3677 | 3553 | 3540 | 3486 | 3499 | 2811 | 3877 | 3445 | 2026 | 3756 | 3782 | 2927 |
| Row 31: | 3687 | 3295 | 3022 | 1917 | 3761 | 3865 | 3001 | 3512 | 1975 | 2012 | 3485 | 3322 | 3438 | 3912 | 3027 | 3862 |
| Row 32: | 3538 | 3675 | 2998 | 3970 | 3646 | 2025 | 3700 | 3875 | 3048 | 3781 | 1535 | 3808 | 3727 | 2940 | 3443 | 3021 |
| Row 33: | 3495 | 3532 | 3754 | 2558 | 2022 | 2807 | 3437 | 3888 | 3537 | 3321 | 3861 | 1915 | 2997 | 3483 | 3969 | 3645 |
| Row 34: | 3740 | 3508 | 2010 | 2911 | 3779 | 3318 | 3698 | 3908 | 3671 | 3753 | 1967 | 2557 | 2021 | 3019 | 3044 | 2938 |
| Row 35: | 3530 | 3854 | 3263 | 1790 | 3750 | 3422 | 2990 | 3435 | 3692 | 3792 | 3859 | 2009 | 3317 | 3643 | 3697 | 2995 |
| Row 36: | 3880 | 3738 | 3032 | 2799 | 3506 | 3853 | 1980 | 3448 | 3479 | 3906 | 3529 | 2006 | 2937 | 3421 | 3749 | 1911 |
| Row 37: | 2989 | 2019 | 1789 | 2555 | 3663 | 3042 | 3500 | 3526 | 3310 | 3015 | 2934 | 3690 | 3431 | 3737 | 3505 | 1951 |
| Row 38: | 2879 | 2812 | 3905 | 3784 | 2032 | 3315 | 3734 | 3851 | 3876 | 2005 | 3639 | 3676 | 3747 | 3041 | 3028 | 3419 |
| Row 39: | 2974 | 3390 | 1978 | 3444 | 2987 | 3525 | 3309 | 3552 | 3689 | 3471 | 2933 | 1787 | 3498 | 3864 | 3199 | 3760 |
| Row 40: | 2551 | 2783 | 3686 | 1903 | 3733 | 1998 | 3000 | 1977 | 3874 | 2973 | 2810 | 2926 | 3389 | 3780 | 3484 | 3847 |
| Row 41: | 3674 | 3026 | 3523 | 3442 | 2003 | 3294 | 3497 | 3415 | 3307 | 1974 | 3726 | 1023 | 3631 | 3685 | 2931 | 2024 |
| Row 42: | 2983 | 3020 | 1916 | 1997 | 3494 | 3731 | 3436 | 3873 | 3860 | 2925 | 3536 | 1534 | 3752 | 3025 | 2809 | 3673 |
| Row 43: | 1783 | 3320 | 3725 | 2543 | 2971 | 3778 | 3441 | 3387 | 3293 | 2996 | 3482 | 3644 | 1973 | 3670 | 2806 | 3683 |
| Row 44: | 1887 | 3493 | 3303 | 2751 | 3407 | 3018 | 1533 | 3968 | 2910 | 1995 | 3858 | 2923 | 2020 | 1914 | 3777 | 3434 |
| Row 45: | 3615 | 2556 | 3696 | 3481 | 1966 | 3723 | 3748 | 3669 | 2805 | 3528 | 3852 | 3291 | 3642 | 3262 | 2994 | 3316 |
| Row 46: | 3478 | 2936 | 2967 | 1971 | 3491 | 3420 | 3383 | 3017 | 2008 | 2909 | 3857 | 1775 | 3736 | 3433 | 2527 | 1913 |
| Row 47: | 3662 | 2988 | 1965 | 3014 | 2798 | 1531 | 3641 | 2919 | 2018 | 3504 | 3746 | 3430 | 2554 | 3477 | 1991 | 3719 |
| Row 48: | 1910 | 2993 | 3261 | 3667 | 3638 | 2803 | 3850 | 3688 | 3314 | 3287 | 3524 | 1788 | 1855 | 2907 | 3661 | 3418 |
| Row 49: | 2878 | 2932 | 3904 | 3013 | 2797 | 3040 | 2959 | 3745 | 3732 | 3470 | 2017 | 2004 | 3375 | 3429 | 1950 | 3308 |
| Row 50: | 2986 | 1963 | 2687 | 2553 | 3637 | 1909 | 3849 | 3475 | 3313 | 3259 | 1527 | 3496 | 1759 | 2550 | 3417 | 2877 |
| Row 51: | 3198 | 3846 | 2972 | 3659 | 3469 | 3684 | 3522 | 3011 | 2930 | 3388 | 2782 | 1976 | 2903 | 3630 | 3730 | 2985 |
| Row 52: | 2795 | 2495 | 3414 | 3279 | 1902 | 1949 | 1786 | 3427 | 2002 | 3306 | 2982 | 3635 | 3724 | 2924 | 3672 | 1022 |
| Row 53: | 3024 | 2549 | 1996 | 1959 | 1907 | 2808 | 2929 | 2875 | 2781 | 3845 | 2970 | 3197 | 3872 | 1901 | 3521 | 3359 |
| Row 54: | 3629 | 3729 | 3440 | 2001 | 1785 | 3467 | 3492 | 3292 | 3682 | 3255 | 2981 | 3413 | 1947 | 1519 | 3305 | 3655 |
| Row 55: | 2542 | 2791 | 1972 | 1021 | 2895 | 3386 | 1782 | 2922 | 3302 | 2969 | 2547 | 3722 | 3480 | 1727 | 1994 | 3016 |
| Row 56: | 2779 | 3681 | 3668 | 2750 | 3614 | 3195 | 3843 | 2804 | 2966 | 1886 | 3627 | 3406 | 1899 | 3776 | 2979 | 2908 |
| Row 57: | 2871 | 3290 | 2541 | 1532 | 3385 | 3490 | 2921 | 1781 | 3247 | 3463 | 3301 | 1019 | 3411 | 2431 | 3721 | 3382 |
| Row 58: | 1970 | 3856 | 1943 | 2992 | 3432 | 3640 | 1912 | 2918 | 1993 | 2965 | 3613 | 1885 | 2749 | 3405 | 3718 | 3666 |
| Row 59: | 3289 | 1503 | 2802 | 3489 | 2775 | 3476 | 3012 | 1964 | 1990 | 1774 | 2526 | 2906 | 3260 | 3623 | 2539 | 1895 |
| Row 60: | 3286 | 3660 | 3191 | 3381 | 2958 | 3299 | 1530 | 1969 | 2863 | 2917 | 2796 | 1779 | 3665 | 3717 | 3744 | 2963 |
| Row 61: | 1015 | 3611 | 2747 | 2801 | 1883 | 3403 | 3636 | 1854 | 3428 | 3231 | 2984 | 1908 | 2552 | 1989 | 3312 | 2905 |
| Row 62: | 3374 | 1773 | 2016 | 1935 | 2686 | 3474 | 2525 | 2957 | 3285 | 3010 | 3848 | 2876 | 3258 | 1962 | 3658 | 1663 |
| Row 63: | 2902 | 1529 | 2767 | 3379 | 2915 | 3468 | 3416 | 2794 | 2535 | 3715 | 3183 | 1526 | 3373 | 1853 | 3473 | 1948 |
| Row 64: | 3278 | 3634 | 2685 | 3426 | 3009 | 2847 | 3607 | 3257 | 1987 | 1906 | 2928 | 1758 | 3657 | 3399 | 2955 | 1879 |
| Row 65: | 1961 | 2980 | 2780 | 1771 | 2743 | 1471 | 2523 | 2548 | 2494 | 3728 | 3283 | 2901 | 2793 | 1007 | 3254 | 3628 |
| Row 66: | 2303 | 3654 | 3304 | 2874 | 1958 | 3196 | 3466 | 1900 | 3844 | 2000 | 2790 | 3277 | 3633 | 3412 | 3425 | 1525 |
| Row 67: | 3358 | 3371 | 2968 | 1784 | 3520 | 1905 | 1851 | 2683 | 1757 | 1946 | 2894 | 2493 | 2778 | 1020 | 3653 | 3253 |
| Row 68: | 2873 | 3680 | 3599 | 2978 | 1957 | 3465 | 3167 | 2546 | 2519 | 2735 | 2951 | 2789 | 3626 | 3384 | 2899 | 1518 |
| Row 69: | 1767 | 1871 | 2870 | 3357 | 2920 | 3275 | 3462 | 3300 | 3410 | 3194 | 3720 | 1898 | 3246 | 1945 | 3842 | 2893 |
| Row 70: | 2540 | 2777 | 1523 | 991 | 3612 | 3367 | 2977 | 2964 | 1992 | 1755 | 3404 | 3651 | 1726 | 1780 | 1942 | 2748 |
| Row 71: | 3625 | 3251 | 2545 | 2491 | 2774 | 1955 | 3288 | 1884 | 1847 | 2679 | 1018 | 2869 | 1517 | 3409 | 2787 | 3461 |
| Row 72: | 3488 | 2430 | 3193 | 3622 | 1897 | 3355 | 3245 | 3380 | 3664 | 1407 | 3298 | 3841 | 2511 | 1968 | 2891 | 3271 |
| Row 73: | 3190 | 2916 | 1894 | 2719 | 2800 | 1941 | 2862 | 3610 | 3716 | 1725 | 2962 | 2538 | 3402 | 2773 | 3135 | 1017 |
| Row 74: | 2746 | 1778 | 1751 | 3621 | 1988 | 3459 | 2429 | 1882 | 1502 | 2867 | 1515 | 3297 | 3284 | 2956 | 2904 | 3230 |
| Row 75: | 3243 | 1014 | 2524 | 1934 | 2487 | 3378 | 3351 | 3189 | 2671 | 1839 | 1893 | 1772 | 3401 | 3609 | 2861 | 2961 |
| Row 76: | 2766 | 2537 | 2914 | 959 | 3472 | 2745 | 1939 | 3656 | 3372 | 2887 | 1777 | 1723 | 3398 | 3606 | 1960 | 3714 |
| Row 77: | 1528 | 3256 | 1881 | 3619 | 2534 | 2771 | 1662 | 3182 | 2742 | 3229 | 1501 | 1933 | 3377 | 1986 | 2792 | 3282 |
| Row 78: | 1013 | 1852 | 2684 | 2954 | 2427 | 1878 | 1743 | 2765 | 2900 | 2913 | 1891 | 3187 | 2846 | 2522 | 2859 | 3632 |
| Row 79: | 3239 | 3397 | 1770 | 3276 | 3424 | 1511 | 3008 | 3343 | 3605 | 3713 | 2479 | 2533 | 3181 | 3370 | 2741 | 1006 |
| Row 80: | 3281 | 1661 | 2302 | 1985 | 1823 | 3652 | 1904 | 2953 | 2655 | 1931 | 3464 | 3227 | 1956 | 1756 | 1719 | 3252 |
| Row 81: | 2872 | 1499 | 1470 | 1524 | 1877 | 3598 | 1769 | 3356 | 2845 | 2521 | 1279 | 1011 | 2763 | 2950 | 2898 | 2788 |
| Row 82: | 1850 | 3395 | 2682 | 2734 | 3369 | 2492 | 3274 | 1766 | 2518 | 3603 | 2423 | 3166 | 2855 | 1005 | 3179 | 1944 |
| Row 83: | 2531 | 2892 | 2301 | 3366 | 3624 | 1659 | 2739 | 1870 | 3408 | 2776 | 3597 | 895 | 1754 | 3650 | 1469 | 2897 |
| Row 84: | 1849 | 2949 | 1954 | 2976 | 3250 | 2681 | 3223 | 3192 | 3354 | 2843 | 3460 | 2733 | 2463 | 1522 | 1765 | 1927 |
| Row 85: | 1495 | 2868 | 1711 | 2544 | 3273 | 1875 | 3165 | 2786 | 1846 | 2517 | 3365 | 1896 | 2759 | 2678 | 2490 | 3244 |
| Row 86: | 3270 | 1869 | 990 | 1516 | 2890 | 1003 | 3175 | 1753 | 3649 | 2299 | 2623 | 1940 | 1953 | 3249 | 3353 | 3595 |
| Row 87: | 3620 | 1724 | 2415 | 2947 | 1521 | 1467 | 1750 | 2510 | 1655 | 2772 | 2785 | 3400 | 1845 | 2718 | 1763 | 2731 |
| Row 88: | 3350 | 3163 | 2489 | 3458 | 1406 | 2866 | 3134 | 2839 | 3188 | 3363 | 3269 | 2677 | 3296 | 1016 | 2889 | 3215 |
| Row 89: | 2515 | 3608 | 989 | 1487 | 2486 | 3242 | 1867 | 2960 | 1892 | 1695 | 2536 | 2860 | 1776 | 2886 | 1838 | 3840 |
| Row 90: | 1938 | 2428 | 1514 | 3376 | 1749 | 2744 | 2509 | 3618 | 3349 | 2670 | 1722 | 2717 | 2865 | 999 | 3457 | 2770 |
| Row 91: | 3228 | 3133 | 3591 | 2295 | 1843 | 1405 | 1932 | 1880 | 3267 | 3396 | 3186 | 3159 | 3241 | 1500 | 1463 | 2485 |
| Row 92: | 2675 | 1647 | 2727 | 2885 | 1837 | 1937 | 3342 | 2764 | 1742 | 2912 | 2831 | 2399 | 1513 | 987 | 1890 | 3238 |
| Row 93: | 958 | 1721 | 1012 | 3180 | 3617 | 3604 | 1863 | 2858 | 2669 | 767 | 1747 | 1510 | 1718 | 3280 | 2769 | 1768 |
| Row 94: | 2507 | 2532 | 2426 | 3347 | 1660 | 1930 | 1403 | 2952 | 2715 | 2478 | 1741 | 3185 | 3131 | 3368 | 1889 | 1876 |
| Row 95: | 2740 | 3226 | 1822 | 1498 | 1835 | 1984 | 3341 | 2844 | 3394 | 957 | 3237 | 2883 | 2483 | 1455 | 1509 | 2520 |
| Row 96: | 1010 | 1717 | 2857 | 2287 | 1278 | 3151 | 983 | 2425 | 2762 | 2477 | 1631 | 1929 | 3712 | 2654 | 1848 | 3178 |
| Row 97: | 2530 | 3602 | 2667 | 3225 | 1821 | 2854 | 1497 | 2503 | 2422 | 1739 | 1926 | 1764 | 1004 | 1874 | 1658 | 2896 |
| Row 98: | 3393 | 3272 | 1710 | 1468 | 3222 | 3339 | 3364 | 2367 | 2300 | 3596 | 1494 | 1399 | 3127 | 3164 | 3235 | 2738 |
| Row 99: | 1831 | 1009 | 2711 | 1507 | 955 | 1277 | 1868 | 2680 | 2842 | 2948 | 2761 | 1715 | 2653 | 2529 | 2516 | 3177 |
| Row 100: | 2462 | 1752 | 3601 | 894 | 2475 | 2853 | 1873 | 1925 | 2421 | 1952 | 3352 | 1657 | 2732 | 3248 | 2758 | 1819 |
| Row 101: | 1439 | 1709 | 3174 | 1520 | 1844 | 975 | 3221 | 2663 | 1762 | 2271 | 1493 | 1735 | 1654 | 1002 | 2737 | 2841 |
| Row 102: | 1466 | 3362 | 2488 | 3268 | 3335 | 3162 | 1866 | 3594 | 2461 | 2298 | 2888 | 2414 | 1275 | 1599 | 893 | 2514 |
| Row 103: | 2838 | 2651 | 2946 | 2622 | 3214 | 1391 | 3119 | 2676 | 3173 | 2851 | 2757 | 1923 | 2784 | 2703 | 3648 | 988 |
| Row 104: | 951 | 1761 | 1748 | 1486 | 2419 | 1694 | 2471 | 1707 | 1842 | 1465 | 3219 | 2508 | 1815 | 2730 | 3361 | 3348 |
| Row 105: | 1653 | 1001 | 1865 | 511 | 3240 | 3161 | 2413 | 1491 | 2864 | 3593 | 1462 | 2513 | 1936 | 3213 | 2297 | 3266 |
| Row 106: | 2837 | 1512 | 1836 | 998 | 3132 | 1862 | 2459 | 1404 | 2945 | 2621 | 3158 | 2716 | 2484 | 1720 | 1485 | 3171 |
| Row 107: | 3590 | 2755 | 1693 | 1646 | 2294 | 2884 | 1841 | 2674 | 2647 | 2729 | 891 | 1746 | 1271 | 986 | 3456 | 2239 |
| Row 108: | 2506 | 3346 | 2830 | 1461 | 1651 | 2726 | 3265 | 3103 | 2398 | 1703 | 1861 | 3184 | 2411 | 2668 | 1888 | 1740 |
| Row 109: | 3211 | 1807 | 1375 | 2768 | 3236 | 943 | 997 | 3157 | 2835 | 766 | 1645 | 3340 | 1834 | 3130 | 2293 | 2673 |
| Row 110: | 2619 | 1745 | 2856 | 3589 | 1483 | 1508 | 2714 | 1402 | 3616 | 2482 | 1928 | 2455 | 1454 | 1691 | 2829 | 2424 |
| Row 111: | 2882 | 3345 | 1716 | 3224 | 985 | 2505 | 2725 | 2397 | 1820 | 3150 | 2639 | 956 | 1859 | 1459 | 3129 | 2476 |
| Row 112: | 1496 | 2502 | 982 | 2286 | 1263 | 887 | 3234 | 995 | 1833 | 765 | 2666 | 2407 | 1401 | 2713 | 3207 | 1738 |
| Row 113: | 1453 | 3155 | 3126 | 2481 | 1630 | 1506 | 3338 | 1643 | 3176 | 2881 | 2760 | 2291 | 1830 | 1479 | 3587 | 1398 |
| Row 114: | 2827 | 2710 | 2852 | 3149 | 927 | 1872 | 2723 | 2652 | 2615 | 1008 | 3392 | 2395 | 2366 | 2420 | 3233 | 3220 |
| Row 115: | 2501 | 1276 | 1343 | 981 | 2285 | 2528 | 2447 | 2665 | 1714 | 1687 | 1656 | 1818 | 1924 | 954 | 1737 | 3125 |
| Row 116: | 1629 | 1505 | 1492 | 2474 | 2840 | 1438 | 3337 | 2175 | 2736 | 3600 | 1451 | 2662 | 1829 | 763 | 2709 | 1397 |
| Row 117: | 1708 | 1734 | 879 | 3334 | 1639 | 2850 | 3147 | 1247 | 3172 | 2365 | 2756 | 974 | 2460 | 2823 | 2650 | 1713 |
| Row 118: | 3118 | 1817 | 953 | 3218 | 2499 | 2418 | 2270 | 1464 | 2473 | 2391 | 2283 | 2702 | 1437 | 2607 | 1274 | 2661 |
| Row 119: | 1864 | 1390 | 979 | 1490 | 1814 | 3123 | 1627 | 950 | 1922 | 1598 | 3212 | 3160 | 1652 | 1827 | 2470 | 1000 |
| Row 120: | 1733 | 2707 | 1760 | 1679 | 2412 | 2836 | 2849 | 3333 | 2512 | 892 | 1395 | 3360 | 2728 | 973 | 1447 | 3117 |
| Row 121: | 2649 | 3217 | 1484 | 1706 | 2296 | 510 | 2701 | 3170 | 2754 | 2417 | 2363 | 2269 | 2458 | 759 | 2620 | 1389 |
| Row 122: | 3143 | 1840 | 2646 | 3592 | 1489 | 1273 | 1813 | 2659 | 949 | 2469 | 1435 | 1921 | 1597 | 1692 | 1460 | 2279 |
| Row 123: | 863 | 2383 | 3210 | 1731 | 1270 | 1650 | 1860 | 2944 | 1623 | 2834 | 1705 | 3169 | 3156 | 3102 | 2753 | 2410 |
| Row 124: | 3331 | 2591 | 3115 | 2672 | 509 | 2457 | 3264 | 2699 | 971 | 2724 | 1806 | 996 | 1482 | 890 | 2645 | 1215 |
| Row 125: | 1702 | 942 | 1374 | 2504 | 2828 | 2267 | 2618 | 1387 | 1644 | 2238 | 2292 | 1744 | 2454 | 2467 | 2396 | 2359 |
| Row 126: | 1811 | 3209 | 3344 | 2833 | 1269 | 3128 | 2712 | 1649 | 1595 | 1832 | 947 | 1690 | 2409 | 984 | 3101 | 751 |
| Row 127: | 2638 | 1458 | 1431 | 3206 | 1805 | 3154 | 1400 | 3588 | 1858 | 2617 | 1481 | 889 | 1701 | 1373 | 2722 | 2480 |
| Row 128: | 941 | 2406 | 507 | 2453 | 2695 | 1615 | 994 | 2643 | 2237 | 3111 | 2614 | 3148 | 1452 | 1262 | 2826 | 1478 |
| Row 129: | 2664 | 967 | 886 | 1642 | 2290 | 2263 | 1383 | 2500 | 1689 | 2637 | 2394 | 3153 | 2880 | 3205 | 1457 | 3232 |
| Row 130: | 764 | 831 | 1857 | 1736 | 2708 | 2721 | 2446 | 3099 | 1267 | 1803 | 1686 | 3124 | 1828 | 2405 | 2351 | 2284 |
| Row 131: | 1699 | 1628 | 1591 | 980 | 993 | 1371 | 2613 | 926 | 1342 | 2825 | 1396 | 939 | 1477 | 2451 | 3336 | 1261 |
| Row 132: | 1504 | 1641 | 2289 | 1423 | 3586 | 2235 | 885 | 2472 | 3146 | 2393 | 2174 | 2822 | 1450 | 1816 | 2635 | 2445 |
| Row 133: | 1712 | 2660 | 1151 | 503 | 2498 | 1638 | 735 | 2606 | 2364 | 1685 | 3203 | 2706 | 2390 | 2255 | 952 | 2403 |
| Row 134: | 3122 | 1826 | 3095 | 878 | 925 | 762 | 2282 | 1732 | 1341 | 1436 | 2611 | 978 | 1626 | 3145 | 1799 | 2700 |
| Row 135: | 2648 | 1475 | 1246 | 3585 | 1394 | 1367 | 1678 | 1449 | 2821 | 1259 | 3216 | 2173 | 2848 | 2497 | 2605 | 1637 |
| Row 136: | 2705 | 2335 | 1583 | 883 | 3116 | 3142 | 2268 | 2231 | 935 | 2658 | 2416 | 2443 | 972 | 2468 | 1446 | 2389 |
| Row 137: | 2281 | 3332 | 2631 | 1683 | 1812 | 1388 | 1825 | 3121 | 877 | 1488 | 2362 | 1704 | 977 | 1625 | 761 | 2278 |
| Row 138: | 1272 | 1677 | 923 | 1730 | 1393 | 495 | 2698 | 1339 | 948 | 1245 | 1434 | 2456 | 1596 | 3141 | 2644 | 2657 |
| Row 139: | 1622 | 3168 | 2590 | 3087 | 2819 | 2171 | 758 | 2603 | 2382 | 1635 | 2752 | 1445 | 2266 | 2361 | 3114 | 2466 |
| Row 140: | 970 | 1255 | 1359 | 703 | 2439 | 2277 | 2223 | 862 | 1810 | 2387 | 1920 | 3208 | 875 | 2697 | 2358 | 1729 |
| Row 141: | 1386 | 1567 | 2832 | 508 | 2408 | 2616 | 3330 | 1648 | 1433 | 1675 | 1700 | 3100 | 2589 | 1621 | 2381 | 757 |
| Row 142: | 2694 | 2642 | 3139 | 1480 | 1804 | 2265 | 1243 | 1594 | 1214 | 1268 | 3113 | 1430 | 2452 | 2465 | 946 | 919 |
| Row 143: | 888 | 1443 | 1372 | 2236 | 1335 | 969 | 2599 | 861 | 1809 | 2636 | 2262 | 1688 | 2167 | 2357 | 2275 | 3110 |
| Row 144: | 1385 | 1614 | 479 | 3152 | 940 | 966 | 750 | 2641 | 2693 | 3329 | 1593 | 2720 | 871 | 1698 | 1456 | 2587 |
| Row 145: | 1382 | 2612 | 2379 | 2404 | 506 | 1429 | 1671 | 3098 | 945 | 2207 | 1619 | 3204 | 1213 | 2288 | 2350 | 1590 |
| Row 146: | 2450 | 755 | 1802 | 1640 | 2261 | 2824 | 1266 | 1239 | 3109 | 1476 | 2234 | 2634 | 859 | 1613 | 1370 | 830 |
| Row 147: | 884 | 965 | 2355 | 1856 | 749 | 2444 | 992 | 2392 | 1684 | 911 | 1697 | 1422 | 2691 | 1381 | 1327 | 1260 |
| Row 148: | 938 | 2159 | 3097 | 2349 | 639 | 2449 | 505 | 2254 | 1589 | 2610 | 1801 | 2583 | 2402 | 1427 | 2233 | 3144 |
| Row 149: | 2633 | 1265 | 1211 | 2375 | 1448 | 3094 | 3202 | 502 | 1369 | 2704 | 3107 | 2259 | 924 | 829 | 1798 | 1150 |
| Row 150: | 2230 | 1611 | 2604 | 1636 | 1421 | 2630 | 1474 | 2280 | 963 | 1582 | 1340 | 734 | 882 | 1682 | 2442 | 855 |
| Row 151: | 937 | 2172 | 747 | 2820 | 447 | 1366 | 2253 | 1231 | 2388 | 2609 | 2401 | 2334 | 934 | 2347 | 1379 | 876 |
| Row 152: | 2496 | 3120 | 976 | 1258 | 1587 | 501 | 3093 | 1676 | 1624 | 760 | 2629 | 2229 | 881 | 2656 | 827 | 3201 |
| Row 153: | 2575 | 922 | 733 | 1797 | 2441 | 1149 | 2143 | 1824 | 1581 | 1681 | 3140 | 2602 | 2360 | 1473 | 1244 | 1207 |
| Row 154: | 1311 | 1634 | 1392 | 933 | 1419 | 2333 | 1444 | 2251 | 2438 | 2386 | 2276 | 1257 | 1607 | 2170 | 1365 | 2696 |
| Row 155: | 494 | 3086 | 743 | 847 | 2222 | 1338 | 2818 | 2588 | 874 | 2343 | 1254 | 921 | 2380 | 2627 | 1674 | 3584 |
| Row 156: | 3091 | 2601 | 499 | 2227 | 1432 | 968 | 2264 | 1620 | 731 | 1633 | 3112 | 2385 | 756 | 702 | 1566 | 918 |
| Row 157: | 1795 | 1147 | 1579 | 1358 | 2437 | 931 | 860 | 2464 | 2598 | 2169 | 2331 | 823 | 1242 | 1728 | 2221 | 2356 |
| Row 158: | 3085 | 1337 | 493 | 3138 | 2640 | 1442 | 2274 | 873 | 2817 | 1253 | 1415 | 2247 | 2166 | 1199 | 1363 | 1673 |
| Row 159: | 383 | 1334 | 2586 | 2378 | 2692 | 944 | 1592 | 1384 | 1808 | 870 | 917 | 1565 | 701 | 2111 | 1618 | 1670 |
| Row 160: | 1357 | 2597 | 1241 | 2435 | 2260 | 1428 | 754 | 2273 | 727 | 1441 | 964 | 2206 | 3137 | 2219 | 858 | 1212 |
| Row 161: | 2354 | 2327 | 478 | 3083 | 1575 | 3108 | 491 | 1612 | 2165 | 1238 | 2377 | 910 | 2585 | 1143 | 1251 | 1333 |
| Row 162: | 869 | 748 | 815 | 2448 | 1617 | 2632 | 2348 | 2374 | 1669 | 2582 | 1696 | 915 | 2690 | 1563 | 2232 | 2595 |
| Row 163: | 753 | 1588 | 699 | 1355 | 1380 | 2158 | 936 | 2205 | 3096 | 2353 | 1183 | 504 | 1264 | 2258 | 857 | 1326 |
| Row 164: | 1426 | 638 | 909 | 1237 | 477 | 962 | 1800 | 1210 | 828 | 3328 | 1610 | 2163 | 854 | 2373 | 2608 | 719 |
| Row 165: | 2215 | 3106 | 2252 | 1331 | 867 | 2400 | 2319 | 1420 | 1368 | 2581 | 746 | 3079 | 2346 | 1667 | 487 | 2157 |
| Row 166: | 2689 | 2257 | 1135 | 1325 | 1425 | 1230 | 2628 | 1586 | 1559 | 1378 | 637 | 2440 | 2203 | 1209 | 2228 | 1609 |
| Row 167: | 732 | 695 | 961 | 1351 | 799 | 2574 | 475 | 2332 | 880 | 907 | 932 | 500 | 446 | 2371 | 1235 | 745 |
| Row 168: | 1680 | 3092 | 3105 | 1206 | 1580 | 1606 | 2345 | 826 | 853 | 255 | 1256 | 2250 | 2142 | 1418 | 2155 | 1148 |
| Row 169: | 2579 | 742 | 1796 | 1229 | 1364 | 1585 | 2342 | 1377 | 2600 | 1310 | 1323 | 920 | 2384 | 1472 | 2573 | 635 |
| Row 170: | 2626 | 846 | 730 | 445 | 1605 | 2226 | 1205 | 2199 | 2168 | 2330 | 825 | 2436 | 1632 | 1551 | 930 | 2249 |
| Row 171: | 1417 | 2141 | 1119 | 498 | 741 | 1578 | 903 | 1336 | 471 | 687 | 2341 | 3090 | 851 | 822 | 1309 | 3200 |
| Row 172: | 2220 | 2246 | 872 | 1362 | 1414 | 1227 | 1252 | 1146 | 1672 | 2151 | 1198 | 845 | 1794 | 2625 | 492 | 729 |
| Row 173: | 2329 | 1564 | 2225 | 2571 | 2596 | 3084 | 916 | 1319 | 1356 | 929 | 1603 | 2376 | 700 | 1577 | 1203 | 497 |
| Row 174: | 1240 | 2326 | 726 | 2139 | 443 | 1361 | 2164 | 2339 | 2434 | 631 | 739 | 821 | 2110 | 3089 | 2245 | 1413 |
| Row 175: | 2272 | 1574 | 2191 | 1145 | 1307 | 1440 | 382 | 1332 | 1197 | 2584 | 1616 | 2218 | 1250 | 1793 | 843 | 868 |
| Row 176: | 1223 | 1142 | 463 | 2352 | 671 | 2816 | 752 | 814 | 1562 | 1354 | 914 | 1668 | 2325 | 490 | 2594 | 725 |
| Row 177: | 2109 | 698 | 2204 | 1087 | 2433 | 3082 | 2567 | 1573 | 2243 | 1411 | 2372 | 2162 | 1236 | 819 | 2217 | 2135 |
| Row 178: | 1249 | 908 | 856 | 381 | 1182 | 1195 | 1330 | 1303 | 2318 | 476 | 439 | 1141 | 1353 | 3136 | 1561 | 2214 |
| Row 179: | 913 | 623 | 813 | 718 | 2580 | 2156 | 489 | 866 | 2593 | 2256 | 697 | 1424 | 1608 | 1350 | 1324 | 2323 |
| Row 180: | 839 | 1558 | 3081 | 1208 | 2344 | 1571 | 2161 | 1666 | 2107 | 486 | 2202 | 723 | 1134 | 694 | 1329 | 1181 |
| Row 181: | 1234 | 2317 | 744 | 2370 | 906 | 2213 | 636 | 3078 | 717 | 852 | 379 | 865 | 1139 | 798 | 474 | 1349 |
| Row 182: | 811 | 2127 | 1584 | 1191 | 1228 | 1295 | 1376 | 960 | 1557 | 2154 | 2578 | 2201 | 1322 | 1133 | 485 | 1665 |
| Row 183: | 431 | 1233 | 2688 | 693 | 254 | 1604 | 2248 | 905 | 2369 | 1416 | 1179 | 2315 | 2198 | 2340 | 1204 | 607 |
| Row 184: | 824 | 2211 | 2572 | 2140 | 1550 | 1308 | 2103 | 797 | 473 | 850 | 3077 | 902 | 715 | 740 | 1347 | 634 |
| Row 185: | 2153 | 1321 | 3104 | 686 | 444 | 2577 | 1226 | 1118 | 1131 | 1555 | 470 | 2328 | 2224 | 807 | 483 | 844 |
| Row 186: | 1318 | 375 | 2150 | 1576 | 253 | 2197 | 1360 | 691 | 728 | 1549 | 849 | 1602 | 2338 | 901 | 2244 | 1202 |
| Row 187: | 1175 | 1144 | 1306 | 633 | 928 | 795 | 1412 | 2138 | 685 | 820 | 1225 | 2311 | 415 | 1117 | 2570 | 738 |
| Row 188: | 496 | 2190 | 1317 | 3075 | 442 | 469 | 2149 | 711 | 2095 | 630 | 1222 | 1196 | 842 | 1127 | 2624 | 2195 |
| Row 189: | 2324 | 1601 | 251 | 2337 | 1305 | 1201 | 575 | 1547 | 1572 | 2216 | 2137 | 899 | 367 | 724 | 462 | 1352 |
| Row 190: | 737 | 2189 | 2242 | 2569 | 670 | 1302 | 1115 | 683 | 3088 | 441 | 1140 | 1315 | 818 | 2108 | 1410 | 791 |
| Row 191: | 1086 | 1221 | 2134 | 841 | 1167 | 629 | 1248 | 2147 | 1560 | 467 | 1194 | 2566 | 438 | 912 | 488 | 838 |
| Row 192: | 812 | 1328 | 380 | 2322 | 696 | 461 | 1792 | 1301 | 1570 | 622 | 2241 | 2432 | 817 | 669 | 2079 | 2187 |
| Row 193: | 2212 | 722 | 1180 | 1085 | 2160 | 1409 | 1543 | 247 | 1219 | 1348 | 2106 | 2133 | 1138 | 1193 | 1111 | 2316 |
| Row 194: | 437 | 627 | 837 | 1294 | 679 | 716 | 2565 | 783 | 864 | 1190 | 351 | 2592 | 2321 | 2200 | 810 | 1556 |
| Row 195: | 1132 | 621 | 1232 | 1569 | 484 | 459 | 378 | 430 | 2126 | 721 | 1299 | 3080 | 1320 | 2105 | 1137 | 1083 |
| Row 196: | 1178 | 2210 | 667 | 904 | 692 | 1293 | 1346 | 2183 | 435 | 796 | 2131 | 472 | 1189 | 2102 | 2314 | 835 |
| Row 197: | 2152 | 239 | 809 | 1103 | 377 | 429 | 2563 | 714 | 2125 | 606 | 482 | 1130 | 619 | 1554 | 1177 | 2196 |
| Row 198: | 806 | 2209 | 455 | 2368 | 374 | 1664 | 1224 | 848 | 1345 | 1291 | 1174 | 1316 | 2101 | 1187 | 1548 | 1116 |
| Row 199: | 252 | 319 | 2313 | 1079 | 690 | 794 | 632 | 663 | 900 | 2576 | 468 | 713 | 1129 | 481 | 605 | 414 |
| Row 200: | 1553 | 427 | 3076 | 805 | 2310 | 2123 | 2148 | 1304 | 373 | 1200 | 1126 | 710 | 684 | 2094 | 2194 | 1173 |
| Row 201: | 615 | 223 | 793 | 1314 | 689 | 1220 | 440 | 2099 | 1114 | 2188 | 366 | 1287 | 2136 | 840 | 1546 | 413 |
| Row 202: | 250 | 466 | 1166 | 790 | 603 | 628 | 803 | 898 | 1125 | 574 | 2309 | 1071 | 709 | 2193 | 2093 | 655 |
| Row 203: | 736 | 2336 | 371 | 2146 | 1171 | 423 | 460 | 1600 | 1300 | 682 | 1313 | 1192 | 2119 | 1113 | 365 | 3074 |
| Row 204: | 816 | 2568 | 465 | 1165 | 1218 | 1545 | 249 | 789 | 1084 | 411 | 1110 | 573 | 2186 | 668 | 436 | 897 |
| Row 205: | 1123 | 2132 | 707 | 836 | 2145 | 1542 | 626 | 246 | 681 | 599 | 2078 | 2091 | 2307 | 782 | 2240 | 1298 |
| Row 206: | 191 | 458 | 350 | 678 | 363 | 1217 | 1408 | 1055 | 1163 | 1188 | 620 | 1136 | 720 | 2185 | 1082 | 787 |
| Row 207: | 1109 | 3073 | 1292 | 2104 | 808 | 625 | 571 | 245 | 666 | 2320 | 1541 | 2077 | 434 | 2130 | 781 | 376 |
| Row 208: | 407 | 2182 | 1176 | 1297 | 834 | 1568 | 457 | 677 | 349 | 2564 | 1102 | 591 | 1081 | 2087 | 1186 | 454 |
| Row 209: | 2124 | 428 | 238 | 618 | 665 | 359 | 1159 | 1107 | 1078 | 2129 | 433 | 1290 | 1128 | 712 | 833 | 2181 |
| Row 210: | 779 | 662 | 804 | 243 | 2075 | 675 | 1539 | 1101 | 2208 | 604 | 2100 | 567 | 347 | 1172 | 372 | 1185 |
| Row 211: | 318 | 453 | 1344 | 617 | 237 | 399 | 480 | 2312 | 426 | 1077 | 792 | 2122 | 688 | 1289 | 614 | 2562 |
| Row 212: | 127 | 661 | 1552 | 2179 | 802 | 1286 | 1099 | 1124 | 708 | 317 | 412 | 1070 | 602 | 1170 | 775 | 451 |
| Row 213: | 2098 | 2071 | 654 | 370 | 425 | 343 | 222 | 235 | 2121 | 613 | 559 | 1075 | 1164 | 1112 | 422 | 2192 |
| Row 214: | 659 | 788 | 2561 | 2118 | 2092 | 801 | 364 | 680 | 464 | 1285 | 1169 | 601 | 1069 | 1312 | 653 | 1122 |
| Row 215: | 248 | 706 | 2308 | 369 | 2097 | 410 | 572 | 315 | 221 | 1095 | 598 | 611 | 1544 | 421 | 2117 | 1162 |
| Row 216: | 231 | 335 | 2144 | 786 | 2063 | 1108 | 1121 | 896 | 362 | 705 | 1054 | 543 | 1067 | 1283 | 651 | 676 |
| Row 217: | 409 | 2090 | 624 | 1216 | 570 | 597 | 456 | 780 | 219 | 244 | 406 | 2184 | 190 | 419 | 348 | 1161 |
| Row 218: | 311 | 2306 | 785 | 664 | 2076 | 1080 | 1296 | 361 | 590 | 2115 | 1053 | 1106 | 1158 | 2089 | 569 | 674 |
| Row 219: | 1540 | 432 | 358 | 647 | 405 | 595 | 189 | 566 | 1063 | 2086 | 1100 | 778 | 616 | 242 | 2128 | 215 |
| Row 220: | 452 | 589 | 1105 | 346 | 2305 | 1157 | 660 | 673 | 398 | 832 | 1051 | 1184 | 2074 | 1076 | 2180 | 357 |
| Row 221: | 236 | 303 | 565 | 777 | 2085 | 403 | 1288 | 241 | 187 | 424 | 1098 | 1538 | 345 | 3072 | 774 | 587 |
| Row 222: | 612 | 126 | 397 | 450 | 558 | 658 | 316 | 1155 | 2073 | 342 | 207 | 355 | 1074 | 1047 | 234 | 2120 |
| Row 223: | 563 | 652 | 600 | 1097 | 2070 | 2178 | 2083 | 125 | 773 | 1168 | 657 | 557 | 1537 | 220 | 800 | 1094 |
| Row 224: | 1068 | 183 | 610 | 287 | 449 | 420 | 395 | 368 | 233 | 583 | 314 | 1284 | 1073 | 341 | 2096 | 230 |
| Row 225: | 650 | 2069 | 408 | 596 | 609 | 1093 | 2177 | 542 | 771 | 555 | 1039 | 123 | 334 | 1120 | 218 | 2116 |
| Row 226: | 704 | 313 | 1066 | 418 | 229 | 2062 | 391 | 175 | 339 | 649 | 1160 | 310 | 568 | 360 | 784 | 1282 |
| Row 227: | 1052 | 541 | 594 | 646 | 333 | 1091 | 217 | 2067 | 404 | 1065 | 417 | 2560 | 2088 | 188 | 551 | 588 |
| Row 228: | 214 | 227 | 119 | 309 | 1062 | 2061 | 2114 | 593 | 1104 | 645 | 1281 | 672 | 539 | 564 | 331 | 356 |
| Row 229: | 1050 | 159 | 1156 | 302 | 402 | 240 | 776 | 186 | 213 | 1061 | 586 | 307 | 2113 | 396 | 344 | 2059 |
| Row 230: | 2084 | 643 | 111 | 1049 | 401 | 301 | 206 | 562 | 354 | 535 | 185 | 585 | 1096 | 327 | 1046 | 1154 |
| Row 231: | 1059 | 211 | 656 | 182 | 2072 | 582 | 556 | 232 | 394 | 124 | 772 | 205 | 340 | 561 | 353 | 286 |
| Row 232: | 2304 | 299 | 2082 | 1072 | 448 | 1045 | 2055 | 581 | 181 | 608 | 527 | 1153 | 393 | 95 | 554 | 312 |
| Row 233: | 1092 | 285 | 338 | 390 | 203 | 228 | 122 | 648 | 1038 | 174 | 2068 | 2081 | 770 | 540 | 295 | 332 |
| Row 234: | 579 | 553 | 1043 | 179 | 216 | 1536 | 337 | 1064 | 389 | 550 | 416 | 121 | 283 | 173 | 308 | 1037 |
| Row 235: | 1090 | 592 | 226 | 769 | 199 | 118 | 538 | 330 | 644 | 2066 | 2176 | 63 | 549 | 387 | 212 | 225 |
| Row 236: | 158 | 171 | 1089 | 2060 | 306 | 279 | 1035 | 1060 | 117 | 329 | 537 | 400 | 2065 | 584 | 300 | 326 |
| Row 237: | 534 | 184 | 547 | 642 | 110 | 305 | 157 | 1048 | 210 | 1280 | 115 | 560 | 325 | 167 | 1058 | 204 |
| Row 238: | 352 | 271 | 2058 | 533 | 1031 | 298 | 109 | 641 | 209 | 580 | 392 | 155 | 180 | 2112 | 526 | 284 |
| Row 239: | 323 | 1044 | 1057 | 297 | 2057 | 202 | 94 | 107 | 531 | 294 | 552 | 336 | 2054 | 525 | 578 | 178 |
| Row 240: | 151 | 120 | 282 | 388 | 201 | 93 | 1042 | 293 | 1152 | 172 | 198 | 103 | 577 | 2053 | 281 | 177 |
| Row 241: | 523 | 548 | 1036 | 2080 | 328 | 278 | 91 | 386 | 116 | 291 | 62 | 1041 | 197 | 143 | 224 | 536 |
| Row 242: | 170 | 768 | 304 | 277 | 546 | 61 | 156 | 385 | 1034 | 2051 | 519 | 195 | 324 | 114 | 169 | 87 |
| Row 243: | 270 | 1088 | 166 | 532 | 108 | 545 | 208 | 275 | 2064 | 1033 | 296 | 113 | 59 | 154 | 269 | 322 |
| Row 244: | 165 | 1030 | 79 | 106 | 530 | 153 | 640 | 200 | 321 | 267 | 150 | 292 | 163 | 524 | 92 | 55 |
| Row 245: | 1029 | 1056 | 105 | 529 | 2056 | 280 | 176 | 102 | 149 | 290 | 196 | 90 | 263 | 522 | 142 | 1027 |
| Row 246: | 101 | 47 | 576 | 147 | 276 | 289 | 168 | 89 | 141 | 521 | 194 | 1040 | 60 | 2052 | 86 | 99 |
| Row 247: | 518 | 274 | 384 | 193 | 31 | 112 | 139 | 164 | 58 | 85 | 268 | 517 | 544 | 273 | 152 | 2050 |
| Row 248: | 78 | 1032 | 57 | 162 | 135 | 83 | 54 | 266 | 104 | 515 | 77 | 148 | 161 | 320 | 2049 | 53 |
| Row 249: | 265 | 528 | 1028 | 262 | 100 | 75 | 46 | 146 | 51 | 140 | 88 | 261 | 145 | 45 | 288 | 98 |
| Row 250: | 71 | 1026 | 520 | 138 | 84 | 97 | 30 | 43 | 259 | 192 | 137 | 1025 | 56 | 272 | 29 | 82 |
| Row 251: | 134 | 516 | 76 | 39 | 81 | 133 | 27 | 52 | 160 | 74 | 264 | 514 | 131 | 50 | 23 | 73 |
| Row 252: | 144 | 70 | 44 | 513 | 49 | 260 | 2048 | 69 | 15 | 96 | 42 | 136 | 28 | 258 | 67 | 41 |
| Row 253: | 38 | 80 | 26 | 257 | 132 | 37 | 25 | 1024 | 72 | 22 | 35 | 130 | 48 | 21 | 68 | 129 |
| Row 254: | 14 | 19 | 40 | 13 | 66 | 512 | 36 | 11 | 65 | 24 | 34 | 7 | 256 | 20 | 33 | 18 |
| Row 255: | 12 | 128 | 17 | 10 | 9 | 64 | 6 | 5 | 32 | 3 | 16 | 8 | 4 | 2 | 1 | 0 |