3GPP TSG RAN WG1 Meeting #86bis                                                         R1-1608663

Lisbon, Portugal, 10th – 14th October 2016

Source:	ZTE, ZTE Microelectronics

Title:	Calibration and evaluation assumptions for NR MIMO

Agenda Item:	8.1.4.5

Document for: 	Discussion and Decision

Introduction

In RAN1#86, the agreements of starting MIMO calibration have been made [1]. The consensus on the following phased approach and calibration related assumptions for phase 1 have been reached in [86-20] email discussion [2].

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

- Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

- Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

This contribution makes further clarifications on Phase 1 calibration assumptions, and then elaborates the assumptions for Phase 2 & 3 calibration.

Further clarifications on Phase 1 calibration assumption

#1 Receive SNR of link-level

In [86-20] email discussion, the CDF of receive SNR w/ beamforming at SNR=0dB the receiver SNR is adopted the metric of link level calibration. Highlighted that this receive SNR here corresponds to the receive SNR over all subcarriers and OFDM symbols with linear average.  A lot of drops are done to cover different translation angle settings.

Observation 1:  The receive SNR, as a metric of link level calibration, is over all subcarriers of one OFDM symbol with linear average.

#2 Directions of beams in indoor hotpot

The MIMO calibration should follow this agreed option 1 of 12 sites with antenna placement in [86-27] for indoor hotpot [3], which has been referred to in MIMO calibration summary. The boresight of antenna placement of these panels is perpendicular to the ceiling, instead of in parallel with the ceiling. In this scenario, the traditional antenna array perpendicular to ground is rotated 90 degrees along clockwise direction and mounted on the ceiling in the calibration layout. That means that the “vertical side” (i.e., “M”) and (“horizontal side” (i.e., “N”) of the array is placed according to Figure 1.

The whole selective beams from one TRP-TXRU should cover the whole hemisphere, and then one beam with the suitable boresight will be selected accordingly. According to this antenna placement of indoor scenario, it is recommended that the directions of beams are within [0, 180] in azimuth and [0, 180] in zenith according to the aforementioned placement of antenna array, instead of [-180, 180] degrees in azimuth domain and [90, 180] degrees in zenith domain.

Observation 2:  Taking into antenna placement in indoor scenario, the directions of beams are within [0, 180] in azimuth and [0, 180] in zenith, instead of [-180, 180] degrees in azimuth domain and [90, 180] degrees in zenith domain.

Figure 1 Placement of antenna array in indoor hotspot

Phase 2 calibration assumptions

Link level calibration

#1 Channel model

In Phase 1, CDL-A and CDL-B channels both are calibrated. In order to accelerate the Phase 2 calibration, one CDL channel would be sufficient for calibration as a proposal.  CDL-A and CDL-B correspond to different scenarios. CDL-A has less dominant channel direction (e.g. Indoor) while CDL-B has more dominant channel directions (e.g. UMi). It is suggested to calibrate CDL-B, which has more support according to the [86-20] email discussion. The other parameter related to channel model remains the same as Phase 1 calibration.

Proposal 1:  Calibrate CDL-B with delay spread =100ns and UE speed=3km/h for Phase 2 link-level calibration.  The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly.

#2 Beam sweeping and selection

Introducing beam sweeping into phase 2 calibration for LLS is necessary for aligning time-varying fast fading channel, e.g. with Doppler shift, and its impact on beam selection. To be more specific, instead of determining beam at the same time, the beam measurement should be modelled in more realistic manner e.g. several OFDM symbols. One assumption we can make is to have each analog beam (weights per panel) pair independently occupying one OFDM symbol.  Without loss of generality, the typical exhaustive sweeping for any TX and RX beam pairs can be used as shown in the following Figure 2. The beam pair by maximizing received signal power over all subcarriers is selected.

Figure 2 Diagram of beam sweeping

For instance, the numbers of TRP and UE beams with oversampling factor = 2 are X=(4*2)*(8*2)=128 and Y=(2*2)*(4*2)= 32 according to Table 1. The number of probing beam pair is X*Y = 4096 in the exhaustive sweeping;

Proposal 2: Beam sweeping is done in a way that each TX and RX beam pair is associated with one OFDM symbol. The best beam pair is selected by maximizing received signal power over all subcarriers with linear average for beam selection in Phase 2 link-level calibration.

#3 Other parameters

According to Phase 1 calibration assumption, the following Table 1 summarizes other parameters for this link level calibration for phase 2. It should be highlighted that the metric for Phase 2 calibration is BLER with beamforming gain as a function of SNR ranging from -20 dB to 0dB, where the fixed LTE MCS=1 is chosen.

Table 1 Other parameters for Phase 2 link-level calibration

| Parameter | Values |
| --- | --- |
| BS antenna configurations | 30GHz: (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ 4GHz: (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ |
| BS antenna pattern | Refer to [4] |
| UE antenna configurations | 30GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 Notes: introduce (Ωmg,ng, Θmg,ng) for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng, where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873]. The panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels. 4GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1) |
| BS array orientation | azimuth 0 degree; downtilt: 110 degree |
| TXRU mapping to antenna elements & TXRU mapping weights | For 30GHz, a single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. For 4GHz, TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x N x 2. |
| Carrier frequency, bandwidth & subcarrier spacing | Carrier frequency = 4GHz, bandwidth = 20MHz, subcarrier spacing = 15kHz. Carrier frequency = 30GHz, bandwidth = 80MHz, subcarrier spacing = 60kHz |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | Refer to [4] |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |
| MIMO mode | SU-MIMO with rank=1 |
| UE receiver type | MMSE-IRC |
| MCS | LTE MCS = 1 |
| Metric | BLER with beamforming gain as a function of SNR ranging from -30 dB to 0dB. |

Proposal 3: Adopt Table 1 for other parameters for Phase 2 link-level calibration.

System level calibration

#1 Scenarios (including carrier frequency)

The scenarios calibrated in Phase 1 are all essential for the subsequent performance evaluation but have different configurations in terms of layout, UE distribution and carrier frequency. Therefore these scenarios are all recommended for Phase 2 calibration.

Proposal 4:   Calibrate the following scenarios in the Phase 2:

Indoor hotspot (carrier frequency 30GHz), Urban macro (carrier frequency 30GHz), Dense Urban (macro layer carrier frequency 4GHz, micro layer carrier frequency 30GHz)

Independent calibration for macro layer and micro layer for dense urban scenario.

- Macro layer: macro-only 10 users per TRP, all UEs are connected to macro layer. Users randomly and uniformly dropped throughout the macro geographical area

- Micro layer: micro-only 3 micro BSs per macro BS and 10 users per TRP

#2 Constraints for the range of selective beams per TRP sector

The following proposal follows that in Phase 1 calibration while considering the further clarification in Section 2.

Proposal 5:  Calibrate the following beam constraints for TRP in the Phase 2:

Urban macro: three sectors; the directions of beams, e.g., DFT codebook, are within [-60, 60] degrees in azimuth domain and [90, 160] degrees in zenith domain, taking into account the UE’s distribution;

Indoor hotspot: Indoor hotspot follows Option 1 of 12 sites agreed in email discussion [86-27].

The directions of beams, e.g., DFT codebook, are within [0, 180] degrees in azimuth domain and [0, 180] degrees in zenith domain, taking into account the UE distribution.

#3 Other parameters

According to Phase 1 calibration assumption, the following Table 2 summarizes other parameters for this link level calibration for phase 2. It should be highlighted that the CDF of spectral efficiency is as metric for this calibration, which has been agreed in [86-20].

Table 2 Other parameters for for Phase 2 system-level calibration

| Parameter | Values |
| --- | --- |
| BS Tx power | Refer to [4] |
| Noise figure for BS | Below 6GHz: 5 dB; Above 6GHz: 7 dB |
| Noise figure for UE | Below 6GHz: 9 dB; Above 6GHz: 10dB |
| Transmission scheme | DL |
| BS antenna configurations | Refer to the baselines in [4] and Option 1 of Indoor Hotspot agreed in [86-27]. |
| BS antenna pattern | Refer to [4] |
| UE antenna configurations | 30GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 The panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels. 4GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1) |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | Refer to [4] |
| TXRU mapping to antenna elements | For 30GHz, a single TXRU per panel per polarization is used. 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x 2. For 4GHz, TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT with oversampling factor=1. Number of TXRUs= Mg x Ng x N x 2. |
| Carrier frequency, bandwidth & subcarrier spacing | Carrier frequency = 4GHz, bandwidth = 20MHz, subcarrier spacing = 15kHz. Carrier frequency = 30GHz, bandwidth = 80MHz, subcarrier spacing = 60kHz |
| Criteria for analog beam selection | Search the beam pair with the objective of maximizing RSRP (post beamforming) |
| Scheduling algorithm | PF scheduler as the scheduler algorithm with SU-MIMO wideband |
| Traffic Model | Full buffer |
| Inter-panel calibration | Ideal |
| Control overhead | Zero |
| UE receiver type | MMSE-IRC |
| Number of UEs | Refer to [4] |
| UE distribution | Refer to [4] |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |
| MIMO mode | SU-MIMO with rank=1 |
| MCS | Use LTE MCS |

NOTE: Any other parameters not specified follow [4].

Proposal 6: Adopt Table 2 for other parameters for Phase 2 system-level calibration.

Phase 3 evaluation assumption

With the aims to check UE movement/rotation/blockage defined in [2], Phase 3 calibration is to add some additional features in SLS calibration. These assumptions and metrics for Phase 2 calibration are adopted as a baseline for Phase 3 calibration. Meanwhile the calibration scenarios can be simplified accordingly, like just choosing one scenario such as urban macro (carrier frequency 30GHz), taking into account the same assumptions used for Phase 2 calibration.

Proposal 7: The assumptions and metrics for Phase 2 calibration with additional features containing UE movement/rotation/blockage is adopted as a baseline for the Phase 3 calibration.

Conclusion

This contribution makes some further clarifications on Phase 1 calibration, and discusses the link-level and system-level calibration for Phase 2 and Phase 3, according to which the following observations and proposals are drawn.

Observation 1:  The receive SNR, as a metric of link level calibration, is over all subcarriers of one OFDM symbol with linear average.

Observation 2:  Taking into antenna placment in indoor scenario, the directions of beams are within [0, 180] in azimuth and [0, 180] in zenith, instead of [-180, 180] degrees in azimuth domain and [90, 180] degrees in zenith domain.

Proposal 1:  Calibrate CDL-B with delay spread =100ns and UE speed=3km/h for Phase 2 link-level calibration.  The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly.

Proposal 2: To continuously sweep any TX and RX beam pair associated with one OFDM symbol, and subsequently select the beam pair by maximizing received signal power over all subcarriers with linear average for beam selection in Phase 2 link-level calibration.

Proposal 3: Adopt Table 1 for other parameters for Phase 2 link-level calibration.

Proposal 4:   Calibrate the following scenarios in the Phase 2:

Indoor hotspot (carrier frequency 30GHz), Urban macro (carrier frequency 30GHz), Dense Urban (macro layer carrier frequency 4GHz, micro layer carrier frequency 30GHz).

Independent calibration for macro layer and micro layer for dense urban scenario.

Macro layer: macro-only 10 users per TRP, all UEs are connected to macro layer. Users randomly and uniformly dropped throughout the macro geographical area

Micro layer: micro-only 3 micro BSs per macro BS and 10 users per TRP

Proposal 5:  Calibrate the following beam constraints for TRP in the Phase 2:

Urban macro: three sectors; the directions of beams, e.g., DFT codebook, are within [-60, 60] degrees in azimuth domain and [90, 160] degrees in zenith domain, taking into account the UE’s distribution;

Indoor hotspot: Indoor hotspot follows Option 1 of 12 sites agreed in email discussion [86-27].

The directions of beams, e.g., DFT codebook, are within [0, 180] degrees in azimuth domain and [0, 180] degrees in zenith domain, taking into account the UE distribution.

Proposal 6: Adopt Table 2 for other parameters for Phase 2 system-level calibration.

Proposal 7: These assumption and metrics for Phase 2 calibration with additional features containing UE movement/rotation/blockage is adopted as a baseline for the Phase 3 calibration.

- References

- R1-168487, WF on NR MIMO calibration, ZTE Corporation, ZTE Microelectronics, Intel Corporation, Samsung, ASTRI, Xinwei, MediaTek, Nokia, ASB, Ericsson, NTT DOCOMO, Qualcomm, CATT

- R1-1608661, Summary of [86-20] email discussion on MIMO calibration for NR, ZTE

- [86-27] email discussion on Indoor TRP Configuration, NTT DOCOMO

- R1-168547, Offline discussion further updated summary of evaluation assumptions for NR, NTT DOCOMO