3GPP TSG RAN WG1 Meeting #86bis	                                                              R1-1608684

Lisbon, Portugal 10th - 14th October 2016

# Source: 	ZTE, ZTE Microelectronics

Title:	Feedback mechanism for linear combination based CSI

Agenda Item:	7.2.2.1.3

Document for:	Discussion and Decision

- Introduction

In RAN1#86, the following agreement is achieved on advanced CSI reporting.

Agreement:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

In [1]-[3], linear combination (LC) based CSI feedback is proposed as a novel CSI reporting scheme with significant performance gain over the legacy codebook based CSI feedback. In this contribution, we discuss some detailed issues involved in the feedback of LC based CSI.

- Discussion on feedback mechanism for LC based CSI

The linear combination of  beams for each pol can be expressed as

(2-1)

where

where  and  are the number of ports of the first and second dimension respectively,  and  are the oversampling factors, and  are relative amplitude and phase respectively, and .

2.1 Feedback overhead analysis

It can be deduced from (2-1) that the CSI feedback of LC codeword involves beam selections, amplitude coefficients and phase coefficients for different polarizations and different layers.

Feedback overhead for beam selection

For the issue of beam selection, W1-based scheme and orthogonal basis scheme have been raised and discussed in [1]-[3]. The W1-base scheme does not require additional beam selection indicator since it can reuse the legacy i1 signal, so the feedback overhead of W1-based scheme is  bits, where s1 and s2 are determined by codebook-config. In [1], we have discussed the rationale and benefit of orthogonal basis used to generate LC codebook. Meanwhile, since the difference of the reduced space of two polarizations is small, the beam selection information for each polarization can be the same. Therefore the beam selection feedback overhead of proposed scheme is , where M is the number of pre-defined beam pattern groups. In our design, the value of M is not greater than 4, which means that at most 2 additional bits are required compared with W1-based scheme. In [3], another orthogonal basis beam selection scheme was discussed, where it defines new signals (q1, q2) and (n1, n2) to indicate beam selection information, which leads to  bits feedback overhead.

Figure 1 Wideband beam selection vs. sub-band beam selection in different antenna setup

Figure 1 shows the system performance comparison between wideband beam selection scheme and sub-band beam selection scheme. It can be observed that sub-band beam selection scheme provide less than 5% mean performance gain over the former scheme. Although the cell edge performance has some loss for WB beam selection, considering the feedback overhead reduction, the beam selection matrix should be fed back by wideband.

From aspect of spec impact, the W1-based scheme requires the least change since it can reuse legacy signaling directly. As for proposed scheme, it only needs to pre-define some orthogonal beam patterns for different antenna layouts which is an easy task based on our study [1]. Obviously, the scheme in [3] impacts specification the most since it needs to re-define W1 and introduce new signal indicators (q1, q2) and (n1, n2).

Proposal 1: Support beam selection with pre-defined orthogonal beam selection pattern.

Proposal 2: Considering overhead reduction, beam selection information should be fed back in wideband.

Feedback overhead for coefficients

For LC CSI, there are  amplitude coefficients and  phase coefficients for each layer. Assume that 2 bits and 3 bits are used to quantize amplitude coefficient and phase coefficient, respectively. Then the feedback overhead for a rank 2 codeword can be calculated as bits.

Let us consider the amplitude coefficients first. Regarding the feedback overhead and performance of amplitude overhead, the following issues need to be studied.

- Whether amplitude should be reported

- Wideband or sub-band

- Whether use same amplitude for two layers

We simulated all these possible schemes, the results are shown in Figure 2. It can be seen from the simulation results that using wideband amplitudes causes marginal mean performance gain loss and about 8%~10% cell edge performance loss compared with sub-band feedback. For the case that two layers share the same amplitude feedback, the mean performance loss is not large, but cell edge performance suffers significant loss. For the phase-only scheme, about 10% mean performance loss and at least 20% cell edge performance loss observed compared with legacy codebook.

Figure 2. Performance comparison among different amplitude feedback schemes

For the phase feedback, as pointed out in several contributions, it should be reported in sub-band. In fact, phase feedback requires more feedback overhead in the LC based CSI. Hence the overhead reduction for phase feedback needs to be studied. One approach is to allow unequal feedback bit allocation for the selected beams. Specifically, for the weaker beams in the beam group, they can be allocated fewer feedback bits. By doing this, the performance is not degraded significantly, but the feedback overhead is reduced. Another approach is to allow the same phase feedback for different layers. As we can observe above, the cell edge performance suffers more impact if the same coefficients are shared in two layers. We compare the cell-edge performance of the LC feedback with the same coefficients for two layers and the legacy Class A codebook in Figure 3. It can be observed that we can still have significant cell-edge performance gain.

Figure 3 Performance comparison of cell edge user in different antenna system

Based on above discussion, in order to reduce feedback overhead, we have the following proposals.

Proposal 3: Both amplitude and phase coefficients should be contained in the LC CSI feedback.

  - Amplitude feedback should be allowed to be wide-band.

  - Coefficients should be allowed to be the same for two layers.

  - Unequal feedback bit allocation for coefficients of the selected beams should be supported.

2.2 Feedback mechanism for PUCCH

Report modes for the CSI components

If all the coefficients are fed per sub-band, it will lead to an unacceptable feedback overhead, i.e. 38 bits per sub-band and 342 bits for 9 sub-bands if K = 3. Moreover, the amplitude coefficient for each beam can be fed back in wideband, whereas the phase coefficients should be fed back in sub-band. Based on the above discussion, we can summarize the feedback overhead of beam selection, phase coefficients and amplitude coefficients as in Table 1.

Table 1. Maximum overhead calculation of proposed scheme

| Feedback granularity | Components | Overhead | Assume that K=3 and codebook-config = 1, (N1,N2,O1,O2) = (4,4,8,4) |
| --- | --- | --- | --- |
| Wideband | Beam selection |  | 11 bits |
|  | Amplitude coefficients |  | 16 bits |
| Sub-band | Phase coefficients | per sub-band | 24 bits per sub-band for two layers |

It can be seen from Table 1 that beam selection overhead is not larger than 11 bits which means that it can be accommodated by one report instance in PUCCH format 2. As for amplitude coefficients and phase coefficients, we can consider using PUCCH format 3 to feed back such large feedback overhead. Notice that there are at most 20 bits available in PUCCH format 3, so the amplitude coefficients can be fed back by PUCCH format 3 in one report instance. For the phase coefficients, we can divide the 24-bit overhead into two report instances if the two layers use different phase coefficients. For example, the phase coefficients for each layers can be reported separately in two instances. For further reduce the burden of PUCCH, periodicity of the amplitude feedback can be an integer multiple of periodicity of the phase feedback.

Proposal 4: PUCCH format 3 can be used to feed back the coefficients.

Proposal 5: Coefficients for each layer can be reported separately in different instances.

Proposal 6: Periodicity of the amplitude feedback can be an integer multiple of periodicity of the phase feedback.

- Conclusions

In this contribution, we discuss the feedback overhead of LC codebook and analyze throughly on overhead reduction. Based on the above considerations, we have made the following proposals.

Proposal 1: Support beam selection with pre-defined orthogonal beam selection pattern.

Proposal 2: Considering overhead reduction, beam selection information should be fed back in wideband.

Proposal 3: Both amplitude and phase coefficients should be contained in the LC CSI feedback.

  - Amplitude feedback should be allowed to be wide-band.

  - Coefficients should be allowed to be the same for two layers.

  - Unequal feedback bit allocation for coefficients of the selected beams should be supported.

Proposal 4: PUCCH format 3 can be used to feed back the coefficients.

Proposal 5: Coefficients for each layer can be reported separately in different instances.

Proposal 6: Periodicity of the amplitude feedback can be an integer multiple of periodicity of the phase feedback.

- Reference

[1] R1-166322, “Discussion on the design of linear combination codebook”, ZTE Corporation, ZTE Microelectronics.

[2] R1-166735, “LC based explicit CSI feedback and simulation results”, Samsung.

[3] R1-165100, “Explicit versus implicit feedback for advanced CSI reporting”, Ericsson.

[4] R1-1608682, “Discussion on LC codebook design”, ZTE Corporation, ZTE Microelectronics.

- Appendix

| System level simulation parameters |  |
| --- | --- |
| Scenarios | 3D-UMi 200m ISD and 3D-Uma 200m ISD |
| Antenna Configurations | 2x1 virtulization with 130° tilt |
| Antenna Spacing | (dV,dH)=( 0.8λ, 0.5λ) |
| Number of UE antenna | 2Rx cross-polarized antenna |
| Traffic model | FTP 1 with packet size 0.5M byte |
| OLLA | Target at 10% BLER |
| CSI-RS | Period is 5 ms and overhead is accounted. |
| Codebook | Extension of Rel-13 Class A codebook |
| HARQ | Max 4 retransmissions |
| Transmission rank | 1 or 2 |
| SU/MU pre-coding | BD |
| Scheduling | Proportional fair, up to 2 UEs, up to 2 layers |
| CQI/PMI reporting interval and frequency granularity | 5ms for CSI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2, Ideal channel covariance /PMI feedback |
| Delay for scheduling and AMC | 6ms |
| Receiver | MMSE-IRC. With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | FTP1 model with 0.5Mbyte |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |