3GPP TSG RAN WG1 Meeting #86bis                                                                  R1-1608739

Lisbon, Portugal, October 10th - 14th 2016

Source:	CATT

Title:	On SRS carrier switching with antenna switching

Agenda Item:	7.2.5.4

Document for:	Discussion / Decision

Introduction

In RAN1#86, following agreements were made on SRS carrier based switching for UEs with multiple antennas:

Agreement:

- Support SRS from each of the multiple antennas with carrier switching for periodic and aperiodic SRS

  - For 2 antenna cases, supported if UE is capable of UL 2x2 MIMO or 2 antenna switching

  - For 4 antenna cases, supported if UE is capable of UL 4x4 MIMO or 4 antenna switching (if introduced)

  - Details FFS

In this contribution we discuss UE antenna switching in conjunction with SRS carrier switching.

Discussion

In Rel-13, UE transmit antenna selection is supported for the UEs with single RF chain but two antennas which is configured by higher layers via parameter ue-TransmitAntennaSelection. 4 antenna switching was discussed in Rel-13 MIMO enhancement WI however there was no agreement to introduce it.

For UL MIMO capable UEs, it can transmit multiple SRSs after carrier switching, before discussing 4 Tx antenna switching in conjunction with carrier switching it should be studied under MIMO technique. It is not clear yet the performance gain of 4Tx antenna switching for DL transmission in MIMO. On the other hand if 4 Tx antenna switching is considered in conjunction with carrier switching it should be justified with performance benefit. Except for SRS carrier switching in special subframe, every instance of carrier switching will render following subframe unusable, that means the number of subframes wasted will be 4 folds if 4 Tx antenna switching is supported. Considering lack of study and unclear benefit it is proposed not to support 4 Tx antenna switching for UL transmission.

Conclusions

In this contribution, we discussed UE Tx antenna switching in conjunction with SRS carrier switching and the proposal is:

- 4 Tx antenna switching for UL transmission is not supported in Rel-14.

References

  - Chairman’s notes RAN1#86