3GPP TSG RAN WG1 Meeting #86bis                                              R1-1608758

Lisbon, Portugal 10th - 14th October 2016

Source:	CATT

Title:	Discussion on MA for URLLC

Agenda Item:	8.1.1.2

Document for: 	Discussion and Decision

Introduction

MA scheme has been extensively discussed and evaluated for mMTC. However, MA scheme for URLLC has not been discussed or evaluated.

In this contribution, we discuss the potential benefit of non-orthogonal MA scheme for URLLC and give our proposals on LLS and SLS assumptions for MA evaluation for URLLC.

Discussion

It was agreed to study grant-free transmission for URLLC as it can potentially reduce the latency. In addition, non-orthogonal multiple access, compared with orthogonal multiple access, can bring the following potential benefits as discussed in [1][2][3][4][5].

- Higher reliability due to lower collision probability and/or better detection performance in case of collision

- Flexible multiplexing with other service, e.g. low latency small packets can be superposed on top of other packets

- Higher spectrum efficiency

- Higher capacity

The evaluation metric and evaluation method for URLLC were discussed in [6] in RAN1#86 with following agreements.

Agreements:

- Evaluation metric and evaluation method for URLLC

  - User plane latency :

    - Definition: Follow the definition in TR38.913, target value is 0.5ms one way, without reliability requirement.

    - Evaluation method: Analytical; re-transmission is considered, but scheduling / queuing delay is not included in analytical evaluation

  - Reliability

    - Definition: Reliability is defined as the success probability R of transmitting X bits within L seconds, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality Q (e.g., coverage-edge).

      - Denoted as R(L, Q, SE), where SE is the required spectral efficiency and SE=X/L/B where B (in Hz) is the user bandwidth that is allocable to one device.

      - The latency bound L includes transmission latency, processing latency, retransmission latency and queuing/scheduling latency (including scheduling request and grant reception if any)

    - Evaluation method: Link level simulation as start point

  - URLLC capacity and URLLC / eMBB multiplexing capacity

    - Definition: Follow RAN1#85 agreements with further clarification, if needed

    - Evaluation method: System-level simulation can be considered

Several physical layer techniques can be applied to improve the performance of URLLC, e.g. channel coding, frame structure, MIMO, grant-free transmission, HARQ and MA [7]. It is better to evaluate each technique individually in order to understand the benefit of each technique. In this contribution, we focus on MA scheme for URLLC. Therefore, the assumptions of channel coding scheme, frame structure, antenna configuration and HARQ scheme for LLS shall be the same for different MA schemes.

It was agreed to use LLS as start point to evaluate URLLC reliability where scheduling/queuing delay is not assumed in LLS. The evaluation assumptions have been agreed in [6] which can be reused as starting point for MA LLS for URLLC.

As discussed above, the purpose of this LLS is to compare different MA schemes for URLLC. It is import to assume same channel coding scheme, HARQ and frame structure while it is less important which scheme is assumed especially considering that all these schemes are still under discussion. In general, it is preferable that LTE schemes can be assumed in the LLS.

Regarding channel coding, it is proposed to assume LTE Turbo coding. The metric is BLER performance versus SNR. In order to accelerate the simulation, it is proposed to compare the required SNR to achieve BLER=10-4 without HARQ according to the guidelines agreed for URLLC channel coding evaluation in RAN1#84bis.

It is proposed that LTE numerology is assumed considering that the latency is not considered in the LLS.

In addition, in order to compare different MA schemes, the total allocated bandwidth should be defined. Furthermore, it is proposed to assume grant-free transmission for non-orthogonal MA while for orthogonal MA, both grant-based and grant-free transmissions can be assumed. The proposed values or assumptions for MA LLS evaluation for URLLC are listed in Table 1.

Table 1: LLS assumptions for MA evaluation for URLLC

| Attributes | Values or assumptions | Proposed values or assumptions for MA evaluation |
| --- | --- | --- |
| Carrier Frequency | 700MHz and 4 GHz (FDD and TDD) | 4GHz |
| Modulation and coding rate | QPSK, 16QAM, 64QAM 1/12, 1/6, 1/3 Other MCS not precluded Comparison should be made for the same spectrum efficiency | QPSK LTE Turbo coding |
| User bandwidth | Companies report | Total allocated bandwidth: 8 RB Number of UEs: 4, 8 For grant-based OFDMA: UEs share the total allocated bandwidth equally in frequency domain For grant-free OFDMA: For non-orthogonal MA: UEs share the total allocated bandwidth according to specific MA scheme |
| PHY Packet size | 32 byte, 50 byte, 200 byte Other values are not precluded. | 32 byte with CRC included |
| Latency bound (NOTE1) | 1ms Other values are not precluded Companies report delay assumptions according to Table 1 in R1-166485. Scheduling / queuing delay is not assumed in LLS. | Single transmission without HARQ Target BLER:10^-4 |
| SINR range | -5dB to 20dB Larger range is not precluded | -5dB to 20dB Larger range is not precluded |
| Sub-carrier spacing | Companies report | 15 kHz |
| TTI length | Companies report | 1ms |
| OFDM symbols per TTI | Companies report | 14 |
| Channel model | TDL/CDL in TR38.900; user speed = 3km/h, 15km/h (other user speed is not precluded) | TDL-C in TR38.900, user speed=3km/h |
| BS antenna configuration | 2/4/8 Tx/Rx ports as start point Other values (i.e., up to 256) are not precluded | 4 Rx ports |
| UE antenna elements | 2/4 Tx/Rx ports as start point Other values (i.e., up to 8) are not precluded | 1 Tx port |
| ACK Feedback assumption | Ideal as start point NOTE: It is also possible that no ACK feedback is needed, | No HARQ |
| Channel estimation | Ideal as start point; Realistic is not precluded when RS design is ready | Ideal |
| CQI feedback assumption | Companies report the feedback scheme if any | No CQI feedback |

Proposal 1: Evaluate the performance of different MA schemes for UL URLLC in terms of reliability via LLS according to the assumptions listed in Table 1.

It was agreed that SLS can be considered for URLLC capacity evaluation where URLLC system capacity C(L, R) is the maximum offered cell load under which Y% of UEs in a cell operate with target link reliability R under L latency bound. In addition, it was agreed that indoor hotspot and urban macro scenarios are used as a starting point for initial URLLC evaluations. It is proposed to assume urban macro scenario as a starting point with the following assumptions listed in Table 2 based on agreed assumptions for urban macro [8]. The metric is the supported packet arrival rate under which Y% of UEs in a cell operate with target link reliability R under L latency bound.

Table 2: SLS assumptions for MA evaluation for URLLC

| Attributes | Values or assumptions |
| --- | --- |
| Layout | Single layer – Urban Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | Companies report simulation bandwidth used in evaluation |
| Channel model | 3D UMa |
| Tx power | UE: Max 23dBm |
| BS antenna configuration | Rx: 4 ports |
| BS antenna pattern | Follow the modeling of TR36.873 |
| BS antenna height | 25m |
| BS antenna tilt | Companies report tilt |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |
| BS receiver noise figure | 5 dB |
| UE antenna elements | 1Tx |
| UE antenna height | Follow TR36.873 |
| UE antenna gain | Follow the modeling of TR36.873 |
| Traffic model | Non-full buffer small packet with Poisson arrival |
| UE distribution | 20% of users are outdoors (3km/h) |
|  | 80% of users are indoors (3km/h) |
|  | Users dropped uniformly in entire cell |
| BS receiver | MMSE-IRC as baseline, Advanced receiver is not precluded |
| Channel coding | LTE Turbo |
| subcarrier spacing | 15kHz |
| OFDM symbols per TTI | Companies report |
| RS overhead | Companies report |
| UL power control | Companies report power control scheme |
| Channel estimation | Realistic |

Proposal 2: Evaluate the performance of different MA schemes for UL URLLC in terms of capacity via according to the assumptions listed in Table 2.

Conclusion

In this contribution, we discuss the potential benefit of non-orthogonal MA scheme for URLLC and give our proposals on LLS and SLS assumptions with following proposals.

Proposal 1: Evaluate the performance of different MA schemes for UL URLLC in terms of reliability via LLS according to the assumptions listed in Table 1.

Proposal 2: Evaluate the performance of different MA schemes for UL URLLC in terms of capacity via according to the assumptions listed in Table 2.

- References

- R1-166466, “Usage scenarios of non-orthogonal multiple access”, CATT, RAN1#86

- R1-162153, “Overview of non-orthogonal mutliple access for 5G”, Huawei, RAN1#84bis

- R1-163111, “Initial views and evaluation results on non-orthogonal multiple access for mMTC uplink”, NTT DOCOMO, RAN1#84bis

- R1-162922, “Discussion on Multiple Access for New Radio systems”, InterDigital, RAN1#84bis

- R1-163125, “Considerations on multiple access sheme for new radio interface”, CATR, RAN1#84bis

- R1-168371, “WF on URLLC evaluation parameter and LLS method”, Huawei, RAN1#86

- R1-166484, “Discussions on KPI and evaluation configuration for URLLC”, CATT, RAN1#86

- R1-168391, “Offline discussion updated summary of evaluation assumptions for NR”, NTT DOCOMO, RAN1#86