3GPP TSG RAN WG1 Meeting #86		R1-1608770

Lisbon, Portugal 10th - 14th October 2016

Source:	CATT

Title:	Flexibility evaluation of channel coding schemes for NR

Agenda Item:	8.1.3.1

Document for:	Discussion and Decision

- Introduction

For NR, it is desired to support the bit-level granularity of the codeword size and code operating rate [1]. Taking the design of LDPC code in eMBB scenario as an example, the proposed H matrices is optimized for particular code lengths and particular code rates.  The actual transmission code length and code rate might be out of the range. Because the channel coding rate are determined by the transmission block size and available resource elements (Res), the actual coding rate used in transmission could not be restricted and optimized for specified ranges.  The flexibility of the channel coding scheme is an important aspect along with the evaluation of the coding performance.

In this paper, we discuss the flexibility of channel coding schemes for NR.   The LDPC codes in [2] with excellent performance over turbo codes in high code rate are the main target for evaluation in this paper.

- Description of the evaluate code

The evaluations are mainly performed based on the simulation assumptions in [3].

  - Symbol description

The following notations are used in this paper.

kb,max: The maximum number of base information bits in the nested family.

kb: The number of base information bits in the nested family.

cb: The number of base parity bits used

sf: The number of shortening bits at final code level

sb: The number of shortening bits at base graph level

pb: The number of puncturing bits in the end of the parity bit sequence

z: The Lifting size of base matrix

K: The number of information bits                            N: The number of transmission bits

R: Code rate

Fd：Doppler shift                                                      Ts：symbol period

The normalized Doppler shift= Fd* Ts

  - Simulation description

The evaluations are performed based on the agreed simulation assumptions in [3]. Additional parameters are added in the simulation as follows,

- LDPC codes:

  - Radio channel: AWGN channel and Rayleigh Fading channel,

    - Fading channel: Fd=200, Ts=0.00005.

  - Modulation: QPSK and 64QAM,

  - Code rate={0.45, 0.5, 0.7}

  - Information block length (bits) ={960, 976, 1000, 1064, 1080, 1352, 1400, 1440, 1472, 1512, 1560, 1600,1672,1728,1920}

  - Scaled MSA decoding algorithm, max iteration number is 30.

- Turbo codes:

  - Radio Channel: AWGN channel and Rayleigh Fading channel,

    - Fading channel:  Fd=200,Ts=0.00005.

  - Modulation: QPSK and 64QAM

  - Code rate={0.45, 0.5, 0.7},

  - Information block length (bits) ={960, 976, 1056, 1344, 1440, 1472, 1568, 1664, 1920}

  - Scaled max-log-map decoding algorithm: iteration number is 8.

- Other simulation parameters of LDPC codes are listed in Table 1, 3-2, 4-1, 4-2.

Information block size flexibility

  - Lifting

LDPC code ensemble with a predefined structure can be constructed by means of protograph. By applying a graph lifting operation, Tanner graphs of various sizes can be constructed that preserve the rate and the degree distribution of the protograph. Based on the agreement in RAN1#85 [4], the multiple code information block sizes can be realized by adjusting the parameters nb and z.

Table 1: Parameter of LDPC with Different Lifting Size

| R | N (bit) | K (bit) | Pb (bit) | Sf(bit) | Sb(bit) | z(bit) | Kmax | Kb | Cb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.5 | 1920 | 960 | 0 | 0 | 240 | 40 | 30 | 24 | 26 |
| 0.5 | 2880 | 1440 | 0 | 0 | 160 | 80 | 20 | 18 | 20 |
| 0.5 | 3840 | 1920 | 0 | 0 | 480 | 80 | 30 | 24 | 26 |

a (QPSK)

b (64 QAM)

Figure 1: The performance comparison of Turbo code and LDPC with different lifting size

(QPSK/64QAM, AWGN, R=0.5, scaled MSA, max Iteration number=30, K=960 bits~1920 bits).

In this paper, we get different code information block sizes by adjusting the parameters z (where z is given by 2j×4,5,6,7 for j∈[1,2,3,4,5,6,7]). For the same information block size, the maximum lift size z should be selected in order to meet the throughput and latency requirement at peak rate.  The selection of lifting size is associated with the structure of base matrix and even constrained by its construction method.

According to the simulation, the result shows that performance of LDPC is affected by the lifting size. At the BLER of 10-2, turbo codes outperform LDPC by about 0.3-0.4dB with 64QAM modulation.   The turbo code gets more gain in performance at BERL of 10-2 from that of LDBC with QPSK modulation. Inappropriate selection of lifting size may lead to performance degradation of LDPC.

  - Puncturing and Shortening

The set of lift values are discrete.  The single-bit information block length granularity should not be supported by lifting only.  Thus, the support of puncturing and shortening should also be considered.  In this paper, we divide the puncturing bits into two categories. They can be interpreted as puncturing at systematic columns and parity columns. As described in [2], the first two variable nodes corresponding to the systematic columns are punctured.   The bits level puncturing is performed in the last parity column. Similar to puncturing operation, two kinds of shortening are proposed, such as base graph and bits level.

Table 2: Parameter of LDPC with different puncturing+shortening bits

| Rate | N (bit) | K (bit) | Pb (bit) | Sf(bit) | Sb(bit) | z(bit) | Kmax | Kb | Cb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.5 | 1952 | 976 | 24 | 24 | 200 | 40 | 30 | 25 | 27 |
| 0.5 | 2704 | 1352 | 48 | 48 | 280 | 56 | 30 | 25 | 27 |
| 0.5 | 3120 | 1560 | 40 | 40 | 320 | 64 | 30 | 25 | 27 |
| 0.5 | 2128 | 1064 | 16 | 16 | 120 | 40 | 30 | 27 | 29 |
| 0.5 | 2944 | 1472 | 40 | 40 | 168 | 56 | 30 | 27 | 29 |
| 0.5 | 3344 | 1672 | 56 | 56 | 192 | 64 | 30 | 27 | 29 |

Figure 2: The performance comparison of Turbo and LDPC

(QPSK, AWGN, R=0.5, K=976 bits~1568 bits).

Figure 3: The performance comparison of Turbo and LDPC

(QPSK, AWGN, R=0.5, K=1056 bits~1672 bits).

The simulation results show that puncturing and shortening may lead to performance degradation of LDPC and Turbo codes.  At the BLER of 10-2, turbo codes outperform LDPC by about 0.1-0.2dB. The performance of LDPC code with the information length k=1560bits is better than k=1672bits. It implies that the performance of inflexible LDPC code is not robust to the effect of puncturing and shortening.

Code rate flexibility

Contrary to LTE Turbo codes, which puncture a low rate code in order to provide higher operating code rates through rate matching.   LDPC codes can be specified with a base graph per operating rate and obtain codes for the other rate by puncturing and shortening.  The performance loss of Turbo codes is larger than LDPC in the case of high code rate.

- Performance evaluations with different rate in AWGN channel

Table 3: Parameter of LDPC with different rate in AWGN channel

| R | N (bit) | K (bit) | Pb (bit) | Sf(bit) | Sb(bit) | z(bit) | Kmax | Kb | Cb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.45 | 4620 | 2080 | 20 | 0 | 320 | 80 | 30 | 26 | 34 |
| 0.45 | 4974 | 2240 | 50 | 64 | 576 | 96 | 30 | 24 | 31 |
| 0.45 | 5334 | 2400 | 42 | 0 | 480 | 96 | 30 | 25 | 33 |
| 0.7 | 2976 | 2080 | 64 | 0 | 320 | 80 | 30 | 26 | 14 |
| 0.7 | 3198 | 2240 | 2 | 64 | 576 | 96 | 30 | 24 | 12 |
| 0.7 | 3426 | 2400 | 30 | 0 | 480 | 96 | 30 | 25 | 13 |

Figure 4: The performance of LDPC with different rates

(QPSK, AWGN, R=0.45, K=2080 bits~2432 bits).

Figure 5: The performance of LDPC with different rates

(QPSK, AWGN, R=0.7, K=2080 bits~2400 bits).

- Performance evaluations in different channel

Table 4: Parameter of LDPC with different rate in AWGN/fading channel

| R | N (bit) | K (bit) | Pb (bit) | Sf(bit) | Sb(bit) | z(bit) | Kmax | Kb | Cb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.45 | 2169 | 976 | 7 | 24 | 200 | 40 | 30 | 25 | 32 |
| 0.45 | 3004 | 1352 | 28 | 48 | 280 | 56 | 30 | 25 | 32 |
| 0.45 | 3467 | 1560 | 13 | 40 | 320 | 64 | 30 | 25 | 32 |
| 0.45 | 2364 | 1064 | 20 | 16 | 120 | 40 | 30 | 27 | 35 |
| 0.45 | 3271 | 1472 | 49 | 40 | 168 | 56 | 30 | 27 | 35 |
| 0.45 | 3717 | 1672 | 3 | 56 | 192 | 64 | 30 | 27 | 34 |

a. AWGN channel                                                               b. fading channel

Figure 6: The performance comparison of Turbo and LDPC in AWGN/fading channel

(QPSK, fading, R=0.45, K=976 bits~1568 bits).

a. AWGN channel                                                               b. fading channel

Figure 7: The performance comparison of Turbo and LDPC in AWGN/fading channel

(QPSK, fading, R=0.45, K=1056 bits~1672 bits).

The simulation results show that LDPC codes have better performance than Turbo codes for high code rate. At the BLER of 10-2 and the code rate of 0.45, turbo codes outperform LDPC by about 0.3-0.4dB in AWGN channel and more gain than in fading channel.

- Conclusions

In this contribution, we provide the flexibility evaluation of channel coding schemes for NR.

- Observation 1: The performance of binary LDPC is affected by the lifting size. Inappropriate selection of lifting size, may lead to performance degradation of binary LDPC.

- Observation 2: The puncturing/shortening are not robustness for binary LDPC.

- Observation 3: The performance loss of turbo codes is larger than binary LDPC in the case of high code rate. In the case of low code rate, turbo codes get more gain than in both AWGN and fading channels.

- Observation 4: The code rate flexibility is an important aspect of evaluating the coding scheme in the practical implementation.  The performance of LDPC is degraded by lifting, shortening and puncturing.

- References

[1] Chairman’s notes, 3GPP TSG RAN WG1 #86bis meeting

[2] R1-166388, “LDPC rate compatible design” , Qualcomm Aug.2016

- [3] Chairman’s notes, 3GPP TSG RAN WG1 #84bis meeting.

- [4] Chairman’s notes, 3GPP TSG RAN WG1 #85bis meeting.

- [5] Xijin Mu, Conghui Shen and Baoming Bai, “A Combined Algebraic and Graph-Based Method for Constructing Structured RC-LDPC Codes,” IEEE Communication Letters, 2016.