3GPP TSG RAN WG1 Meeting #86bits		R1-1608772

Lisbon, Portugal 10th- 14thOctober 2016

Source:	CATT

Title:	LDPC codes design for eMBB

Agenda Item:	8.1.3.1

Document for:	Discussion and Decision

- Introduction

In RAN #71[1],Polar codes, Turbo codes and LDPC codes were proposed as the candidates for the NR eMBB scenario.

In RAN1#84b [2], the following simulation assumptions were agreed for the eMBBscenario:

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 64 QAM |  |  |  |
| Coding Scheme | Turbo | B-LDPC | Polar |  |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |  |  |
| Decoding algorithm** | Max-log-MAP | min-sum |  | List-X |
| Info. block length*** (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K) |  |  |  |

In RAN1#85 [3], an agreement about LDPC codes has been reached:

- For the purpose of study and comparisons, quasi-cyclic like LDPC codes  are defined as follows:

  - The Parity check matrix of Quasi-cyclic like LDPC Codes is defined at least by a matrix H of size (mb×z)×(nb×z), which consists of sub-block matrices of size z×z,  where each sub-block matrix is composed by circularly shifted matrices or zero matrices. Wherein, mb, nb and z are integers larger than 1.

    - The values of mb, nb and z  are FFS.

- Companies providing evaluations or proposals for LDPC codes are encouraged to show how:

  - Multiple code rates and multiple code sizes would be supported,

  - Suitable granularity of  information block size and code rate would be supported,

  - How to support HARQ with/without IR.

In RAN1#86, the performance evaluation results of LDPC codes and the complexity of the LDPC codes had been shown in [4].

In this contribution, the construction details of LDPC codes for eMBB scenario and the performance evaluations of LDPC codes are shown.

- Construction of LDPC codes

The following notations are used for the description of the codes

- K：Information length (bits)；

- R：code rate；

  - Code parameters of constructed LDPC codes

The basic code family flexibly supports different information lengths K and code rates R. More code rates and information lengths can also be supported.  The supported information lengths of basic code family include 368bits≤K≤16304 bits with the interval of 16 bits. Extended information lengths with 8-bit interval can also be supported through puncturing operation.

The supportedcode rates of basic code are R=16/n, where 18≤n≤48. Other code rates in the range 1/3≤n≤8/9 can also be supported.

  - Construction process of basic code family

The basic code family with code rate R and information length K. Parameters R and K are in the range of supported code parameters of basic code family. The following is the steps of constructing the parity check matrix of target code.

Step 1:generate the parameters CPM= K/16;

Step 2: generate circulate coefficient matrix CircMat with 32 rows and 17 columns.;

Step 3: generate base matrixBaseMat based on the circulate coefficient matrix CircMat and the masking matrixMaskMat;

MaskMat is a matrix with 32 rows and 17 columns.   The entries are 1 or 0. BaseMat is a matrix with 32 rows and 17 columns. The (i,j)-th entry in BaseMat(i,j) equals to CircMat(i,j) if MaskMat(i,j)=1, and BaseMat(i,j) equals to -1 if MaskMat(i,j)=0.

Step 4: generate matrix H1 based on base matrix BaseMat;

The size of matrix H1 is 32×CPM rows and 17×CPM columns. It’s constructed by replacing the (i,j)-th entry in BaseMat with a circulant permutation matrix whose circulate coefficient is BaseMat (i,j), if BaseMat (i,j)≠-1, or with a zero-matrix if BaseMat (i,j)= -1.

Step 5: generate matrix H2;

H2 is a matrix with 32×CPM rows and 31×CPM columns.

,

where

,

Ha1 is zero-matrix with CPM rows and CPM columns.

Ha2 is consist of a identity matrix with CPM rows and CPM columns adding a circulant permutation matrix whose circulate coefficient is CPM-1, and the (1,CPM)-th entry in Ha2 is set to zero.

Hb is a zero-matrix with 31×CPM rows and CPM columns.

Hc is a zero-matrix with 2×CPM rows and  31×CPM columns.

Hd is an identity matrix with 31×CPM rows and 31×CPM columns.

Step 6: generate main matrix ;

The main matrix Hmain=[H1,H2].

Step 7: generate assist matrix ;

Given an assist base matrix BaseMatAssi, whose size is 2 rows and 18 columns.

Replace the (i,j)-th entry in BaseMatAssi with a circulant permutation matrix whose circulate coefficient is BaseMatAssi (i,j) if BaseMatAssi (i,j)≠-1, or with a  zero-matrix if BaseMatAssi (i,j)= -1. The size of resulting matrix  is 2×CPM rows and 18×CPM columns.

Step 8: generate the parity check matrix H of targetwith code rate R and information length K;

is the first K/R columns and (K/R-K) rows of

If R≠8/9, H=.

If R=8/9, H = +

Based on above steps, LDPC codes with various information lengths K and code rates R can be constructed.

Observation 1: LDPC codes are capable of supporting various code rates and information lengths.

- Performanceevaluations

The evaluations are mainly based on the agreed simulation assumptions in [2].

  - K=400

Simulation Parameters for Figure 1 and Figure 2:

| Code Type | K | R | Decoder | Max Ite | Channel | Modulation |
| --- | --- | --- | --- | --- | --- | --- |
| LDPC | 400 | 1/3, 2/5, 1/2, 4/7,2/3, 3/4, 5/6, 8/9 | BP/Flooding | 50 | AWGN | QPSK/64QAM |
| Turbo | 400 | 1/3, 2/5, 1/2, 4/7,2/3, 3/4, 5/6, 8/9 | scaled max-log-map | 8 | AWGN | QPSK/64QAM |

  - k=1000

Simulation Parameters for Figure 3 and Figure 4:

| Code Type | K | R | Decoder | Max Ite | Channel | Modulation |
| --- | --- | --- | --- | --- | --- | --- |
| LDPC | 992 | 1/3, 2/5, 1/2, 4/7,2/3, 3/4, 5/6, 8/9 | BP/Flooding | 50 | AWGN | QPSK/64QAM |
| Turbo | 1008 | 1/3, 2/5, 1/2, 4/7,2/3, 3/4, 5/6, 8/9 | scaled max-log-map | 8 | AWGN | QPSK/64QAM |

Observation 2: LDPC codes can achieve better performance compared with Turbo codes for high rate.

Observation 3: Turbo codes can achieve better performance compared with LDPC codes for low rate.

- Conclusions

In this contribution, we provide the performance evaluation results of binary LDPC codes for NR.

Observation 1: LDPC codesare capable of supportingvarious code rates and information lengths.

Observation 2: LDPC codes can achieve better performance compared with Turbo codes for high rate.

Observation 3: Turbo codes can achieve better performance compared with LDPC codes for low rate.

- References

RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO, March 2016.

Chairman’s notes, 3GPP TSG RAN WG1 #84bis meeting.

Chairman’s notes, 3GPP TSG RAN WG1 #85bis meeting.

R1-166475, “Evaluation of binary LDPC codes for NR”, CATT, 3GPP TSG RAN WG1 Meeting #86	Gothenburg, Sweden, 22nd - 26th August 2016.