3GPP TSG RAN WG1 Meeting #86	bis                                                         R1-1608800

Lisbon, Portugal 10th - 14th October 2016

Agenda Item:	           8.1.1.1

Source:	           Fujitsu

Title:	           Discussion on real domain OFDM for mMTC

Document for:	         Discussion/Decision

# Introduction

In [1], the newly approved SI about NR (new radio access technology) has some agreements about waveform as the follows:

- At least up to 40 GHz for eMBB and URLLC services, NR supports CP-OFDM based waveform with Y greater than that of LTE (assuming Y=90% for LTE) for DL and UL, possibly with additional low PAPR/CM technique(s) (e.g., DFT-S-OFDM, etc.)

  - Y (%) = transmission bandwidth configuration / channel bandwidth * 100%

  - RAN1 specification will support transmission bandwidth configuration corresponding to Y up to approximately100%

  - Some evaluations in RAN1 show that Y for a NR carrier can be up to 98% of the evaluated channel bandwidths for both DL and UL without complexity and latency constraints

  - Note: additional pre-processing techniques on top of CP-OFDM are not precluded, e.g., OTFS

- Additional waveforms may be supported by NR for e.g. other services (e.g. mMTC)

- …

In addition, the following mMTC related agreement about MA are achieved [1]:

- NR should target to support UL “autonomous/grant-free/contention based” at least for mMTC

In this contribution, we mainly focus on the new waveform design for the mMTC usage scenario. For providing mMTC services, connection density is an important and challenging KPI. Multiple technologies should be used together to achieve it. In the area of multiple access (MA) technology, many non-orthogonal MA schemes have been proposed [3]-[6]. However, in the area of waveform, there is no such candidate optimized for connection density yet. On the other hand, battery power and coverage are also the main optimization goals of mMTC [7]. This makes the waveforms with low PAPR very attractive since low PAPR leads to long battery life, high PA efficiency [7][8] and possible cost reduction.

# Waveform design for mMTC

In this section, we will describe our new waveform design for mMTC, including the motivation and the proposed waveform, i.e., real-domain OFDM (r-OFDM) and discrete cosine transform precoded r-OFDM (DCT-r-OFDM). According to our analysis, the subcarrier spacing of OFDM can be halved to keep orthogonality only in the real domain. By doing so, the resource element (RE) density can be doubled. This will benefit the mMTC service which has an optimization goal of connection density.

## Background and motivation

Currently, OFDM-based waveforms have been considered in NR for their high spectral efficiency with reasonable complexity. This property comes from the minimized subcarrier spacing together with inter-subcarrier orthogonality in the complex domain.  The expression for the k-th subcarrier with the rectangular window is:

pkt=1T∙ej2πkFt0<t<T0othersk=0,1,2,…,K-1

where T is the symbol duration time, F is the subcarrier spacing, k is subcarrier index and K is the total number of all subcarriers. Therefore, the frequency domain correlation of any two subcarriers k1and k2 can be written as:

ρ∆f=1T0Tej2πk1-k2Ftdt=sinπT∆fπT∆fejπT∆f∆k=k2-k1,∆f=∆k∙F

The real and imaginary parts of ρ∆f are shown in Figure 1 respectively.

Figure 1. Subcarriers’ correlation in frequency domain

It can be seen that both real and imaginary parts of ρ∆f are zero at multiples of 1T.Thus 1T is the minimized subcarrier spacing to achieve orthogonality in complex domain for OFDM. On the other hand, zero-crossing points of real part are located at multiples of 12T, which means the minimized subcarrier spacing to achieve orthogonality in the real domain can be halved compared to that in complex domain.

Observation 1: OFDM’s subcarrier spacing can be halved when only keeping orthogonality in the real domain.

This halved subcarrier spacing waveform is called real domain OFDM (r-OFDM) in this contribution.

## Description of r-OFDM

The signal of K-subcarrier conventional OFDM can be expressed as:

sOFDMt=1Tk=0K-1ak∙ej2πktT0<t<T

where ak could be a complex number modulated on k-th subcarrier .

Then r-OFDM is obtained by halving the subcarrier spacing and transmitting real-valued data. Given the same total bandwidth and symbol duration time, r-OFDM can accommodate 2K subcarriers which can be formulated as:

sr-OFDMt=1Tk=02K-1bk∙ej2πkt2T0<t<T

where bk is k-th subcarrier data and real-valued.  The received signal in AWGN is:

yr-OFDMt=sr-OFDMt+nt

where nt is additive white Gaussian noise. The k-th subcarrier data can be detected by:

bk=real1T0Tyr-OFDMt∙e-j2πkt2Tdt=bk+nk

where nk is the noise part after k-th de-filter.

In this way, the resource element (RE) density of the proposal is doubled over that of the OFDM as shown in Figure 2.

Figure 2 Resource grid of OFDM (left) and r-OFDM (right)

## Description of DCT-r-OFDM

Similarly to DFT-s-OFDM in LTE uplink, in order to reduce PAPR, a discrete cosine transform (DCT) matrix based pre-coding/spreading is performed before r-OFDM modulation at the transmitter, as shown in Figure 3. DCT was chosen instead of DFT because it is a real domain transform and has the same subcarrier spacing as that of r-OFDM. And DCT is a well-known method to reduce PAPR of multicarrier systems [9]-[12] due to its energy compaction property [9] [10]. It is also found that, in terms of PAPR reduction, DCT is better than DFT [9].

Figure 3 Transmitter of DCT-r-OFDM

## Performance evaluation of DCT-r-OFDM vs. DFT-s-OFDM

For real domain modulation schemes such as BPSK, DCT-r-OFDM can have doubled spectral efficiency over that of DFT-s-OFDM due to its halved subcarrier spacing. We did some simulations to verify the performance of DCT-r-OFDM with BPSK and take DFT-s-OFDM as the baseline for comparison in terms of BLER, bandwidth and PAPR. Simulation parameters are listed in Table 1.

Table 1 simulation parameters

| Parameters | Values or assumptions |
| --- | --- |
| Carrier frequency | 4GHz |
| System Bandwidth | 1.4 MHz |
| Subcarrier spacing | 15KHz for DFT-s-OFDM 7.5KHz for DCT-s-OFDM |
| DFT size/Total subcarrier number | 128 |
| Data subcarrier | 72 |
| Modulation order | BPSK |
| TB size | 300bits |
| Antenna configuration | 1Tx 1Rx |
| Channel Model | TDL-A 3km/h |

Figure 4. BLER performance and PSD of BPSK + DFT-s-OFDM and BPSK+DCT-r-OFDM

It is evident in Figure 4 that, compared with conventional DFT-s-OFDM, DCT-r-OFDM has almost identical performance but halved bandwidth. This can be beneficial for some special use cases such as mMTC services, which do not require high data rate. When low order modulation such as BPSK is applicable, each device’s data transmission bandwidth can be halved without loss of performance. As a result, DCT-r-OFDM has a potential to support twice the number of connected devices than DFT-s-OFDM in the same total bandwidth. Potentially, DCT-r-OFDM could enable an enhancement on connection density [13] for mMTC. On the other hand, BPSK is a good candidate for the uplink control channel. For example, in NB-IoT, BPSK is used for NPUSCH format 2 to carry uplink control information [14]. If using DCT-r-OFDM with BPSK, uplink control signaling capacity could also be doubled.

Observation 2: When real domain modulation is applicable, DCT-r-OFDM has the potential to support double the number of concurrently connected devices over DFT-s-OFDM.

As pointed out by [7] [8], PAPR is one of the main optimization goals of mMTC. Low PAPR can provide wider coverage, save power consumption and allow small-size devices [10]. From energy compaction point of view, DCT is a better transform than DFT [9]. Thus low PAPR can be achieved by DCT-r-OFDM. From Figure 5, it can be seen that the PAPR of DCT-r-OFDM is about 1.8 dB lower than that of DFT-s-OFDM when CDF is 90%.

Figure 5. PAPR of BPSK + DFT-s-OFDM and BPSK+DCT-r-OFDM

Observation 3: Compared to DFT-s-OFDM, DCT-r-OFDM has a lower PAPR.

## Further Discussion of r-OFDM and DCT-r-OFDM

In addition to data channels, other UL channels, such as control channels or random access (RA) channels, can also be benefitted by this frequency-efficient waveform.

For the control channel, it is originally designed for low order modulations (BPSK, QPSK). Therefore, its capacity can also be doubled by use of the proposed waveform.

According to the agreements in RAN1#86, contention-based RA could be supported in mMTC services. If so, the random access channel should be used and its capacity should be optimized as well.

For the random access channel, it carries a sequence which is used as the device’s signature. The number of orthogonal sequences is equal to the length of the sequence in principle. Consequently, its capacity is limited by the maximum sequence length. By use of the r-OFDM, the length of the RA sequence can be doubled. As a result, the RA channel capacity can be doubled and the contention probability can be significantly reduced.

In summary, by use of r-OFDM and DCT-r-OFDM, all the uplink channels’ capacity could be doubled in potential. This makes our proposed waveform more attractive for the uplink-dominant services, such as mMTC.

Observation 4: Using r-OFDM and DCT-r-OFDM, all the uplink channels’ capacity could be doubled in potential.

# Conclusion

Based on the analyses and comparisons above, we have the following observations:

Observation 1: OFDM’s subcarrier spacing can be halved when only keeping orthogonality in the real domain.

Observation 2: When real domain modulation is applicable, DCT-r-OFDM has the potential to support double the number of concurrently connected devices over DFT-s-OFDM.

Observation 3: Compared to DFT-s-OFDM, DCT-r-OFDM has a lower PAPR.

Observation 4: Using r-OFDM and DCT-r-OFDM, all the uplink channels’ capacity could be doubled in potential.

This leads to the following conclusion:

Conclusion: DCT-r-OFDM waveform has the following advantages:

  - Potentially supporting double the number of devices and control signaling capacity;

  - Lower PAPR;

We propose to study and evaluate the OFDM-based real domain orthogonal waveforms due to its advantages as above.

Proposal: Real domain orthogonal waveform for mMTC should be further investigated.

# References

- 3GPP TSG RAN WG1 Meeting #86 Chairman’s notes, Gothenburg, Sweden 22nd - 26th August 2016

- 3GPP TSG RAN WG1 Meeting #84bis Chairman’s notes, Busan, Korea, April 11-15th, 2016.

- R1-164037, “LLS results for uplink multiple access”, Huawei, HiSilicon, RAN1#85.

- R1-164247, “Performance of LLS of PDMA”, CATT, RAN1#85.

- R1-163992, “Non-orthogonal multiple access candidate for NR”, Samsung, RAN1#85.

- R1-164329,“Initial LLS results for UL non-orthogonal multiple access”,Fujitsu, RAN1#85

- R1-164684,“Single Carrier Waveform Evaluation”， Qualcomm，RAN1#85

- R1-165015,“Waveform proposal for carrier frequencies beyond 40 GHz”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#85

- Prashant Kumar, Preetam Kumar; “Performance Evaluation of DFT-Spread OFDM and DCT-Spread OFDM for Underwater Acoustic Communication”, 2012 IEEE Vehicular Technology Conference (VTC Fall), pp1-5.

- Shanlin Sun; Chunping Hou; Lei Yan; Jinlin Fu , “A Novel Method Based DCT to Reduce PAPR of OFDM Systems ”, 2008 4th International Conference on Wireless Communications, Networking and Mobile Computing

- Imran Baig; Varun Jeoti ; “DCT precoded SLM technique for PAPR reduction in OFDM systems ”, 2010 International Conference on Intelligent and Advanced Systems (ICIAS)

- Imran Baig; Varun Jeoti; “A new DCT matrix precoding based RI-OFDMA uplink system for PAPR reduction”, 2012 4th International Conference on Intelligent and Advanced Systems (ICIAS)

- 3GPP TR 38.913 v0.3.0,’study on scenarios and requirements for next generation access technologies’, March, 2016

- 3GPP TR 36.211 v13.2.0.’physical channels and modulation’, June, 2016