3GPP TSG RAN WG1 Meeting #86bis	R1-1608855

Lisbon, Portugal, Oct 10-14, 2016

Agenda Item:	8.1.1.2

Source:	Huawei, HiSilicon

Title:	Advanced multi-user detectors for grant-free transmissions

Document for:	Discussion and decision

Introduction

In RAN1#86, the following agreements on the study of UL grant-free transmissions were achieved [1]:

- Continue study at least the following:

  - Handling of  potential collisions of MA signatures

  - Retransmission/repetition and potential combining, e.g. HARQ

  - Potential link adaptation, e.g. MCS/signature re-assigning

  - Relationship between grant-free and grant-based transmissions and associated UE behavior

  - Advanced receiver capabilities including complexity analysis

Advanced receiver capabilities including complexity analysis is necessary for UL grant-free transmission. In this contribution, we discuss different types of advanced multi-user detection (MUD) receivers and their application in UL grant-free transmission.

Receiver Procedure for UL Grant-free

For UL grant-free transmission, eNB does not have prior information about which UEs will transmit. In our companion contribution [2], it presents a receiver procedure for UE activity detection as shown in Figure 1, in which eNB will first detect the UE activity, and then do the data detection.

Figure 1. Example of receiver procedure for UL grant-free

UE Activity Detection

As discussed in [2], RS can be used by eNB to decide which UEs transmit on the physical resources of UL grant-free. As RS will only be transmitted by active UEs, it is the indicator of UE activity. Due to the interference and noise, some active UEs may be regarded to be inactive, and vice versa. We referred to these UEs as miss-detected UEs and false-alarmed UEs, respectively. As a general detection problem, there will be a tradeoff between the miss detection and false alarm, as shown in Figure 2. On the one hand, the number of miss-detected UEs should be as small as possible, as the packets transmitted by these UEs will be dropped. On the other hand, the smaller number of miss-detected UEs will lead to larger number of false-alarmed UEs. With more false-alarmed UEs, the receiver complexity and the processing latency will be higher. Thus, the UE activity detection should achieve a good tradeoff between miss-detection and false-alarm.

Observation 1: UE activity detection should achieve a good tradeoff between miss-detection and false-alarm.

Figure 2. Tradeoff between miss-detection and false-alarm for UE activity detection

Data Detection

Given the UE activity detection, the general MUD problem is given by

y=i∈AHixi+n⟹y=Hx+n,

where A is the set of active UEs, H=[H1, H2, …, HN] is the channel matrix, x=[x1T,  x2T, …, xNT]Tis the vector of transmitted signal, and n is the noise vector.

Firstly, the MUD receiver should support overloading scenarios for grant-free at least for UL mMTC, i.e., H is rectangular and has more columns than rows. In [3] and [4], many advanced MUD receivers for non-orthogonal multiple access have been introduced, including SIC, PIC, MPA, MMSE, and their combinations. These receivers have been shown to support high overloading and provide tradeoffs between link performance and implementation complexity. These receivers can also be used for grant-free transmission.

Figure 3. Example of factor graph with signature collision

Secondly, as there is no explicit scheduling for grant-free, the signature assignment is likely to be a problem if only relying on pre-defined rules or UE-specific semi-statistic configuration when overloading. Therefore, multiple UEs may choose the same signature, e.g., codebook, sequence, interleaver, and so on. Thus, the MUD receivers for grant-free should be robust to signature collision. Figure 3 shows an example of factor graph with signature collision. There are 6 variables nodes (VNs) which represent 6 UEs, and 4 function nodes (FNs) which represent 4 resources. The factor graph means every two UEs choose the same SCMA codebook. With signature collision, the maximum degree of FNs can be higher than the case without signature collision. In Figure 3, the degree of the second and the third FNs is 4, and the maximum degree is 3 if there is no signature collision.

Note the signature collision does not include RS collision, which has been discussed in [2].

Observation 2: The MUD receivers for grant-free should be robust to signature collision.

MUD Receivers under Signature Collision

In this section, we evaluate the LLS performance of different MUD receivers under signature collision, and the evaluation parameters for the LLS are listed in Table 1 in the Appendix. The following MUD receivers will be investigated: MMSE-SIC, SIC-MPA and EPA. EPA (expectation propagation algorithm [5]) is a low complexity message passing algorithm similar to MPA. More details and complexity analysis of EPA is provided in the Appendix. For simplicity, one representative MA scheme is considered for each receiver. For MMSE-SIC receiver, the considered MA scheme is MC-CDMA. For SIC-MPA and EPA receiver, the MA scheme is SCMA. In the simulation, the SE per UE is fixed to be 0.2 bps/Hz.

MMSE-SIC

Signature collision will increase the correlation between UEs, which can cause performance loss to the MMSE-SIC receiver. Assume there are 4 UEs, and each UE chooses a spreading sequence from one of the following sequences:

C1=1, 1, 1, 1, C2=1, -1, 1,- 1, C3=1, -1,- 1, 1, C4=1, 1,- 1,- 1.

Three scenarios with different collision levels are considered here: no collision, collision between 2 UEs, and collision between 4 UEs. No collision means the 4 UEs choose C1, C2, C3, C4, respectively. Collision between 2 UEs means the 4 UEs choose C1, C1, C2, C2, respectively. Collision between 4 UEs means the 4 UEs choose C1, C1, C1, C1, respectively. As shown in Figure 4, collision between 2 UEs will cause about 1dB loss compared with no collision, and collision between 4 UEs will cause about 6 dB loss at BLER equals to 0.1.

Figure 4. BLER curves of MMSE-SIC (SE=0.2bps/Hz per UE, Realistic Channel Estimation)

Observation 3: For UL grant-free transmission, MMSE-SIC receiver suffers from signature collision.

SIC-MPA

For UL grant-free transmission based on SCMA, each UE may choose any of the 6 codebooks. It has been shown that SCMA is robust to codebook collision in [6] and [7] with SIC-MPA receiver.  In Figure 5, we compare the BLER performance with fixed and random codebook allocation under different traffic load. As fixed codebook allocation has the minimum collision level, it can act as a lower bound to the random codebook allocation. MPA receiver is applied for the case of fixed codebook allocation, and SIC-MPA receiver is applied for the case of random codebook allocation.  As the complexity of SIC-MPA is on the order of Mdf, where df denotes the maximum degrees of FNs allowed in each round of MPA decoding, it becomes much higher when df increases. The results show that SIC-MPA receiver with random codebook allocation can perform close to MPA receiver with fixed codebook allocation. This means SIC-MPA receiver is robust to signature collision and can be applied for UL grant-free transmission. The results also show that SIC-MPA with df=2 can achieve a good tradeoff between performance and receiver complexity.

(a). 4UE										(b). 6UE

(c). 8UE										(d). 12UE

Figure 5. BLER curves with SIC-MPA receivers (SE=0.2bps/Hz per UE, Ideal Channel Estimation)

Observation 4: For UL grant-free transmission, SIC-MPA receiver is robust to signature collision.

EPA

According to the analysis in the Appendix, the complexity of EPA is on the order of dfM, which is much lower than the full-MPA. In Figure 6, the BLER performance of MPA and EPA receiver are compared under different traffic load. For fair comparison, the number of outer-loop iterations is fixed to be 3. It can be observed that EPA and MPA have similar performance with fixed codebook allocation, and EPA with random codebook allocation has negligible performance loss compared with fixed codebook allocation. Note with random codebook allocation, the maximum degrees of FNs can be very large. EPA is another option of low complexity receiver besides SIC-MPA, which has linear complexity w.r.t. the modulation order as well as the maximum degree of FNs.  The results in Figure 6 show that though with linear complexity, EPA receiver is robust to signature collision and is very suitable to  be applied for UL grant-free transmission.

(a). 4UE										(b). 6UE

(c). 8UE										(d). 12UE

Figure 6. BLER curves with EPA receivers (SE=0.2bps/Hz per UE, Realistic Channel Estimation)

Observation 5: For UL grant-free transmission, EPA receiver is robust to signature collision and has linear complexity w.r.t. modulation order and the number of collisions on each RE.

Conclusions

Based on the above discussions and those in [3] and [4], we have the following observations:

Observation 1: UE activity detection should achieve a good tradeoff between miss-detection and false-alarm.

Observation 2: The MUD receivers for grant-free should be robust to signature collision.

Regarding the performance of different receiver types on the tolerance to signature collision, we have

Observation 3: For UL grant-free transmission, MMSE-SIC receiver suffers from signature collision.

Observation 4: For UL grant-free transmission, SIC-MPA receiver is robust to signature collision.

Observation 5: For UL grant-free transmission, EPA receiver is robust to signature collision and has linear complexity w.r.t. modulation order and the number of collisions on each RE.

- References

“RAN1 Chairman’s Notes,” RAN1#86, Gothenburg, Sweden, Aug 22-26, 2016.

R1-1609446, “Reference signal design for UL grant-free transmission,” Huawei, HiSilicon, RAN1#86bis, Lisbon, Portugal, Oct 10-14, 2016.

R1-164390, “Transeiver implementation and complexity analysis for SCMA,” Huawei, HiSilicon, RAN1#85, Nanjing, China, May 16-20, 2016.

R1-166098, “Discussion on the feasibility of advanced MU-detector,” Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, Aug 22-26, 2016.

T. P. Minka, “A family of algorithms for approximate Bayesian inference,” Ph.D. dissertation,  Massachusetts Institute of Technology, Cambridge, MA, USA, Jan. 2001.

R1-164703, “LLS results for uplink multiple access,” Huawei, HiSilicon, RAN1#85, Nanjing, China, May 16-20, 2016.

R1-166094, “LLS Results for UL MA schemes,” Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, Aug 22-26, 2016.

- Appendix

The evaluation parameters for the LLS are listed in Table 1.

Table 1. Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Transmission Bandwidth | 4RB |
| Target spectral efficiency | Per UE spectral efficiency: 0.2 bps/Hz |
| Channel coding | LTE Turbo |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiplexed UEs | 4, 6, 8, 12 |
| Propagation channel & UE velocity | TDL-A 30ns |
| Max number of HARQ transmission | 1 |
| Channel estimation | Realistic or Ideal |

- EPA Receiver

EPA is a technique in Bayesian machine learning for approximating posterior beliefs with exponential family [x]. Mathematically, the projection of a particular distribution p into some distribution set Φ is defined as

ProjΦp=argminq∈ΦD(p||q)                                                            (1)

Where D(p|q denotes the Kullback-Leibler divergence. If p∈Φ, then the projection reduces to an identity mapping. However, in general, p∉Φ and hence the distribution projection is a nonlinear operation.

We first give an overview of the message passing procedure of MPA algorithm. As shown in Figure 7, assume that Xi is the symbol at VN i，Yk is the received symbol at FN k, and let V(k) be the set of neighboring VNs of k，F(i) be the set of neighboring FNs of i. During the t-th iteration, the message passed from i to k is given by

Ii⟶ktXi=IΔ⟶iXil∈Fi,l≠kIl⟶it-1Xi,                                              (2)

where IΔ⟶iXi is the prior information coming from the channel decoder, and Il⟶it-1Xi is the message from other FNs in F(i). The message passed from k to i is given by

Ik⟶itXi=Xj,j∈Vk,j≠if(X)j∈Vk,j≠iIj⟶ktXj,                                       (3)

where f(X) denotes the probability of receiving symbol Yk when vector X is transmitted. The bottleneck of complexity is to calculate the Equation (3).

Figure 7. Example of message passing between FNs and VNs

With EPA algorithm [5], the message update function can be written as:

Ii⟶ktXi=ProjΦpt(Xi)Ik⟶it-1Xi                                                                     (4)

Ik⟶itXi=ProjΦqt(Xi)Ii⟶ktXi                                                                   (5)

where ptXi and qt(Xi) is given by the left part of (2) and (3), respectively.

Let Φ be the set of complex Gaussian distribution, which is given by

Φ=ρ: ρXi=NCXi;μi,σi.

Then, ProjΦpt(Xi) can be described by the mean and variance as follows:

μi=a∈SptXi=aa,                                                                     (6)

σi=a∈S|a-μi|2ptXi=a.                                                              (7)

Both Ii⟶ktXi和Ik⟶itXi can be approximated with Gaussian distribution, which are denoted as NCXi;μi,kt,σi,kt and NCXi;μi,kt,σi,kt, respectively. With the above approximation, (4) can be rewritten as 
Ii⟶ktXi=NCXi;μi,σiNCXi;μi,kt-1,σi,kt-1∝NCXi;μi,kt,σi,kt,

then we have

σi,kt=1σi-1σi,kt-1-1,                                                                  (8)

μi,kt=σi,ktμiσi-μi,kt-1σi,kt-1,                                                                 (9)

where μi,kt-1 and σi,kt-1 are from the (t-1)-th iteration.

Similarly, (5) can be rewritten as

Ik⟶itXi==X/XifXj∈Vk,j≠iNCXj;μj,kt,σj,kt∝NCXi;μi,kt,σi,kt.

where the mean and variance can be calculated as follows

μi,kt=1hk,iYk-j∈Vk,j≠ihk,jμj,kt                                                          (10)

σi,kt=1hk,i2σn2+j∈Vk,j≠ihk,j2σj,kt                                                      (11)

The message passing procedures of MPA and EPA are summarized in Table 2. It can be observed that the complexity of all the equations for EPA is linear to the modulation size and the number of UEs, thus the complexity is much reduced for grant-free scenarios. Detailed complexity analysis on the other advanced receivers has been given in [4]. Here the complexity of EPA is summarized in Table 4, with the parameters defined in Table 3.

Table 2: Message passing procedures of MPA and EPA

|  | MPA | EPA |
| --- | --- | --- |
| VN ⟹FN | (2) | (6), (7), (8), (9) |
| FN ⟹VN | (3) | (10), (11) |

Table 3: Key parameters in the complexity order analysis.

| Parameters | Description | Example values |
| --- | --- | --- |
| NRx | Number of receiver antennas | 2, 4, 8, etc. |
| SF | Spreading length of a spreading block | 4, 8, etc. |
| NUE | Number of users/layers | UL: 6, 12, etc. DL: 2, 3, etc. |
| Niter | Including inner-loop and number of EPA rounds | 6, 9, 18, etc. |
| M | Constellation size | 4, 8, 16, etc. |
| df | Degree of FN nodes | 1, 2, 3, etc. |

Table 4: Complexity analysis of EPA receiver

| Rx Type | Operation Complexity Order per spreading block (Only dominant part considered) |
| --- | --- |
| EPA | OSF∙NiterdfM |