3GPP TSG RAN WG1 #86	R1-1609072

# Lisbon, Portugal, 10th – 14th October 2016

Agenda item:		8.1.3.1

Source:	Samsung

Title: 				Performance of Short-Length Polar Codes

Document for:		Discussion and Decision

1. Introduction

In RAN-1 #86 meeting, the following simulation assumptions are agreed for eMBB control channel schemes.

Agreement: Simulation assumptions for eMBB control channel coding

- Evaluate the block error rate (BLER) performance versus SNR

- Evaluate the false alarm rate versus SNR

| Channel | AWGN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | QPSK |  |  |  |  |  |  |
| Coding Scheme | Repetition | Simplex | TBCC | Turbo | LDPC | Reed-Muller | Polar |
| Code rate (for evaluation purposes) | 1/24*, 1/12, 1/6, 1/3, 1/2, 2/3 |  |  |  |  |  |  |
| Decoding algorithm** | ML | ML | List-Viterbi | Scaled max-log-MAP | Adjusted min-sum | FHT | SC-list |
| Info. block length (bits w/o CRC) (for evaluation purposes) *** | 1, 2, 4, 8, 16, 32, 48, 64, 80, 120, 200 |  |  |  |  |  |  |
| * Code rate 1/24 is valid for info block length of 1-2 bits ** Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** Each of these info. block lengths shall be evaluated at least one of the code rates. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis. Companies are encouraged to provide information on complexity of their decoders, and on decoding latency |  |  |  |  |  |  |  |

In this contribution, we provide initial evaluation results of short-length polar codes. Based on the agreements above, polar codes are designed by using the density evolution method [1]. To construct polar codes of variable length, we consider known-bit puncturing method proposed in [2]. Then, the block error rates (BLERs) and false alarm rate (FARs) of these polar codes are evaluated over AWGN channels.

2. Polar Code Construction, Encoding, and Decoding

Fig. 1 shows a general procedure of polar encoder. Let K be the number of information bits and let R denote the code rate. Then, the number of codeword bits, N=K/R. We consider a general implementation of polar codes with kernel matrix of size 2, and the length of mother polar codes, N0 is determined as

N0=2log2N.

First, N0-K zero bits are attached to the information bits to make a bit stream of length N0. The position of the insertion zero bits are determined by the polar code sequence. Then this bit stream of length N0 is fed into the polar encoder. In the encoder, the input bit stream is multiplied by N0×N0 generator matrix, and the output bit stream of length N0 is obtained. To adjust the desired length of codeword, N, N0-N bits are punctured from the output bit stream of the encoder.

Figure 1. Polar encoding procedure

To make a good polar code for a specific code length K and core rate R, a puncturing pattern and a frozen/data bit sequence should be carefully designed. First, some puncturing methods are proposed in the literature, and we consider the known-bit puncturing method [2] in this contribution. Based on the puncturing pattern, we use the density evolution (DE) method [1] to decide frozen/data bit sequence. The DE method is known as a general way of evaluating and designing low-density parity-check (LDPC) codes, and it can also be adopted to make good polar codes. According to the DE, bit error rate of each effective bit channel can be estimated for given input SNR. Then, bit channels with low BER are assigned to information bits, and bits transmitted over those with high BER are fixed to be 0.

For each combination of K and R, polar code is individually designed. In Table 1, input construction SNRs of the DE for each polar code are given.

Table 1. Polar code parameters and construction SNR

| Info. length (K) | Code rate (R) | Code length (N) | Mother code (N0) | # punctured bits | # parity bits | Construction SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 1/12 | 48 | 64 | 16 | 44 | -4 dB |
|  | 1/6 | 24 | 32 | 8 | 20 | -2 dB |
|  | 1/3 | 12 | 16 | 4 | 8 | 1 dB |
|  | 1/2 | 8 | 8 | 0 | 4 | 4 dB |
|  | 2/3 | 6 | 8 | 2 | 2 | 6 dB |
| 8 | 1/12 | 96 | 128 | 32 | 88 | -4 dB |
|  | 1/6 | 48 | 64 | 16 | 40 | -2 dB |
|  | 1/3 | 24 | 32 | 8 | 16 | 1 dB |
|  | 1/2 | 16 | 16 | 0 | 8 | 4 dB |
|  | 2/3 | 12 | 16 | 4 | 4 | 6 dB |
| 16 | 1/12 | 192 | 256 | 64 | 176 | -4 dB |
|  | 1/6 | 96 | 128 | 32 | 80 | -2 dB |
|  | 1/3 | 48 | 64 | 16 | 32 | 1 dB |
|  | 1/2 | 32 | 32 | 0 | 16 | 4 dB |
|  | 2/3 | 24 | 32 | 8 | 8 | 6 dB |
| 32 | 1/12 | 384 | 512 | 128 | 352 | -4 dB |
|  | 1/6 | 192 | 256 | 64 | 160 | -2 dB |
|  | 1/3 | 96 | 128 | 32 | 64 | 1 dB |
|  | 1/2 | 64 | 64 | 0 | 32 | 4 dB |
|  | 2/3 | 48 | 64 | 16 | 16 | 6 dB |
| 48 | 1/12 | 576 | 1024 | 448 | 528 | -4 dB |
|  | 1/6 | 288 | 512 | 224 | 240 | -2 dB |
|  | 1/3 | 144 | 256 | 112 | 96 | 1 dB |
|  | 1/2 | 96 | 128 | 32 | 48 | 4 dB |
|  | 2/3 | 72 | 128 | 56 | 24 | 6 dB |
| 64 | 1/12 | 768 | 1024 | 256 | 704 | -4 dB |
|  | 1/6 | 384 | 512 | 128 | 320 | -2 dB |
|  | 1/3 | 192 | 256 | 64 | 128 | 1 dB |
|  | 1/2 | 128 | 128 | 0 | 64 | 4 dB |
|  | 2/3 | 96 | 128 | 32 | 32 | 6 dB |
| 80 | 1/12 | 960 | 1024 | 64 | 880 | -4 dB |
|  | 1/6 | 480 | 512 | 32 | 400 | -2 dB |
|  | 1/3 | 240 | 256 | 16 | 160 | 1 dB |
|  | 1/2 | 160 | 256 | 96 | 80 | 4 dB |
|  | 2/3 | 120 | 128 | 8 | 40 | 8 dB |
| 120 | 1/12 | 1440 | 2048 | 608 | 1320 | -4 dB |
|  | 1/6 | 720 | 1024 | 304 | 600 | -2 dB |
|  | 1/3 | 360 | 512 | 152 | 240 | 1 dB |
|  | 1/2 | 240 | 256 | 16 | 120 | 6 dB |
|  | 2/3 | 180 | 256 | 76 | 60 | 6 dB |
| 200 | 1/12 | 2400 | 4096 | 1696 | 2200 | -4 dB |
|  | 1/6 | 1200 | 2048 | 848 | 1000 | -2 dB |
|  | 1/3 | 600 | 1024 | 424 | 400 | 1 dB |
|  | 1/2 | 400 | 512 | 112 | 200 | 4 dB |
|  | 2/3 | 300 | 512 | 212 | 100 | 6 dB |

3. Performance Evaluation

In this section, the BLER, FAR, and the error detection failure rate of polar codes in Table 1 are evaluated. In the performance evaluation, the CRC-aided successive cancellation list (CA-SCL) decoder is used. We consider list size 4 and 32, and the maximum length of CRC is set to 8. Details about the length of CRC used for each case are described in each subsection.

The FAR and error detection failure rate are defined as follows:

False alarm rate= # frame errors but CRC passed#  frames

Error detection failure rate= # frame errors but CRC passed# frame errors=FARBLER

If no CRC is used (for very-short length polar codes), BLER equals FAR and error detection failure rate is 1.

1) K=4

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 4 | 1/12 | 48 | 44 | 8 | 32 |
|  | 1/6 | 24 | 20 | 8 | 12 |
|  | 1/3 | 12 | 8 | 4 | 4 |
|  | 1/2 | 8 | 4 | 0 | 0 |
|  | 2/3 | 6 | 2 | 0 | 0 |

2) K=8

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 8 | 1/12 | 96 | 88 | 8 | 80 |
|  | 1/6 | 48 | 40 | 8 | 32 |
|  | 1/3 | 24 | 16 | 8 | 8 |
|  | 1/2 | 16 | 8 | 4 | 4 |
|  | 2/3 | 12 | 4 | 0 | 0 |

3) K=16

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 16 | 1/12 | 192 | 176 | 8 | 168 |
|  | 1/6 | 96 | 80 | 8 | 72 |
|  | 1/3 | 48 | 32 | 8 | 24 |
|  | 1/2 | 32 | 16 | 8 | 8 |
|  | 2/3 | 24 | 8 | 4 | 4 |

4) K=32

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 32 | 1/12 | 384 | 352 | 8 | 344 |
|  | 1/6 | 192 | 160 | 8 | 152 |
|  | 1/3 | 96 | 64 | 8 | 56 |
|  | 1/2 | 64 | 32 | 8 | 24 |
|  | 2/3 | 48 | 16 | 8 | 8 |

5) K=48

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 48 | 1/12 | 576 | 528 | 8 | 520 |
|  | 1/6 | 288 | 240 | 8 | 232 |
|  | 1/3 | 144 | 96 | 8 | 88 |
|  | 1/2 | 96 | 48 | 8 | 40 |
|  | 2/3 | 72 | 24 | 8 | 16 |

6) K=64

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 64 | 1/12 | 768 | 704 | 8 | 696 |
|  | 1/6 | 384 | 320 | 8 | 312 |
|  | 1/3 | 192 | 128 | 8 | 120 |
|  | 1/2 | 128 | 64 | 8 | 56 |
|  | 2/3 | 96 | 32 | 8 | 24 |

7) K=80

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 80 | 1/12 | 960 | 880 | 8 | 872 |
|  | 1/6 | 480 | 400 | 8 | 392 |
|  | 1/3 | 240 | 160 | 8 | 152 |
|  | 1/2 | 160 | 80 | 8 | 72 |
|  | 2/3 | 120 | 40 | 8 | 32 |

8) K=120

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 120 | 1/12 | 1440 | 1320 | 8 | 1312 |
|  | 1/6 | 720 | 600 | 8 | 592 |
|  | 1/3 | 360 | 240 | 8 | 232 |
|  | 1/2 | 240 | 120 | 8 | 112 |
|  | 2/3 | 180 | 60 | 8 | 52 |

9) K=200

| # info. bits (K) | Rate (R) | # codeword bits (N) | # total parity bits | # CRC bits | # polar parity bits |
| --- | --- | --- | --- | --- | --- |
| 200 | 1/12 | 2400 | 2200 | 8 | 2192 |
|  | 1/6 | 1200 | 1000 | 8 | 992 |
|  | 1/3 | 600 | 400 | 8 | 392 |
|  | 1/2 | 400 | 200 | 8 | 192 |
|  | 2/3 | 300 | 100 | 8 | 92 |

4. Observations

Observation 1: The larger the list size of CA-SCL decoder is, the better the BLER performance becomes.

Observation 2: The larger the list size of CA-SCL decoder is, the worse the FAR and the error detection failure rate performance becomes

Observation 3: Use of CRC for polar codes is not free. If CRC is used for correcting errors in lists of CA-SCL decoder, then error detection capability of the CRC is degraded in proportion to the list size of the CA-SCL decoder. The error detection failure rate of CA-SCL decoder with list size L and m-bit CRC is approximately L∙2-m.

References

- R. Mori and T. Tanaka, “Performance of polar codes with the construction using density evolution,” IEEE Commun. Lett., vol. 13, no. 7, pp. 519 – 521, July 2009.

- R. Wang and R. Liu, “A novel puncturing scheme for polar codes,” IEEE Commun. Lett., vol. 18, no. 12, pp. 2081 – 2084, Dec. 2014.