3GPP TSG RAN WG1 #86-bis	R1-1609102

# Lisbon, Portugal, 10th – 14th October 2016

Agenda item:	8.1.4.2

Source: 	Samsung

Title: 	Evaluation assumptions for NR network coordination

Document for:	Discussion and Decision

Introduction

In RAN1#86, the following agreements on network coordination were made [1]:

Agreements: - Study to support various interference management schemes: - Interference management over different time scales: - Semi-static/preconfigured interference management - Dynamic interference management - Interference management where signals/channels from/to UE(s) is - Transmitted from/to multiple TRPs - Transmitted from/to single TRP - The above study should consider: - Forward compatibility, e.g., for future introduction of additional interference management schemes (if any) - Low and high NR frequencies - Take into account backhaul/fronthaul latency constraints - Both TDD and FDD - Both data and control channels - Interference measurement/reporting - Taking into account interference management for advanced receivers - Taking into account various scenarios - Taking into account beam management, different antenna structures, etc.

This contribution provides simulation assumptions for evaluation of network coordination schemes for NR.

Proposals on Evaluation Assumptions for NW Coordination

As evaluated in numerous LTE study and work items, network coordination is one of the key technologies for enhanced cellular performance. In dense urban deployment scenarios, for instance, being able to provide effective network coordination in the network could change UE experience by as much as 50%. Therefore, specification support on network coordination should be carefully studied from the early phase of NR.

Since the network deployment affects various aspects such as TRP and UE distributions, statistics of channel coefficient, etc., deployment scenarios are one of the most important key factors in network coordination. Table 1 provides a brief summary on the deployment scenarios in NR [2]. Although each deployment scenarios in Table 1 has bunch of attributes, the efficiency of network coordination can be affected by some part of the parameters such as carrier frequency, layout, ISD, UE speed more significantly, since the number of coordination points in a given time duration is determined by those parameters. Table 1 shows that among twelve deployment scenarios for NR, following three scenarios are expected to have high network coordination efficiency; indoor hotspot, dense urban, and urban macro. Furthermore, these three deployment scenarios have been investigated through several LTE SIs and WIs, e.g. CoMP or FeCoMP, etc. Therefore, it is desirable to evaluate network coordination schemes with high priorities on those three deployment scenarios.

Table 1. Summary on NR deployment scenarios

| Scenarios | Carrier Freq. [GHz] | Layout | ISD | UE speed | Reference | NW coordination efficiency |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot | ~ {4, 30, 70} | 1 layer | 20m | 3km/h | FeCoMP | High |
| Dense urban | ~ {4 + 30} | 2 layers | 200m for Macro | 3km/h (80%), 30km/h (20%) | CoMP, FeCoMP | High |
| Rural | ~ {0.7, 2, 4} | 1 layer | {1732, 5000}m | 3km/h (50%), 120km/h (50%) | - | Low |
| Urban macro | ~ {2, 4, 30} | 1 layer | 500m | 3km/h (80%), 30km/h (20%) | CoMP, FeCoMP | High |
| High speed (trains) | ~ {4, 30, 70} | Macro (+ relay) | 1732m | Up to 500km/h | - | Med |
| Extreme long range | ~ {0.7, <3} | 1 layer | Isolated | Up to 160km/h | - | Low |
| Massive connection | {0.7, 2.1(opt.)} | 1 layer | {500, 1732}m | 3km/h (80 or 100%), 100km/h (20 or 0%) | - | Med |
| Highway | < 6 (~ 6) | Macro (+ RSU) | {1732, 500(opt)}m for Macro {50, 100}m for RSU | 100~300km/h | - | Med |
| Urban grid | < 6 (~ 6) | Macro (+ RSU) | 500m for Macro {50, 100}m for RSU | 15~120km/h [TBD] Ped./bicycle | - | Med |
| Air to Ground | < [4] | 1 (+ relay) | - | [1000]km/h | - | Low |
| Light aircraft | < [4] | 1 (+ relay) | TBD | Up to [370]km/h | - | Med |
| Satellite | 2~50 | - | - | - | - | Low |

Proposal 1: For NR network coordination schemes, it is recommended that the following three deployment scenarios be evaluated in phase 1:

- Indoor hotspot, dense urban, urban micro

As aforementioned, the three deployment scenarios, i.e. indoor hotspot, dense urban, urban micro, have been studied in FeCoMP SI [3]. However, given that the scope of [3] is limited to specific transmission schemes, such as non-coherent JT, and the general simulation assumptions for LTE were adopted, some of parameters in the simulation assumption of [3] should be carefully updated. Although TR 38.802 [4] including detailed simulation assumptions for NR can be a good reference for this update, the number of possible simulation pairs could be quite large and the results from each company can diverge. Therefore, it is recommended to follow the agreements from the email discussion [86-20] at the first stage, especially for the TRP and UE antenna configurations. Enough number of evaluation results with comparable environment would be beneficial to get intuitions and common understanding among companies. Based on the discussions above, following proposals can be made:

Proposal 2: Simulation assumptions of FeCoMP (TR 36.741) can be a starting point for the evaluation of NR network coordination schemes with following refinement

- Carrier frequency

- Subcarrier spacing

- System bandwidth

- Coordination cluster size

- Channel model

- TRP and UE antenna configuration

- UE mobility (UE movement, UE rotation, …)

- Baseline scheme

- Etc.

Proposal 3: For further refinement on proposal 2, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted

A detailed example for the proposed simulation assumptions for network coordination is provided by Appendix.

Conclusions

This contribution discusses about evaluation assumptions for NR network coordination and provides following proposals.

Proposal 1: For NR network coordination schemes, it is recommended that the following three deployment scenarios be evaluated in phase 1:

- Indoor hotspot, dense urban, urban micro

Proposal 2: Simulation assumptions of FeCoMP (TR 36.741) can be a starting point for the evaluation of NR network coordination schemes with following refinement

- Carrier frequency

- Subcarrier spacing

- System bandwidth

- Coordination cluster size

- Channel model

- TRP and UE antenna configuration

- UE mobility (UE movement, UE rotation, …)

- Baseline scheme

- Etc.

Proposal 3: For further refinement on proposal 2, at least the agreements from the email discussion on NR MIMO calibration [86-20] can be adopted

- References

- 3GPP, RAN1#86, Chairman’s Notes

- 3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies"

- 3GPP TR 36.741: “Study on Further enhancements to Coordinated Multi-Point (CoMP) Operation for LTE”

- 3GPP TR 38.802: “Study on New Radio (NR) Access Technology”

- Appendix

| Parameters | Indoor Hotspot | Urban Macro | Dense urban |
| --- | --- | --- | --- |
| Layout | Refer to TR 38.802 |  |  |
| ISD |  |  |  |
| Minimum distances |  |  |  |
| Carrier frequency | {4, 30}GHz | {4, 30}GHz | Macro layer: 4GHz Small cell layer: 4GHz (co-channel) 30GHz (non co-channel) |
| Coordination cluster size for ideal backhaul | All sites | 3 macro sites, 7 macro sites is optional, other coordination cluster size are not precluded | 3 macro sites with 3*3*N small cell TRPs 1 macro sites with 1*3*N small cell TRPs 7 macro sites with 7*3*N small cell TRPs is optional, other coordination cluster size are not precluded |
| Subcarrier spacing | 15kHz for CF=4GHz, 60kHz for CF=30GHz |  |  |
| System Bandwidth | 20MHz for CF=4GHz, 40MHz for CF=30GHz |  |  |
| Channel model | Refer to TR 38.802 |  |  |
| TRP antenna configurations (M,N,P,Mg,Ng), (dH,dV,dH,g,dV,g) | Baseline refer to [86-20] Other configurations in TR 38.802 are not precluded |  |  |
| TRP Tx power | Refer to TR 38.802 |  |  |
| TRP antenna pattern |  |  |  |
| TRP antenna height |  |  |  |
| Small cell TRP dropping |  |  |  |
| UE antenna height/UE dropping |  |  |  |
| Association of UE to TRP | Association method (including CRE) should be reported |  |  |
| Maximum CoMP measurement set size | Baseline 3TPs. If a different value is used, it should be indicated. |  |  |
| UE antenna configuration (M,N,P,Mg,Ng), (dH,dV,dH,g,dV,g) | Refer to [86-20] |  |  |
| UE antenna pattern | Refer to TR 38.802 |  |  |
| UE array orientation | Refer to [86-20] |  |  |
| UE receiver noise figure | Below 6GHz: 9 dB Above 6GHz: 10dB |  |  |
| Traffic model | FTP traffic model 1 as a baseline, S = 0.1Mbytes (optional) or 0.5Mbytes |  |  |
| Traffic load (Resource utilization) | <5%, 20%, 40%, 70%, Optional 80% (S=0.1Mbytes) |  |  |
| UE receiver | MMSE-IRC and CWIC as the baseline receiver (other advanced NAICS receivers are not precluded) |  |  |
| Feedback assumption | Realistic |  |  |
| Transmission scheme |  |  |  |
| RS modelling |  |  |  |
| Channel estimation |  |  |  |
| Overhead modelling |  |  |  |
| Handover margin | 3dB |  |  |
| Backhaul link delay | 0ms, 2ms (optional), 5ms, 50ms |  |  |
| Baseline scheme | Provided by each company |  |  |