3GPP TSG RAN WG1 Meeting #86bis			R1-16xxxx

Lisbon, Portugal 10th - 14th October 2016

Agenda item:	8.1.5.1

Source: 	Samsung

Title: 	Overview on issues on NR downlink synchronization

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following agreement related to initial access was made [1]:

- RAN1 should strive for a common framework, including for example structure of synchronization signals, for initial access - More specifically, especially within a group of frequency bands in the frequency range, RAN1 should strive for an unified framework covering - Single beam based and multi-beam based deployments - TDD and FDD operations - Different/mixed numerologies - Standalone and non-standalone operations - Licensed band and unlicensed band operations - FFS: mMTC use case - RAN1 should take at least following requirements into account to design initial access - Providing at least following functionalities - Detection of NR cell and its ID - Note: In this context, NR cell corresponds one or multiple TRP(s) - Initial time/frequency synchronization to the cell - Providing necessary information for random access - Providing sufficient number of the identity values to allow deployment flexibility - FFS: supporting efficient mobility - FFS: supporting efficient inter-RAT measurement - Reducing the frequency hypothesis UE needs to search for compared to LTE - FFS: detecting beam ID(s)

This contribution addresses the following issues pertaining to NR DL synchronization signals and primary broadcast channel, including functions, contents, and design consideration.

# Initial access procedure

Following the high-level principle of initial access for LTE, initial access for NR can be illustrated in Figure 1. Three signals/channels are utilized: synchronization signals which include primary and secondary synchronization signals (termed nrPSS and nrSSS), primary BCH (nrPBCH) which carries the Master Information Block (MIB), and nrPRACH.

Figure 1 High-level initial access procedure

Once initial synchronization, MIB acquisition, and random access are completed, a UE is connected to a cell and RRC connection is established (hence the UE is in a CONN-ACTIVE state). From now on, the UE is continuously engaged in RRM measurement by monitoring signal strengths from the neighbouring cells. If the UE is engaged in cell-level mobility, the UE needs to monitor nrPSS and nrSSS from other cells. For NR, this is termed RRC-level mobility (which involves L3, analogous to LTE RRM). Otherwise, the UE can simply monitor some type of RS from other cells. For NR, this is termed beam management (which is an L1/L2 procedure, without any L3 involvement).

For the rest of this contribution, we will focus on synchronization signals (nrSS, consisting of nrPSS and nrSSS) and primary broadcast channel (nrPBCH) – depicted in the first three steps of Figure 1.

# Synchronization signals and primary broadcast channel

The functions and contents of nrPSS/nrSSS as well as their design issues are discussed below.

## Functions and contents

The functions, content, and other features of LTE PSS/SSS and PBCH are shown in Table 1. For LTE, cell ID is included in both PSS (one out of three hypotheses) and SSS (cell ID group, one out of 168 hypotheses).

Table 1 LTE PSS/SSS and PBCH

|  | LTE PSS/SSS | LTE PBCH |
| --- | --- | --- |
| Function | Coarse time-frequency synchronization & cell ID acquisition | MIB acquisition, [confirming cell ID acquisition] |
| Parameters included | Cell ID (504 hypotheses), frame timing (2 hypotheses) | MIB: system BW (3 bits), PHICH info (3 bits), System Frame Number (SFN) (8 bits) + [10 reserved bits] |
| Need for blind detection | CP size (from SSS), [TDD vs. FDD] | Number of CRS ports |
| Reliability | Low to moderate | High (protected with 16-bit CRC + 1/48 effective code rate ) |
| Transmission bandwidth | 1.4 MHz (center 6 PRBs): minimum BW from eNB perspective |  |

For NR, the content and functions nrPSS/nrSSS and nrPBCH can be different from LTE PSS/SSS and PBCH in the following aspects:

- Possibility for transmitting nrPSS/nrSS and nrPBCH with variable sub-carrier spacing [2] may require blind detection of sub-carrier spacing. This may also hold for other numerology-related parameters.

- When nrPSS/nrSSS and nrPBCH are transmitted multiple (N) times across N OFDM symbols (e.g. beam sweeping is used for coverage enhancement), symbol timing information needs to be detected either from nrPSS/nrSSS or nrPBCH.

  - Since nrPBCH is expected to have higher reliability (just as LTE), including symbol timing information in nrPBCH is slightly preferable.

- If the MIB is protected with low coding rate just as LTE PBCH, the need for non-UE-transparent Tx diversity is obviated. Hence, there is no need to detect the number of CRS ports from nrPBCH.

- MIB payload can be reduced by not including PHICH information (which is not needed when the number of symbols used for DL control channel, such as PDCCH, does not change from subframe to subframe). In this case, MIB includes DL and UL system bandwidth information and SFN, but not PHICH information.

Based on the above considerations, Table 2 depicts the proposed functions and content of nrPSS/nrSSS and nrPBCH. The new parameters are highlighted in blue.

- Depending on whether nrPSS/nrSSS and nrPBCH are transmitted with variable sub-carrier spacing, it is proposed that sub-carrier spacing be blindly detected from nrPSS (while nrSSS can be complementary). This also holds if another numerology parameter needs to be variable in nrPSS/nrSSS and nrPBCH.

- Symbol timing information can be included in the MIB. Alternatively, this can either be blindly detected from nrPBCH or included in nrSS (either as a part of nrSSS or a separate (new) signal from nrPSS/nrSSS).

Table 2 nrPSS/nrSSS and nrPBCH

|  | nrPSS/nrSSS | nrPBCH |
| --- | --- | --- |
| Functions | Coarse time-frequency synchronization & cell ID acquisition | MIB acquisition, [confirming cell ID acquisition] |
| Parameters included (content) | Cell ID (NCID hypotheses) | MIB: system bandwidth information (n1 bits), SFN (8 bits), [symbol timing (n3 bits)] + [m reserved bits] |
| Need for blind detection | [Alt 1] n/a [Alt 2] sub-carrier spacing (or other numerology-related parameters) | n/a |

Proposal: Use Table 2 as a starting point in deciding functions and content of synchronization signals (consisting of nrPSS and nrSSS) and nrPBCH:

- Symbol timing can either be included in MIB, as a part of nrSS, or blindly detected from nrPBCH

## Design considerations

Following that for LTE (see Table 1), low channel coding rate should be used for nrPBCH as well. Since the payload of MIB is sufficiently small, a tail biting convolutional code similar to that of LTE can be reused along with repetition or multi-symbol transmission.

However, the following factors may lead to some differentiation in the design of nrPSS/nrSSS and nrPBCH compared to LTE PSS/SSS and PBCH:

- Possibility for transmitting nrPSS/nrSS and nrPBCH with variable sub-carrier spacing [2] needs to be taken into account in designing nrPSS and nrSSS.

- For LTE, PSS/SSS/PBCH occupies 1.4MHz (which corresponds to the minimum system bandwidth from eNB perspective). While 1.4 MHz seems to be sufficient for LTE, whether it is sufficient for NR (<6GHz and >6GHz) is still unclear. This is especially true for >6GHz (since the minimum system BW can be much larger in this case).

- When nrPSS/nrSSS and nrPBCH are transmitted multiple (N) times across N OFDM symbols (e.g. beam sweeping is used for coverage enhancement in >6GHz scenarios), multiplexing nrPSS, nrSSS, and nrPBCH solely in time-domain (across contiguous OFDM symbols) may be either infeasible or inefficient.

- For NR, an ‘always-on’ RS (such as LTE CRS) is most likely not present. Therefore, nrPBCH may either require a special demodulation RS within the nrPBCH-carrying OFDM symbol(s) or use other signals for demodulation RS (such as nrPSS and/or nrSSS).

Based on the above considerations, the following properties are proposed for designing nrPSS, nrSSS, and nrPBCH:

- Considering UE complexity and detection reliability, it is desirable to minimize the number of blind detection hypotheses. In addition, the design should accommodate multiple transmission (e.g. for beam sweeping). Therefore, a single-format nrPSS (with small transmission BW) and a dual-format nrSSS design should be studied where:

  - Format 1 nrSSS: for instance, same BW as nrPSS, TDM with nrPSS (targeted for N=1 and/or <6GHz)

  - Format 2 nrSSS: for instance, larger BW than nrPSS, FDM with nrPSS (targeted for N>1 and/or >6GHz)

- If variable sub-carrier spacing is applicable for nrPSS/nrSSS and nrPBCH transmissions, the following two alternatives are applicable and need to be studied:

  - Alt1. Maintain the same transmission BW regardless of the sub-carrier spacing, following the above dual-format nrSSS design.

  - Alt2. Linearly scale the transmission BW with sub-carrier spacing, following the above dual-format nrSSS design.

Proposal: Study the following designs for nrPSS, nrSSS, and nrPBCH:

- Single-format nrPSS and dual-format nrSSS where the two nrSSS formats are differentiated by, e.g. transmission bandwidth, multiplexing scheme with nrPSS (TDM and FDM for formats 1 and 2, respectively)

- Two alternative approaches in addressing variable sub-carrier spacing:

  - Alt1. Maintain the same transmission BW regardless of the sub-carrier spacing

  - Alt2. Linearly scale the transmission BW with sub-carrier spacing

# Conclusions

In this contribution, an overview of NR initial access, focusing on synchronization signals and PBCH (termed nrSS including nrPSS/nrSSS and nrPBCH), is given. Our proposal can be summarized as follows:

- Use Table 2 as a starting point in deciding functions and content of synchronization signals (consisting of nrPSS and nrSSS) and nrPBCH

  - Symbol timing can either be included in MIB, as a part of nrSS, or blindly detected from nrPBCH

- Study the following designs for nrPSS, nrSSS, and nrPBCH:

  - Single-format nrPSS and dual-format nrSSS where the two nrSSS formats are differentiated by, e.g. transmission bandwidth, multiplexing scheme with nrPSS (TDM and FDM for formats 1 and 2, respectively)

  - Two alternative approaches in addressing variable sub-carrier spacing:

    - Alt1. Maintain the same transmission BW regardless of the sub-carrier spacing

    - Alt2. Linearly scale the transmission BW with sub-carrier spacing

# References

- 3GPP, RAN1#85, Chairman’s Notes

- R1-16xxxx, Samsung, Numerology for synchronization signal