3GPP TSG RAN WG1 Meeting #86bis	R1-1609422

Lisbon, Portugal, October 10-14, 2016

Agenda Item:	8.1.6.2

Source:	Huawei, HiSilicon

Title:	Evaluation of flexible duplex in indoor hotspot scenario

Document for:	Discussion and decision

Introduction

In RAN1 #86 meeting, it was agreed that [1]

- NR should support at least following design targets: - It should allow FDD operation on a paired spectrum - It should allow different transmission directions in either part of a paired spectrum - It should allow TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changing - It should allow TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing - FFS: It should allow support of full duplex in a forward compatible way - Note: transmission directions include all of downlink, uplink, sidelink, and backhaul link - Note that additional discussion is needed about the timing to support above targets, particularly the second sub-bullet - Note that some design targets may or may not be transparent to UE

To study the feasibility as well as to identify the challenges of flexible duplex in various NR deployment scenarios including dense urban, urban macro and indoor hotspot, system-level evaluations should be conducted based on the agreed evaluation assumptions [3][4][5]. In this contribution, some preliminary simulation results in indoor hotspot scenario are given and discussed.

Discussion

Deployment and configuration

According to the summary on configuration of indoor TRP [2], some baseline parameters are given as:

•        Option 1 (baseline at least for calibration): Bore sight direction is perpendicular to the ceiling.  Antenna model is taken from alternative 3 in [6] (90 degree HPBW in azimuth and zenith).

–       The number of sites is 3 and 12. Placements of 12 sites and 3 sites are shown as Figure 1.

–       Antenna array baseline configuration:

•        (M,N,P,Mg,Ng) = (4, 4, 2, 1, 1) , dV=dH=0.5λ for 4GHz

- Placement for 3 TRPs                         (b) Placement for 12 TRPs

Figure 1: Layout of indoor office scenarios.

In this evaluation contribution, two layouts of TRP placement are considered for different user density.

Resource allocation and methodology

The downlink and uplink resource allocation for unpaired spectrum is discussed in [8]. To study the potential benefit of flexible duplex for unpaired spectrum, the baseline employs a semi-static subframe allocation, and all TRPs share the same subframe ratio allocation.

In RAN1 #86 meetings [3], it was agreed that FTP traffic model 3 with packet size 0.5M bytes can be used and the ratio of DL/UL traffic load are  {2:1} and{4:1}.

When the traffic ratio of DL/UL is 2:1, the baseline downlink-uplink subframe ratio is 6:4, and when the traffic ratio of DL/UL is 4:1, the baseline downlink-uplink subframe ratio is 4:1. For flexible duplex on unpaired spectrum, dynamic DL/UL subframe ratio allocation is used to adapt to the instantaneous DL/UL traffic need.

Based on assumptions and parameters shown in appendix A, several scenarios with different TRP deployments are evaluated by varying system traffic load, ratio of DL to UL, static or adaptive DL/UL subframe configurations, as shown in Table 1.

Table 1: Simulation parameters and resource allocations

|  | Baseline (Traditional TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | semi-static subframe allocation - For DL:UL traffic ratio = 2:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 4:1, DL/UL subframe = 4:1 | Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | 4GHz: 20MHz (DL+UL) |  |
| TRP Number | 3 or 12 |  |

To evaluate flexible duplex with flexible resource allocation, user perceived throughput (UPT) is used as evaluation metric, same as in LTE eIMTA [7].

Modeling of cross-link interference mitigation (CIM)

In flexible duplex, cross-link interference, e.g. TRP-to-TRP and UE-to-UE interference, exists in case that neighboring cells use different transmission directions on the same time-frequency resource as shown in Figure 2.

Figure 2: Example of cross-link interference

For the indoor office scenarios with multiple TPRs in one room, cross-link interference including TRP-to-TRP and UE-to-UE will severely degrade the potential performance gain resulted from flexible resource allocation, especially for cases with dense TPR/UE deployment. Herein, some preliminary CIM schemes, e.g. coordinated scheduling and beam-forming, are considered to assure the performance gain. The CIM discussion in detail is provided in contributions [9][10].

Simulation results

This section provides the evaluation results for some typical traffic types. Fixed ratio of UL-DL subframes is the baseline, and flexible duplex schedulers can change its subframe direction according to the instant traffic radios.

Table 2: DL and UL Performance with 3-TRP deployment on 4GHz carrier

| Radio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2:1 | Baseline | 6:4 | 26.38 | 33.96 |  | 9.3 | 44.63 |  |
|  | Flexible duplex | Dynamic |  | 50.63 | +49.09% |  | 54.87 | +22.95% |
| 4:1 | Baseline | 4:1 | 22.27 | 46.74 |  | 11.45 | 23.98 |  |
|  | Flexible duplex | Dynamic |  | 50.95 | +9.01% |  | 54.50 | +127.26% |

Table 3: DL and UL Performance with 12-TRP deployment on 4GHz carrier

| Radio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2:1 | Baseline | 6:4 | 31.31 | 28.25 |  | 11.13 | 41.69 |  |
|  | Flexible duplex | Dynamic |  | 42.69 | +51.11% |  | 48.81 | +17.09% |
| 4:1 | Baseline | 4:1 | 31.12 | 37.04 |  | 11.56 | 22.96 |  |
|  | Flexible duplex | Dynamic |  | 39.06 | +5.46% |  | 47.99 | +108.99% |

The following observations can be made from Tables 2 and 3.

Observation:

Flexible duplex with cross-link interference mitigation can improve the DL and UL performance in indoor hotspot scenario for 4GHz unpaired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission, it achieves significant gain in downlink (e.g., up to 51%) and in uplink (e.g., up to 127%) in indoor hotspot scenario.

Conclusions

In this contribution, we provided some evaluation results of flexible duplex focused on indoor hotspot scenario. The simulation result show gains for typical DL/UL traffic types.

The following observations can be made,

Observation: Flexible duplex with cross-link interference mitigation can improve the DL and UL performance in indoor hotspot scenario for 4GHz unpaired spectrum. Preliminary evaluation results shows that with flexible DL and UL transmission, it achieves significant gain in downlink (e.g., up to 51%) and in uplink (e.g., up to 127%) in indoor hotspot scenario.

- References

- 3GPP, Chairman's Notes, RAN1 #86, Aug 2016.

- Email discussion on “Indoor TRP Configuration”.

- R1-168053, “WF on scenarios and evaluation assumptions for flexible duplex,” RAN1#86, Gothenburg, Sweden, Aug 22 - 26, 2016.

- R1-168372, “WF on channel model issues for flexible duplex evaluation,” RAN1#86, Gothenburg, Sweden, Aug 22 - 26, 2016.

- R1-168373, “ WF on penetration issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, LG Electronics, Gothenburg, Sweden, Aug 22 - 26, 2016.

- R1-165850, “WF on antenna placement in indoor scenarios,” RAN1#85, Nanjing, China, May 23 - 27, 2016.

- 3GPP TR 36.828 V2.0.0, “Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation,” June 2012.

- R1-1609421, “General discussion on flexible duplex,” RAN1#86bis, Huawei, HiSilicon, Lisbon, Portugal, Oct 10 - 14, 2016.

- R1-1608831, “Discussion on UE-to-UE interference mitigation schemes,” RAN1#86b, Huawei, HiSilicon, Lisbon, Portugal, October 10-14, 2016

- R1-1608830, “Discussion on TRP-to-TRP interference mitigation schemes,” RAN1#86b, Huawei, HiSilicon, Lisbon, Portugal, October 10-14, 2016

- Appendix A: Summary of simulation parameters

| Parameters | Indoor hotspot |
| --- | --- |
| Layout | Single layer - Indoor floor: (3/12TRPs per 120m x 50m) |
| Inter-BS distance | 40m/20m |
| Minimum BS-UE (2D) distance | 0m |
| Carrier frequency | 4GHz on unpaired spectrum |
| Aggregated system bandwidth | 4GHz: Up to 200 MHz (DL+UL) |
| Simulation bandwidth | 20MHz per CC for 4GHz |
| Channel model | Below 6GHz: TRP-to-UE: ITU InH TRP-to-TRP: ITU InH (h_UE=3m) UE-to-UE: A.2.1.2 in TR36.843 |
| Penetration loss | Follow [5] |
| BS Tx power | Below 6GHz: 24 dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24 dBm |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | Below 6GHz: Baseline: (M,N,P,Mg,Ng)=(4,4,2,1,1) (dH,dV)=(0.5,0.5)λ |
| BS antenna configuration |  |
| BS antenna height | 3m |
| BS anteena element gain + connector loss | 5dBi |
| BS antenna tilt | 0deg |
| BS receiver noise figure | Below 6GHz: 5 dB |
| UE antenna elements | Omni TX2RX2 for start point |
| UE antenna height | 1.5m |
| UE antenna gain | For below 6GHz: Follow the modeling of TR36.873 |
| UE receiver noise figure | Below 6GHz: 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes Ratio of DL/UL traffic ={2:1}with {λDL, λUL} ={0.25, 0.125}, and Ratio of DL/UL traffic ={4:1}with {λDL, λUL} ={ 0.3,0.075} |
| UE distribution | For FTP traffic model 3: 10 users per macro TRP 100% indoor (3km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| UE association | based on RSRP measurement |
| Transmission mode | SU-MIMO with rank adaptation |