3GPP TSG-RAN WG1 #86bis  							R1-1609498

# Lisbon, Portugal, 10th – 14th October, 2016

Source: 	Intel Corporation

Title:	System-level analysis of UL NOMA schemes

Agenda item:	8.1.1.2

Document for:	Discussion and Decision

# Introduction

The following set of agreements was made by RAN1 WG at the previous RAN1#86 meeting [1] with respect to analysis of different UL NOMA schemes:

- NR should target to support UL non-orthogonal multiple access, in addition to the orthogonal approach, targeting at least for mMTC

- NR should target to support UL “autonomous/grant-free/contention based” at least for mMTC

- At least the following options for “autonomous/grant-free/contention based” UL transmission should be studied

  - Opt. 1: a UE performs random resource selection

    - Details FFS

  - Opt. 2: a UE’s resource is pre-configured by eNB or pre-determined

    - Details FFS

  - Other options are not precluded

- Continue study at least the following:

  - Handling of  potential collisions of MA signatures

  - Retransmission/repetition and potential combining, e.g. HARQ

  - Potential link adaptation, e.g. MCS/signature re-assigning

  - Relationship between grant-free and grant-based transmissions and associated UE behavior

  - Advanced receiver capabilities including complexity analysis

In this contribution, we provide initial system-level analysis of NOMA schemes, focusing on analysis of main benefits from UL power control, HARQ and advanced PIC receivers for UL NOMA scenario.

# On System-level Analysis of NOMA Schemes

In our companion contribution [2], we show that NOMA techniques have significant advantage over OMA schemes in terms of sum spectral efficiency. In this document, we highlight main technical challenges of grant-free transmission schemes and provide initial system-level analysis of NOMA performance.

## Impact of In-band Emission

The UL grant-free transmissions assume minimum control signaling from serving Base Station(BS), reduced latency and power consumption increasing device battery lifetime. In case of inaccurate power control settings or mix of UEs with different levels of RX power share the same subframe.

There are several solutions that could help to decrease the impact of IBE effects:

- Transmission time pattern – UEs with different received power levels could be separated from each other by specific transmission time pattern.

- Configuration of different resource pools for UEs with different coverage extension levels

- Open loop power control mechanism – UE may adjust it’s transmission power, based on DL measurements, ACK statistic from serving BS and etc.

Observation 1

- Appropriate OLPC mechanisms should be studied and the potential impact from IBE should be investigated.

## On HARQ Support

In this section, we highlight the importance of HARQ support in scenario with high loading condition. One of the requirements for mMTC is the support of UEs in deep coverage [3]. In this case, UEs typically experience very low SNRs, even if maximum power for transmission is utilized. Note that even if link budget is sufficient, there may be still substantial interference that may cause failure of packet reception. In order to cope with these effects the retransmissions can be utilized to improve NOMA performance.

Figure 1 shows system level performance in terms of PDR (Packet Drop Ratio) vs PAM (Racket Arrival Rate). The initial analysis assumes ideal HARQ feedback and shows significant performance gain from utilization of HARQ operations for NOMA transmission. Depending on synchronous and asynchronous HARQ mode, serving BS can perform combining for initial transmission and retransmission at the receiver side (synchronous HARQ) that further improves performance in contrast to asynchronous HARQ mode when there is no combining at the receiver side. It can be also observed that higher HARQ performance gains are achieved at high loading condition in case when power control is applied.

Figure 1. Simulation setup.

| Number of PRBs per UE transmission | 1 |
| --- | --- |
| HARQ reTX number | 0,1,3 |
| Receiver | MMSE-IRC |
| TBS | 20 bytes including CRC |

| 1 PRB, power control | 1 PRB, TX max power |
| --- | --- |

Figure 2. System performance with HARQ support.

Observation 2

- HARQ retransmissions can significantly increase PDR system performance

- For scenarios with high PAR, power control becomes more important and essential to increase system performance.

## Comparisons for various NOMA schemes

In this subsection, we provide initial performance comparison of different NOMA schemes, described in [2]. In Figure 2, system-level analysis of different NOMA schemes is presented. The system level evaluation result were obtained under the following assumptions:

- No HARQ.

- Grant-free random resource selection (except ‘Genie-aided FDMA’ scheme – see details below).

- Resources for transmission are chosen

- Transmission bandwidth: 6 PRBs per UE transmission for all schemes (except ‘Genie-aided FDMA’ scheme – see details below).

- Evaluated transmission schemes:

- Genie-aided FDMA – OMA transmission with optimized number of PRB for each PAR value.

  - Different UE allocation sizes 1, 2, 3, 6 were analyzed for each PAR value and the best PDR value was used for each PAR (the packet drop due to packet drop timer is included, 40 ms)

  - MMSE-IRC receiver is used

- LCRS with MMSE-IRC PIC – low code rate transmission with MMSE-IRC parallel interference cancellation (PIC)

- System bandwidth: 6 PRBs.

- PIC simulation assumption: all decode UEs are suppressed with certain dB level (20 dB)

- Power control: fixed target SNR = 0. dB.

For small PAR values (interference free region), all schemes provide similar performance. As PAR increases, the difference between curves become noticeable. For high PAR values, PIC receivers provide significant performance gains compared to OMA and NOMA with MMSE-IRC receiver.

6 PRB, power control

Figure 3. System performance of different NOMA schemes.

Observation 3

- Utilization of enhanced MMSE-IRC PIC receivers (LCRS+PIC) significantly increase UL OMA system performance.

Proposal 1

- UL NOMA scenarios should be studied with IC receivers that have potential to increase overall system performance.

# Conclusions

In this contribution, we provided initial system-level analysis for NOMA schemes with grant-less uplink transmission in NR. Based on the presented discussion, we summarized our view through the following proposals and observations:

Observation 1

- Appropriate OLPC mechanisms should be studied and the potential impact from IBE should be investigated.

Observation 2

- HARQ retransmissions can significantly increase PDR system performance

- For scenarios with high PAR, power control becomes more important and essential to increase system performance.

Observation 3

- Utilization of enhanced MMSE-IRC PIC receivers (LCRS+PIC) significantly increase UL OMA system performance.

Proposal 1

- UL NOMA scenarios should be studied with IC receivers that have potential to increase overall system performance.

# References

- Chairman’s notes, RAN1 #86, Gothenburg, Sweden, August, 2016.

- R1-1609497, “On UL non-orthogonal multiple access schemes”, Intel Corporation, Lisbon, Portugal, October, 2016.

- 3GPP TR 39.913: " Study on Scenarios and Requirements for Next Generation Access Technologies ".

# Appendix: Simulation Assumptions

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 700 MHz |
| Waveform | OFDM |
| Channel coding | Turbo |
| Numerology | Same as Release 13 |
| System Bandwidth | 1.4 MHz / 6 PRB |
| Total allocated bandwidth for transmission | 1,6 PRB |
| Resource allocation assumption | Random resource selection |
| TBS | 20 bytes including CRC |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1 Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| Propagation channel | CM 36.873 |
| Max number of HARQ retransmission | 1,3 |
| Receiver structure | MMSE-IRC and MMSE-PIC |
| Channel estimation | Ideal |