3GPP TSG-RAN WG1 #86 Bis																		R1-1609658

Lisbon, Portugal, October 10-14, 2016

Agenda item:		8.1.2.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Supported numerologies for NR Phase I

Document for:		Discussion and Decision

# 1	Introduction

An objective of the 5G study item [1] is to identify and develop technology components needed for new radio (NR) systems being able to use any spectrum band ranging at least up to 100 GHz. The goal is to achieve a single technical framework addressing all usage scenarios, requirements and deployment scenarios defined in TR38.913 [2]. The learnings from LTE have shown that in order to support this wide range of spectrum bands is not efficient with a single OFDM numerology.

The following agreements on numerology was made in RAN1#86 [3]:

Agreements:

- NR numerology scalability should allow at least from [3.75 kHz] to 480 kHz subcarrier spacing

- Necessity of support for less than 15 kHz subcarrier spacing (e.g., 3.75 kHz) should be studied

- Note that scalability does not mean everything should be scalable (e.g., RS density, UE/gNB processing time, signalling overhead)

In this contribution we provide the Nokia view on sub-carrier spacing that should be supported part of the first NR specifications.

# 2	Discussion on sub-carrier spacing options for Phase I

The coexistence and compatibility with LTE as well as the shown suitability of 15 kHz sub-carrier spacing to macro-cellular deployments makes 15 kHz support as the natural selection from the f0*2m, where f0 = 15 kHz. The downside of 15 kHz sub-carrier spacing is its limited bandwidth support without large-scale carrier aggregation, and its suitability for deployment in higher bands.

The NR design should be forward compatible so that regardless of what subset of the numerologies in the table 1 is adopted in the first NR release, it should be possible to introduce additional numerologies from the same set in a straight forward manner.

Table 1: Range of potential numerologies with normal CP scaled from LTE numerology

| Subcarrier spacing [kHz] | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol duration [us] | 267 | 133 | 66.7 | 33.3 | 16.7 | 8.33 | 4.17 | 2.08 | 1.04 |
| Nominal CP [us] | 19 | 9.4 | 4.7 | 2.3 | 1.2 | 0.59 | 0.29 | 0.15 | - |
| Nominal max BW [MHz] | 5 | 10/12 | 20/25 | 40/50 | 80/100 | 160/200 | 320/400 | 640/800 | 1280/1600 |
| Max FFT size | 2048 | 2048 | 2048 | 2048 | 2048 | 2048 | 2048 | 2048 | 2048 |
| Basic scheduling unit* (symbols) | 7 | 7 | 7 | 7 | 7 | 14** | 28** | 56** | 120** |

*	assuming slot-based scheduling and 7-symbol slot
**	assuming either concatenation of slots or a longer slot to limit the UL/DL switching frequency

The 60 kHz sub-carrier spacing offers a good combination of wider bandwidth, robustness against phase noise as well as still fairly decent CP suitable for micro-cellular deployments.

In addition to 15 and 60 kHz, another numerology should be considered that allows for support of wide-bandwidth without carrier aggregation for bands >6 GHz and smaller cells. We see 240 kHz as a good sub-carrier spacing candidate for this purpose.

Sub-carrier spacings below 15 kHz could be used to serve a special purpose, such as mMTC traffic, and could be considered in Phase II

Sub-carrier spacings above 240 kHz, especially 960 kHz would be beneficial for ultra-wide band single carrier waveforms targeted for > 40 GHz systems, and should be considered further in Phase II.

# 3	Proposed sub-carrier spacing set for Phase I

Hence we propose the following set of numerologies to be initially supported for NR targeting to bands below 40 GHz.

Table 2: Proposed set of numerologies to proceed with in the NR Study Item (Normal CP only shown)

| Subcarrier spacing [kHz] | 15 | 60 | 240 |
| --- | --- | --- | --- |
| Symbol duration [us] | 66.7 | 16.7 | 4.17 |
| Nominal CP [us] | 4.7 | 1.2 | 0.29 |
| Nominal max BW [MHz] | 20/25 | 80/100 | 320/400 |
| Max FFT size | 2048 | 2048 | 2048 |
| Basic scheduling unit* (symbols) | 7 | 7 | 28** |

*	assuming slot-based scheduling and 7-symbol slot
**	assuming either concatenation of slots or a longer slot to limit the UL/DL switching frequency

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #86, 3GPP

Approximate values scaled from LTE Normal CP – extended CP may be considered