3GPP TSG RAN WG1 Meeting #86-bis                                                         R1-1609680

Lisbon, Portugal 10th - 14th May 2016

Agenda Item:	7.1.6

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Clarifications about NPRACH configuration parameters

Document for:	Discussion and Decision

# Introduction

This document discusses the validity of some values related to the time configuration of the NPRACH resource. The objective of this document is to clarify the constraints related to some NPRACH resource parameters for the sake of Specifications completeness and in order to avoid wrong implementation.

# Discussion

## Issue #1: 240ms periodicity

NPRACH transmission can start only NstartNPRACH·30720·TS=NstartNPRACH·1ms time units after the start of a radio frame fulfilling nf·modNperiodNPRACH10=0 where nf  is the System Frame Number (SFN).

Table 1: Allowed Starting SFNs to Initiate a NPRACH period

| NperiodNPRACH(ms) | Starting SFNs to initiate a NPRACH Period |
| --- | --- |
| 40 | SFNs that are multiple of 4 |
| 80 | SFNs that are multiple of 8 |
| 160 | SFNs that are multiple of 16 |
| 240 | SFNs that are multiple of 24 |
| 320 | SFNs that are multiple of 32 |
| 640 | SFNs that are multiple of 64 |
| 1280 | SFNs that are multiple of 128 |
| 2560 | SFNs that are multiple of 256 |

On the other hand, SFN takes value from 0 to 1023. If the periodicity value of the NPRACH resource is 240ms, the NPRACH period can start only at frames that are multiple of 24, but in this case the latest period into a set of 1024 SFNs will be shorter than the other periods (See Table 2). With a 240ms periodicity there will be 42 periods of 240ms and one period of 160ms, meaning that for the 240ms periodicity value the maximum length of the NPRACH opportunity is actually 160ms instead of 240ms.

Table 2: Allowed SFNs to Initiate a NPRACH Period with 240ms Periodicity

| Periodicity = 240ms SFNs multiple of 24 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 240 | 360 | 480 | 600 | 720 | 840 | 960 |
| 24 | 144 | 264 | 384 | 504 | 624 | 744 | 864 | 984 |
| 48 | 168 | 288 | 408 | 528 | 648 | 768 | 888 | 1008 |
| 72 | 192 | 312 | 432 | 552 | 672 | 792 | 912 | - |
| 96 | 216 | 336 | 456 | 576 | 696 | 816 | 936 | - |

Observation 1: for the 240ms NPRACH periodicity the maximum length of the NPRACH opportunity is actually 160ms instead of 240ms.

## Issue #2: Restriction between start-time and periodicity of NPRACH preamble

There are two parameters that define the start of a NPRACH preamble:

- nprach-Periodicity: Periodicity of a NPRACH resource. Unit in millisecond. {40, 80, 160, 240, 320, 640, 1280, 2560} ms. NperiodNPRACH

- nprach-StartTime: Start time of the NPRACH resource in one period. Unit in millisecond. {8, 16, 32, 64, 128, 256, 512, 1024} ms. NstartNPRACH

It has no sense to define a start-time higher than the periodicity because in this case the NPRACH preamble will start in a SFN of a different period.

Figure 1: Example of confused configuration of start-time and periodicity of the NPRACH resource

Observation 2: the following restriction holds:

“NPRACH configurations with NperiodNPRACH<NstartNPRACH are not valid.”

## Issue #3: Restriction between start-time, periodicity and number of repetitions of the NPRACH preamble

Into the set of values defined for the configuration of a NPRACH resource, there are four parameters that are directly related to the timing of the NPRACH opportunity:

- nprach-Periodicity: Periodicity of a NPRACH resource. Unit in millisecond.{40, 80, 160, 240, 320, 640, 1280, 2560} ms.

- nprach-StartTime: Start time of the NPRACH resource in one period. Unit in millisecond. {8, 16, 32, 64, 128, 256, 512, 1024} ms

- nprach-NumRepetitions: Number of NPRACH repetitions per attempt for each NPRACH resource. {1, 2, 4, 8, 16, 32, 64, 128} repetitions.

- nprach-CP-Length: Cyclic prefix length for NPRACH transmission. {66.7 or 266.7}s

If any combination of these values is allowed, it is possible that the NPRACH preamble of two periods (or more, depending on the number of repetitions) overlaps. There should be some restrictions at time of defining these parameters in order to avoid the overlapping of the NPRACH opportunities. Figure 2 shows an example of how the overlapping could occur depending on the values of the parameters listed above. Table 3 shows the value of the parameters used in the example. We can see how when we introduce 8 repetitions, the NPRACH preamble invades the next period, but with a lower number of repetitions there is not any overlapping (Figure 3).

Table 3: Configuration of the parameters used for the Overlapping explanation

|  | Non-Overlapping Situation | Overlapping Situation |
| --- | --- | --- |
| nprach-Periodicity | 40 ms | 40 ms |
| nprach-StartTime | 8ms | 8ms |
| Nprach-NumRepetitions | 4 repetitions | 8 repetitions |
| nprach-CP-Length | 66.7 us | 66.7us |

Figure 2: Example of NPRACH overlapping

Figure 3: Example of NPRACH Non-Overlapping

Taking this fact into account, there will be some not valid combination of parameters. Table 4 and Table 5 show the different allowed and not allowed combinations for NPRACH Format 0 and Format 1 respectively, taking into account the observation 1 related to the 240ms NPRACH periodicity.

Table 4: Maximum Number of Repetitions for Format 0 Depending on the Periodicity and the Start-Time Parameters

| Format 0 Start time | NPRACH Periodicity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40 | 80 | 160 | 240 | 320 | 640 | 1280 | 2560 |
| 8 | 4 | 8 | 16 | 16 | 32 | 64 | 128 | 128 |
| 16 | 4 | 8 | 16 | 16 | 32 | 64 | 128 | 128 |
| 32 | 1 | 8 | 16 | 16 | 32 | 64 | 128 | 128 |
| 64 | - | 2 | 16 | 16 | 32 | 64 | 128 | 128 |
| 128 | - | - | 4 | 4 | 32 | 64 | 128 | 128 |
| 256 | - | - | - | - | 8 | 32 | 128 | 128 |
| 512 | - | - | - | - | - | 16 | 128 | 128 |
| 1024 | - | - | - | - | - | - | 32 | 128 |

Table 5: Maximum Number of Repetitions for Format 1 Depending on the Periodicity and the Start-Time Parameters

| Format 1 Start time | NPRACH Periodicity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40 | 80 | 160 | 240 | 320 | 640 | 1280 | 2560 |
| 8 | 4 | 8 | 16 | 16 | 32 | 64 | 128 | 128 |
| 16 | 2 | 8 | 16 | 16 | 32 | 64 | 128 | 128 |
| 32 | 1 | 4 | 16 | 16 | 32 | 64 | 128 | 128 |
| 64 | - | 2 | 8 | 8 | 32 | 64 | 128 | 128 |
| 128 | - | - | 4 | 4 | 16 | 64 | 128 | 128 |
| 256 | - | - | - | - | 8 | 32 | 128 | 128 |
| 512 | - | - | - | - | - | 16 | 64 | 128 |
| 1024 | - | - | - | - | - | - | 32 | 128 |

These restrictions can be formulated mathematically with the next expressions:

If  NperiodNPRACH ≠240:

NperiodNPRACH-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ 				NrepNPRACH<64

NperiodNPRACH-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ+40·30720·Ts 		NrepNPRACH≥64

If NperiodNPRACH=240:

160-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ

Observation 3: the following restriction holds:

“To avoid overlapping between the NPRACH preambles the configuration of NperiodNPRACH, NstartNPRACH, and NrepNPRACH shall fulfil the next statements:

If  NperiodNPRACH ≠240:

NperiodNPRACH-NstartNPRACH≥NrepNPRACH·4·TCP+TSEQ 	 	                            NrepNPRACH<64

NperiodNPRACH-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ+40·30720·Ts 		NrepNPRACH≥64

If NperiodNPRACH=240:

160-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ

Observation 4: the restriction corresponding to observation 2 is taken into account by observation 3.

Proposal: to include the restriction corresponding to observation 3 within the TS 36.211. A companion CR is provided (see [3]).

# Conclusion

In this contribution, we consider the constraints related to some NPRACH resource parameters and make the following observations and proposals:

Observation 1: for the 240ms NPRACH periodicity the maximum length of the NPRACH opportunity is actually 160ms instead of 240ms.

Observation 2: the following restriction holds:

“NPRACH configurations with NperiodNPRACH<NstartNPRACH are not valid.”

Observation 3: the following restriction holds:

“To avoid overlapping between the NPRACH preambles the configuration of NperiodNPRACH, NstartNPRACH, and NrepNPRACH shall fulfil the next statements:

If  NperiodNPRACH ≠240:

NperiodNPRACH-NstartNPRACH≥NrepNPRACH·4·TCP+TSEQ 	 	                            NrepNPRACH<64

NperiodNPRACH-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ+40·30720·Ts 		NrepNPRACH≥64

If NperiodNPRACH=240:

160-NstartNPRACH≥NrepNPRACH*4*TCP+TSEQ

Observation 4: the restriction corresponding to observation 2 is taken into account by observation 3.

Proposal: to include the restriction corresponding to observation 3 within the TS 36.211. A companion CR is provided (see [3]).

# References

[1]	3GPP TS 36.211 V13.3.0 Physical channels and modulation (Release 13), September 2016

[2]	3GPP TS 36.231 V13.3.0 Radio Resource Control (RRC); Protocol specification (Release 13), September 2016

[3]	R1-1609679, TS 36.211 CR, “Clarifications about NPRACH configuration parameters”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#86-bis, Lisbon, Portugal