3GPP TSG RAN WG1 Meeting #86bis		R1-1609703

Lisboa, Portugal, 10th - 14th October 2016

Source: 	Ericsson

Title:	Antenna pattern at UE/RSU in NR eV2X

Agenda Item:	8.1.8

Document for:	Discussion and Decision

# Introduction

In 3GPP TR 38.802 (v0.1.0) [1], the agreements on UE antenna related evaluation assumptions are made for NR eV2X study, as shown below.

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink | Macro to/from vehicle/pedestrian UE : 2 GHz or 4GHz Between vehicle/pedestrian UE: 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz Note: Agreed value does not mean non-ITS band is precluded for real deployment for sidelink |
| UE Tx power | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded | Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBm is not precluded |
| UE antenna height | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m |
| UE antenna gain | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi |
| Number of UE antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements UE-type RSU: Up to 8 TX/RX antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements. UE-type RSU: Up to 8 TX/RX antenna elements |
| UE antenna element gain pattern | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded Note: uniform antenna models should be used for 2-D channel models | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded Note: uniform antenna models should be used for 2-D channel models |

In this paper, we will revisit the UE antenna gain by different antenna pattern at a vehicle UE and at a UE-type RSU, respectively. The antenna pattern proposed in this paper applies to all carrier frequencies. Discussions about operating NR eV2X at high carrier frequencies e.g., 63GHz, are detailed in our company contribution [2].

# Antenna pattern at a vehicle UE

In the current assumption, the antenna gain at a vehicle UE is 3dBi. In fact, this gain can be further improved by directional antenna pattern. Hence, we propose the following two alternatives for antenna deployment at a vehicle UE.

In Alt 1, there are 4 sets of antenna arrays that point to the front, rear, left, and right, respectively. Moreover, each antenna element irradiates with half spherical uniform distribution over a 90 degree azimuth angle. In this way, the antenna gain is obtained as 10*log10(2*4)=9dBi. For instance, when the number of antenna elements is 8, a deployment example is illustrated in Fig. 1. As shown in Fig. 1, there are 4 antenna arrays with 2 elements each and the arrays point to the front, rear, left, and right, respectively.

Fig.1 Alt 1 of antenna pattern at a vehicle UE

In Alt 2, there are 2 sets of antenna arrays that point to the front and rear, respectively. Moreover, each antenna element irradiates with half spherical uniform distribution over a 180 degree azimuth angle. In this way, the antenna gain is obtained as 10*log10(2*2)=6dBi. For instance, when, when the number of antenna elements is 8, a deployments example is ullustrated in Fig. 2. As shown in Fig. 2, there are 2 antenna arrays with 4 elements each and the arrays point to the front and rear, respectively.

Fig.2 Alt 2 of antenna pattern at a vehicle UE

Proposal 1: There are two options for antenna pattern at a vehicle UE.

- Alt 1: there are 4 sets of antenna arrays that point to the front, rear, left, and right, respectively. Moreovev, each antenna element irradiates with half spherical uniform distribution over a 90 deg azimuth angle.

- Alt 2: there are 2 sets of antenna arrays that point to the front and rear, respectively. Additionally, each antenna element irradiates with half spherical uniform distribution over a 180 deg azimuth angle.

- FFS: select between Alt 1 and Alt 2 based on the imput from automotive companies.

# Antenna pattern at a UE-type RSU

In the current assumption, the antenna gain at a UE-type is 3dBi. Similarly, this gain can also be improved by directional antenna pattern. We propose the following antenna patter.

At a UE-type RSU, there are 4 sets of antenna arrays that point to the 4 streets in the intersection, respectively. Moreover, each element irradiates with half spherical uniform distribution over a 90 degrees azimuth angle. In this way, the antenna gain is obtained as 10*log10(2*4)=9dBi. When the number of antenna elements is 8, a deployment example is illustrated in Fig. 3. As shown in Fig. 3, there are 4 antenna arrays with 2 elements each and the array point to the 4 streets in the intersection, repectively.

Fig.3 antenna pattern at a UE-type RSU

Proposal 2: At a UE-type RSU, there are 4 sets of antenna arrays that point to the 4 streets in the intersection, respectively. Moreovev, each antenna element irradiates with half spherical uniform distribution over a 90 degrees azimuth angle.

# Conclusions

In this paper, we revisit and discuss the antenna pattern at a vehicle UE and at a UE-type RSU, and have proposed the following.

Proposal 1: There are two options for antenna pattern at a vehicle UE.

- Alt 1: there are 4 sets of antenna arrays that point to the front, rear, left, and right, respectively. Moreovev, each antenna element irradiates with half spherical uniform distribution over a 90 deg azimuth angle.

- Alt 2: there are 2 sets of antenna arrays that point to the front and rear, respectively. Additionally, each antenna element irradiates with half spherical uniform distribution over a 180 deg azimuth angle.

- FFS: select between Alt 1 and Alt 2 based on the imput from automotive companies.

Proposal 2: At a UE-type RSU, there are 4 sets of antenna arrays that point to the 4 streets in the intersection, respectively. Moreovev, each antenna element irradiates with half spherical uniform distribution over a 90 deg azimuth angle.

# References

[1]   3GPP TR 38.802 V0.1.0, Study on new radio (NR) access technology; physical layer aspects.

[2]   R1-1609713 Discussions on simulation scenarios of NR eV2X at 63GHz.