3GPP TSG RAN WG1 Meeting #86bis	R1-1609787

Lisbon, Portugal, 10th – 14th October 2016

Agenda item:		7.2.1.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On resource selection for V2P communication

Document for:		Discussion and Decision

# Introduction

In RAN#72, a work item proposal [1] for LTE-based V2X services was approved. The following objective for V2P service was specified:

3) To specify enhancements for support of V2P service:

  - Random resource selection for P-UEs potentially on the PC5 resource pool shared with V-UE transmissions, with additional study on sensing operation during a limited time for P-UEs [RAN1, RAN2]

In this contribution, we first discuss resource selection options for V2P communication, and then discuss congestion control at pedestrian UEs to avoid resource pool overloaded.

# Resource selection for V2P communication

## Resource selection

To improve road safety for pedestrians, either V2P (vehicular to pedestrian) communication or P2V (pedestrian to vehicular) communication can be used. While the majority of vehicular data traffic is of 100ms period, pedestrian data traffic usually has much longer period (i.e.1s). It’s not power efficient for pedestrian UEs to receive road safety packets from vehicular UEs all the time. Hence it’s preferred to support only P2V communication for road safety purpose.

For resource selection at pedestrian UEs, there are two options. One option is random resource selection, where a pedestrian UE randomly select transmission sub-frames and frequency resources (RBs) in transmission sub-frames. This removes the need of sensing based on packet reception. The other option is resource selection based on sensing operated during a limited time for pedestrian UEs. This is proposed to improve system level performance for P2V communication. The option consumes more power than random resource selection.

On the top of random resource selection, semi-persistent transmission can be employed at pedestrian UEs to improve their system level performance. For a duration of multiple pedestrian data traffic (e.g.10 periods), a pedestrian UE uses the resources of the same time and frequency locations. Because the resource selection is fixed, the information can be sensed by vehicular UEs to aid their resource selection. Essentially vehicular UEs can try to avoid using resources occupied by pedestrian UEs. The scheme can improve P2V performance similar as resource selection based on sensing at pedestrian UEs.

For V2V traffic at PC5 interface, semi-persistent transmission is realized with counter based and probability based mechanisms. The following relevant agreements were reached for V2V [2]:

- Reselection is triggered if any of the following conditions are met

  - A counter meets an expiration condition

    - The counter decrements

      - Working assumption: every TB transmission (can be revisited if a problem is identified with TBs other than every 100 ms)

    - The counter is reset to a value when reselection is triggered for all the semi-persistently selected resources.

      - The value is uniformly randomly selected within a range (which is independent of the TB arrivals) between 5 and 15

- In Step 3,

  - Step 3-0: When the counter reaches zero,

    - With probability p, the UE keeps the current resource and the counter is reset.

    - With probability 1-p, resource is reselected by Step 3-1 and 3-2.

    - Carrier-specific parameter p is (pre)configured in the range [0, 0.2, 0.4, 0.6, 0.8].

The same counter based and probability based mechanisms can be applied at pedestrian UEs to realize random selection with semi-persistent transmission. In addition, the reset counter value should include 1. Then random resource selection with fresh random selection for each pedestrian data traffic period can also be realized with the reset counter value (pre)configured as 1 and probability p (pre)configured as 0.

For limited time sensing based resource selection, a suitable choice of sensing period at pedestrian UEs is 100ms since the majority of data traffic is of 100ms period. A pedestrian UE performs sensing during a limited time (e.g.10ms) every 100ms and then selects transmission resource from the sensed resources.

We performed system-level simulation to compare performance of different resource selection options. The system level simulator is compliant with the evaluation methodology agreed in RAN1 and summarized in [3]. The urban deployment scenario is simulated. The vehicular speed is set to 60 km/hr. The size of data pool is 100 sub-frames in time domain and 42 RBs in frequency domain. Each PSSCH occupies 14 RBs. The number of transmissions for a MAC PDU is 1. The period of vehicular data traffic is 100ms and the period of pedestrian data traffic is 1s. For limited time sensing based resource selection, the sensing duration is 10ms every 100ms. The following figure shows the system level performance in terms of packet reception ratio versus distance.

In the simulations, vehicular UEs perform full time sensing based resource selection. The baseline is that pedestrian UEs also perform full time sensing. Three cases are simulated. In case 1, pedestrian UEs perform random selection with fresh random selection for each pedestrian data traffic period. In case 2, pedestrian UEs perform random selection with semi-persistent transmission (spt). In case 3, pedestrian UEs perform limited time sensing based resource selection. Simulation results show that compared with random selection (case 1), random selection with semi-persistent transmission (case 2) at pedestrian UEs can improve the system level performance of P2V communication. Limited time sensing based resource selection (case 3) can further improve P2V performance at the cost of power consumption. And there are still improvement room for both random selection and limited time sensing based selection to approach the performance baseline.

Fig.1. System level performance for coexistence of V2V and P2V communication

Observation 1: The performance (in terms of PRR) for P-UE using random resource selection with semi-persistent transmission is very close to the performance using limited time sensing, but it benefits from much lower power consumption and allows UE implementation without sidelink reception.

Proposal 1: Consider semi-persistent transmission on top of random resource selection at pedestrian UEs to improve P2V performance.

Proposal 2: Consider the same counter based and probability based mechanisms as V2V at pedestrian UEs to realize random resource selection with semi-persistent transmission.

Proposal 3: Consider 100ms sensing period for limited time sensing based resource selection at pedestrian UEs.

## Congestion control

Congestion control is a mechanism to adapt the transmit behaviors of a transmitting UE based on the congestion level of the transmission (TX) resource pool. Congestion control is necessary to avoid a resource pool overloaded. The congestion level information is a critical input for congestion control. To reduce power consumption, it’s preferred that a pedestrian UE doesn’t receive and only transmits with random resource selection. Hence a pedestrian UE may not be able to estimate the congestion level directly.

In the same area, a vehicular UE can estimate the congestion levels of the resource pools it monitors since it performs sensing based resource selection. In RAN2, it was agreed [4] that mapping of geographical location and resources can be done on a zone concept. Each cell can be partitioned into multiple zones and the eNB broadcasts the zone configuration to the UEs. The zone configuration includes the information about the zones and their associated resource pools. Based on its location and the zone configuration, a UE can determine which zone it belongs to. It’s beneficial that the UE reports the congestion levels of resource pools it monitors to the eNB.

The eNB can adjust resource pool configuration in the cell based on congestion levels of resource pools if there are available resources. The eNB can also broadcast zone indices and congestion levels of resource pools (especially the pool in which pedestrian UEs transmit) in those zones via system information. Pedestrian UEs can then adjust transmission parameters, which include transmission power, number of retransmissions, etc.

Proposal 4: Consider that vehicular UEs report congestion levels of resource pools to the eNB.

Proposal 5: Consider sending LS to RAN2 to check whether the eNB can broadcasts zone indices and congestion levels of resource pools (especially the pool in which pedestrian UEs transmit) in those zones via system information.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Observation 1: The performance (in terms of PRR) for P-UE using random resource selection with semi-persistent transmission is very close to the performance using limited time sensing, but it benefits from much lower power consumption and allows UE implementation without sidelink reception.

Proposal 1: Consider semi-persistent transmission on top of random resource selection at pedestrian UEs to improve P2V performance.

Proposal 2: Consider the same counter based and probability based mechanisms as V2V at pedestrian UEs to realize random resource selection with semi-persistent transmission.

Proposal 3: Consider 100ms sensing period for limited time sensing based resource selection at pedestrian UEs.

Proposal 4: Consider that vehicular UEs report congestion levels of resource pools to the eNB.

Proposal 5: Consider sending LS to RAN2 to check whether the eNB can broadcasts zone indices and congestion levels of resource pools (especially the pool in which pedestrian UEs transmit) in those zones via system information.

# References

[1]	RP-161298, “New WI proposal: LTE-based V2X Services”, LGE, Huawei, CATT.

[2] R1-168217, “SL-V2V agreements after RAN1#86”.

[3] 3GPP TR 36.885.

[4]	3GPP RAN2#93bis Chairman meeting notes.