3GPP TSG-RAN WG1 #86bis	R1-1609964

10th – 14th October 2016

Lisbon, Portugal

Agenda item:	7.2.2.1.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on advanced CSI reporting for eFD-MIMO

Document for:	Discussion/Decision

# Introduction

In RAN1#86 meeting, the following agreement was made for advanced CSI reporting in eFD-MIMO [1].

Agreement:

- Specify CSI feedback enhancement with the following advanced CSI feedback framework:

  - Reduced space (eigenvectors)/W1 is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Orthogonal basis (e.g. orthogonal DFT matrix)

    - Alt2. Non-orthogonal basis (e.g. Rel.13 Class A W1 for rank-1 and/or 2)

  - Reduced space representation/W2 is to further combine selected beams

  - Granularity of weighting(phase and/or amplitude) can be either wideband only or wideband/subband, and is constructed based on one of the following alternatives (TBD RAN1#86bis):

    - Alt1. Phase and amplitude

    - Alt2. Phase-only weighting

  - How the enhanced framework can be applicable for Class A and/or Class B eMIMO-Types is FFS

  - FFS: How to handle the relationship between advanced CSI feedback and legacy CSI feedback framework

- Companies are encouraged to provide results comparing the above alternatives, considering a mix of smaller and larger numbers of ports within the following antenna port configurations

  - {4,8,12,16,20,24,28,32} ports

  - Focus on rank<=2 scenario MU-MIMO for evaluation

  - Feedback overhead needs to be taken into account

  - For {4,8,12,16, 20,24,28,32}-port scenario, companies are encouraged to compare their proposals to dual-stage codebook enhancement with increased number of beams in W1

In this contribution, we further discuss advanced CSI feedback for the structure of the high resolution codebook enhancements using linear combination structure and also feedback enhancements to support enhanced codebook.

# Advanced codebook design

For advanced CSI feedback for eFD-MIMO, a channel compression technique via multiple DFT beam basis has been proposed, aiming at improving system performance at reasonable complexity and overhead. In general, both explicit CSI feedback and implicit CSI feedback can leverage same basis for channel compression. The main controversy lies in whether the basis has to be orthogonal or non-orthogonal beams (such as Rel.13 classA codebook pattern), and whether power quantization needs to be taken into account. In our view, a common framework of codebook structure can be defined for both two alternatives and represented by the following equation.

W(1)=b1b2…bk00b1b2…bkp1⋯⋮⋱⋮⋯p2kθ1,1⋮θ1,2k= B×P×Θ

A rank1 precoder can be generated based on above formulation, which is essentially targeting to reconstruct the dominant eigen vector. The k DFT beams formulate the beam basis as B'=b1b2…bk,B=B'00B'. The power weight of each DFT basis is represented by =p1⋯⋮⋱⋮⋯p2k=Ppol1Ppol2. The X-pol power weighting factor can be the same if Ppol1=Ppol2. Phasing weighting factor combining multiple beams is denoted by Θ=θpol1θpol2,θpol1=θ1,1⋮θ1,k,θpol2=θ1,k+1⋮θ1,2k. If a common phasing for each DFT basis is applied, then Θ can be simplified by Θ=Φ⨂θpol1,Φ=1φ, Φ is the cophasing factor that combines two polarizations.

Rank2 codebook can be generated from extension of rank1 codebook and is denoted by

W(2)=12b1b2…bk00b1b2…bkP1/P1P2/P2θ1,1⋮θ1,2kθ2,1⋮θ2,2k

To keep the orthogonality between two layers, ΘHΘ=I is required. For simplicity, we assume that θ2,m=(-1)m/kθ1,m.

W1 design options

For beam selection in W1 design, different options have been proposed by many contributions in RAN#86. Three major issues of beam selection design are listed below.

- Orthogonal beam basis vs. non-orthogonal beam basis

For orthogonal beam basis, multiple orthogonal beam are selected. To reduce the amount of required beams, beam rotation, or oversampling between orthogonal beams is generally required. Power weight is needed for orthogonal beam, as the power offset between DFT beams may be large. Non-orthogonal beam basis which was proposed in [7] is using a Rel.13 pattern in correspondence with ‘Config’ setting, consisting of 4 beams in W1 and further used in linear combination. No power weighting is considered in non-orthogonal beam selection.

- Unconstraint beam selection vs. structured beam selection pattern

In [5], the beam selection is unconstraint which covers all orthogonal space of DFT beams. Each selected beam is indexed with log2(N1×N2) bits, after the rotation factor is determind. This approach can be regarded as the performance wise optimal in beam selection, at the cost of a larger W1 codebook size. Consider the correlation of DFT beams with the channel, the selected orthogonal beams should be usually adjacent to the dominant DFT beam. A pre-defined beam selection pattern was designed in [6]. The beam selection pattern with the largest probability was traced and fixed in its W1 design, which essentially saves log2(N1×N2)×(k-1) bits compared with optimal beam selection case. The main issue for this mechanism is that different patterns may needs to be defined for different array size, channel type configurations.

Another alternative for the structured orthogonal beam basis is to use Rel.13 class A rank 4 or 8 codebook. For 2 orthogonal beam combining, the rank 4 W1 codebook is used, which consists of four orthogonal beam pairs. For 2 beam combining, a subband beam pair selection may be applied to select one of four orthogonal beam pair for linear combination. Similarly for 4 orthogonal beam combining, the rank 8 W1 codebook is used and the 4 orthogonal beams will be linearly combined by the selected W2 matrix. The benefit of the proposed approach is to reuse the existing Rel-13 codebook as much as possible. For different RRC signaling of ‘Config’, different orthogonal basis vectors can be selected.

- Number of beams for combining

As a common understanding, the performance improves together with number of beams, and approaches a upper bound when all orthogonal beams are utilized. However, the number of beams has larger correlated with total overhead, therefore the number of beams for combining needs to determined based on joint consideration of performance and overhead.

W2 design options

As shown in the codebook structure above, there are two possible structure for both power and phase weighting in W2. For power weighting, it could be either Ppol1=Ppol2(common power weighting) or Ppol1≠Ppol2(different power weighting). For phase weighting, it could be either θpol1=φθpol2(common beam phase weighting and cophasing) or  θpol1≠θpol2(different phase weighting). Since the power weighting reflects the power of the cluster, it generally have wideband and long term characteristic. Therefore we assume wideband feedback of power weighting and subband level feedback of phase weighting. Given a subband number L, k beams basis, power and phase weighting bits of (m,n), cophasing bits of 2, the overhead of power plus phase weighting factors for each codebook structure is calculated as the following table.

Table 1 Overhead calculation of power and phasing weight options

|  | Ppol1=Ppol2 | Ppol1≠Ppol2 |
| --- | --- | --- |
| θpol1=φθpol2 | k×m+k-1×n×L+2L | 2×k×m+k-1×n×L+2L |
| θpol1≠θpol2 | k×m+k-1×(2n-1)×L | 2×k×m+k-1×(2n-1)×L |

We conduct system simulation to verity the performance of different W1 design options. Both Alt. 1 orthogonal and Alt. 2 non-orthogonal beam basis are evaluated, where we assume unquantized Ppol1≠Ppol2 for Alt1, and fixed Ppol1=Ppol2=1 for Alt2.  Unquantized phase with θpol1≠θpol2 is assumed for both alternatives. The baseline is using Rel.13 classA codebook with Config3. Orthogonal beam selection vs. non-orthogonal beam selection

Figure 1 Alt 1 and Alt 2 performance comparison, 4 beams combing

As shown in the system simulation result Alt. 1 with orthogonal basis significantly outperforms Alt. 2 with non-orthogonal basis for both cell edge and mean throughput. Up to 38% cell edge gain can be observed for Alt. 1, while only 12% edge gain is observed for Alt.2. The results show the orthogonal basis vectors shall be used for beam combining.

Observation 1: Beam combining using orthogonal beam basis outperforms the non-orthogonal beam basis.

The performance of different construction of orthogonal beam basis is also evaluated and shown in Figure 2 below. It is observed that the performance from the proposed orthogonal beam basis construction is small. For example, cell edge gain lose for the proposed mechanism is below 5%. The loss is reduced for smaller number of beams. Compared to the unconstrained beam selection, the proposed mechanism using Rel-13 rank 4/8 codebook has the benefit for lower W1 feedback overhead, which also reduce UE complexity.

Figure 2 proposed vs. unconstraint beam selection

Observation 2: The orthogonal beam basis based on Rel-13 Class A rank 4/8 codebook only suffers small performance loss but saving W1 feedback overhead.

The performance of different number of beam combining is shown in Figure 3. We can observe that 5 beam combining provide only marginal gain over 4 beam combining. Similar observation is found between 2 beams and 3 beams. Therefore it can be concluded that either 2 beam sor 4 beams combining is sufficient.

Figure 3 Gain w.r.t. number of beams in unconstraint beam selection

Observations 3: The beam combining based on 2 or 4 beams is sufficient considering performance and overhead tradeoff.

Proposal 1: For W1 design, orthogonal beam basis based on Rel-13 rank 4 or 8 codebook can be considered.

Next, we evaluate the performance of different power and phase weighting structure. The following 4 options are considered.

- Option 1: Ppol1=Ppol2, θpol1≠θpol2

- Option 2: Ppol1≠Ppol2, θpol1≠θpol2

- Option 3: Ppol1=Ppol2, θpol1=φθpol2

- Option 4: Ppol1≠Ppol2, θpol1=φθpol2

For quantization, power and phase quantization of (m,n) = (4,3), (4,2), (3,3), (3,2) are evaluated. Power quantization levels are within [-15.0,0] dB with zero power included, phasing elements are uniformly quantized in a PSK alphabet. Figure 4 summarized the performance of option1~4 for different number of quantization bits.

Figure 4 Gain of W2 options under quantization

It is illustrated from the result that option 2 always provide the best performance of all candidates, however, the codebook size of option 2 is also larger than the other options. The scatter plot of cell edge gain with respect to the overhead increment of the codebook is also plotted in Figure 5. Option 1~4 are individually plotted with a linear trend line for each. We noticed from the trend line that option 2 shows the best performance when the overhead increment is less than 200%. But the performance difference between option 2 and 4 is small when overhead increment is between 100% and 200%. It implies that option 4 could achieve best tradeoff between performance and overhead. From the figure, it can also be seen that the overhead increment between 100% and 200% may be a good operation point for advanced CSI. After the overhead increment of 200% the gain increase is reduced. Therefore it indicates that the number of subband feedback bits including 4-bits CQI for advanced CSI shall be within a range of 8~16 bits.

Figure 5 throughput gain vs. overhead increment @ 32 ports, 3D-UMi, 50% RU

Proposal 2: W2 structure with Ppol1≠Ppol2, θpol1=φθpol2  is preferred .

With the above structure, we also evaluate performance of different number of antenna ports. The results are illustrated in Figure 6. According to the system simulation result, up to 25% cell edge gain are observed for 16 ports. Meanwhile 4 or 8 CSI-RS ports shows less gains than 16 or 32 CSI-RS ports, especially 4 port. This is because advanced CSI is mainly beneficial for higher order MU-MIMO which is more likely to a larger number of antenna ports, such as 16 or 32 CSI-RS ports. The performance gain for 4- and 8-ports will be limited as seen from Figure 6. Therefore advanced CSI reporting design should be prioritized for 16 or 32 CSI-RS ports.

Figure 6 throughput gain @ (4,8,16,32) ports, 3D-UMi, 50% RU

Proposal 3: Prioritize advanced CSI feedback design for 16~32 CSI-RS ports.

# Wideband covariance matrix feedback

In last RAN1 meeting, wideband only CSI feedback is also supported for advanced CSI. For wideband only feedback, one possible is to feedback a wideband W1 and W2 for linear combination. But this is not efficient since the frequency selectivity makes it difficult to find an optimal beam combing to approximate the wideband channel. There could be multiple eigenvectors from the aggregated wideband channel, the dominant eigenvector feedback cannot fully represent each subband channel.

In [8], a wideband channel covariance matrix using the reduced dimension representation was proposed. By combing with hybrid CSI-RS, the performance can be much better than the Rel-13 Class A codebook. There is also a benefit to reduce the CSI-RS and CSI feedback overhead since the measurement and feedback of the wideband channel covariance can be on the long-term basis.

In Figure 7, we shows the performance comparison of advanced W1/W2 codebook based on wideband linear combination and the proposed wideband compressed covariance matrix feedback. 4 beams basis is assumed for both implicit and explicit CSI feedback. Proposed W1 and option 2 type W2 with (4,2) bit quantization is adopted for implicit CSI evaluation. It is observed that cell edge performance gap for implicit vs. compressed covariance matrix CSI feedback can be as large as 6.4%.

Figure 7 wideband CSI feedback performance comparison

Proposal 4: Wideband covariance matrix feedback with reduced dimension representation shall be supported.

# Feedback design for advanced CSI

Higher layer configuration such as RRC signalling could be used to trigger advanced CSI reporting. Nevertheless, such semi-static mechanism fails to track dynamic change of system loading. For example, when system loading is low, SU-MIMO might be good enough with legacy CSI feedback such as Rel.13 classA codebook. The reporting of advanced CSI in such scenario is a waste in both CSI overhead and UE complexity. It is appreciated that eNB can trigger advanced CSI only when the performance can be largely improved by advanced CSI. Dynamic triggering of advanced CSI is therefore beneficial. Layer1 signalling with indicator of advanced CSI feedback, such as ‘CSI request’ of aperiodic CSI triggering can be considered. Semi-persistent triggering of advanced CSI via enabling and disabling commands through L1 signaling could be another option.

Proposal 5: Dynamic triggering of advanced CSI should be supported.

# Conclusions

In this contribution, we discussed and presented the evaluation results for advanced CSI feedback. Based on observations and discussions so far, we have the following proposals.

Proposal 1: For W1 design, orthogonal beam basis based on Rel-13 rank 4 or 8 codebook can be considered.

Proposal 2: W2 structure with Ppol1≠Ppol2, θpol1=φθpol2  is preferred .

Proposal 3: Prioritize advanced CSI feedback design for 16~32 CSI-RS ports.

Proposal 4: Wideband covariance matrix feedback with reduced dimension representation shall be supported.

Proposal 5: Dynamic triggering of advanced CSI should be supported.

# References

- RP-151028 WID_EBF_FD_MIMO

- 3GPP Chairman's Notes RAN1_86

- R1-167971 WF on Advanced CSI feedback framework in Rel-14

- R1-167643 Explicit versus implicit feedback for advanced CSI reporting	Ericsson

- R1-167645 W1 design for implicit advanced CSI reporting	Ericsson

- R1-166322 Discussion on the design of linear combination codebook	ZTE Corporation, ZTE

- R1-166734 Linear combination (LC) codebook based CSI reporting and simulation results	Samsung

- R1-166274Discussion on advanced CSI reporting and initial evaluation results	Qualcomm Incorporated

# Appendix

Table 2 System simulation parameters

| Parameter | Value |
| --- | --- |
| Layout | 57 cells wrap-around |
| Scenario | 3D-UMi ,200m ISD |
| System bandwidth | 10MHz |
| Carrier frequency | 2GHz |
| UE association | RSRP on CRS port0 with 3dB handover margin |
| eNB antenna configuration | 16port: (M,N,P,Q) = (8,4,2,16) 32port: (M,N,P,Q) = (8,4,2,32) |
| Oversampling | (4,4) for 2D port mapping, (4,-) for 1D port mapping |
| UE antenna configuration | 2RX with X-Pol |
| UE mobility | 3km/h |
| Traffic model | FTP Model 1, 500kB packet size |
| Packet arrival rate | λ=(2.8,3.3,4.0,4.2) for (4,8,16,32) ports, respectively |
| MIMO configuration | SU/MU-MIMO adaptive |
| MU-MIMO precoding | SLNR precoding |
| Scheduling algorithm | Proportional fair scheduling |
| Receiver | MMSE-IRC receiver |
| CSI feedback | PUSCH Mode 3-2, 5ms delay |
| Transmission rank | Rank 1, Rank2 adaptive |
| Overhead | CRS, PSS/SSS, DMRS, PCFICH,PDCCH (total 36.3%) |