3GPP TSG-RAN WG1 Meeting #86bis	R1-1610090

Lisbon, Portugal, 10th – 14th October 2016

Agenda Item:	8.1.7.2

Source:	InterDigital Communications

Title:	Scheduling and support for service multiplexing

Document for:	Discussion

# Introduction

NR supports an orthogonal multiple access scheme where the eNB schedules DL and UL transmissions at least for the eMBB case.

In RAN1#86, several potential options for further consideration were identified for the case of eMBB and URLCC service multiplexing. One possibility is semi-static resource sharing between eMBB and URLCC in FDM/TDM. This case for example includes short UL transmission durations such as UL grant-free transmissions for URLCC. Another identified option in the RAN1#86 agreement is dynamic resource sharing which for example includes the case of DL scheduling for a short transmission where the allocated resources can overlap with those of an already ongoing longer transmission at least from network perspective. This case may also include preemption or superposition mechanisms. Another identified possibility is the use of scheduling-based approaches. For example, the transmission durations can be adopted or different subbands can be used to allow service multiplexing of eMBB and URLCC.

In this contribution we discuss DL and UL scheduling and impacts onto NR resource allocation for the case that eMBB and URLCC type of transmissions are multiplexed onto NR carriers.

# FDM/TDM multiplexing for eMBB and URLCC

The two NR use case families, eMBB and URLCC have very different requirements in terms of user plane latency and required coverage levels. One of the most important NR design requirements is the ability to simultaneously support these different services in a scalable and spectrally efficient manner. This design requirement has profound impact in several areas such as the choice of the NR numerology, frame structure, L1 timing, scheduling and HARQ aspects.

For example, URLLC operation will require very low Uu transfer delay latency (<0.5 ms). In order to meet high reliability targets in terms of extremely low residual packet error rates following L1 and L23 processing, the supported link budget may often need to be sacrificed. URLCC will typically result in short bursts of data transmission with the order of 100-200 us in L1. There is therefore only limited opportunity for the number of HARQ re-transmissions per HARQ process. In addition, very tight requirements are imposed onto allowable scheduling delay due to the very much compressed Uu transfer delay timeline.

For the case of eMBB, latency requirements not as stringent as observed for URLCC. Very low latency for packet transfers is mainly beneficial at the initial stage of data transmission in order to avoid that TCP slow start negatively affects the overall user packet delay for during packet transfers. Given the significant amounts of data transferred for an eMBB user, long sequential bursts of high volume data are then often transferred. This results in many cases in a wide instantaneous bandwidth occupation for a scheduled eMBB transmission and in the use of long DL or UL transfer intervals in the order of 0.5-1 ms for an eMBB UE.

In future NR deployments, the network will need to support multiple types of traffic at the same time. There will be eMBB users and dedicated URLCC applications using radio resources.

It can be expected that to some extent, eMBB and URLCC type of transmissions are segregated and can use different NR frequency channels. These may possibly be located on different frequency bands and may likely use different OFDM numerologies (Figure 1). In cases such as when dedicated type of URLCC applications are deployed by an operator, it can be expected that dedicated frequency deployments are at least initially preferred due to a much better control over service quality.

The use of FDM for the purpose of multiplexing different NR traffic types extends to the case of a single shared NR frequency channel with different allocated bandwidth regions located on that NR carrier. Possibly different numerologies are used in the different bandwidth regions. In the example shown in Figure 1, a 20 MHz NR FDD carrier is split into an eMBB and a dedicated URLCC bandwidth region. Already in LTE, a comparable approach is followed by NB-IoT when guard-band or inband type of deployment options are used.

FDM in particular when different dedicated NR frequency channels on different bands are used is not particularly spectrally efficient. This drawback is significant especially in the case where traffic is expected to be sparse and bursty such as for URLLC.

Figure 1: FDM for multiplexing different NR service types

TDM for the purpose of multiplexing different NR traffic types is a second component for the case where eMBB and URLCC devices are simultaneously being serviced on a NR frequency channel (Figure 2).

In the example in Figure 2 where both FDM and TDM are used on an NR frequency channel, the eMBB and URLLC UEs 1-3 can be assigned variable-length TTIs of different duration. If no transmission activity occurs in the bandwidth region allocated to URLCC in Figure 2, eMBB can reclaim transmission resources as long as flexible control channel and DL PSCH assignment protocols are supported in NR.

Figure 2: FDM/TDM for multiplexing different NR service types

Several design consequences from the NR multiplexing example in Figure 2 can be observed. These result from considering the signaling and actual traffic characteristics of the non-eMBB services.

NR will require variable-length DL and UL transmission intervals and support for TDM multiplexing of different users in any given variable-length TTI. This is shown in the case of TTI #n in Figure 2. In LTE with fixed-length 1 ms TTIs, an entire RB would always be allocated to a single user for the duration of the entire TTI (subframe). DL assignments and UL grants are needed which can allocate a given RB in TDM to more than a single UE over the duration of the DL transmission interval. This requires much heavier DL signaling for the RB allocation fields and will require additional time-domain allocation information for a TTI resulting in heavier payload for NR DL control messages. Reclaiming the unused transmission bandwidth of a given bandwidth portion on the NR carrier for eMBB is a non-trivial task. In the case of TTIs #n and #n+3 in Figure 2, the remainder of the DL RBs granted to URLCC UEs 2 and 3 goes unused if the logic of frequency-domain restricted allocations from LTE is used also in NR.

Most importantly, low and medium data rate intermittent traffic generated by many factory and machine-type communication use cases will generally result in short interference bursts. On a given NR frequency channel, i.e. some TTIs will contain both eMBB and URLCC data packets while in many others, there will only be eMBB data carried on DL PSCH. Due to very tight Uu delay budgets, the required scheduling delay for URLLC must be much smaller than what eMBB type of scheduling can afford. If URLLC data arrives while an eMBB transmission just started, the latency requirement of URLLC is significantly more difficult to meet.

It is therefore extremely important that non-eMBB type of traffic can be scheduled only very shortly before the beginning of an actual DL transmission interval.

It is worthwhile to consider that LTE based eNB implementations need to perform several functions before a DL transmission can start [1]. These considerations will equally apply in the context of NR.

Figure 3: eNB processing prior to DL transmission in LTE

eMBB has a much larger allowable Uu delay budget than URLCC, therefore it is affordable that the initial eNB scheduling step for multiplexed eMBB users selects the essential transmission parameters which determine the L1 and front-end processing and memory buffering requirements much earlier, i.e. in the order of 1-1.5 ms prior to the start of the actual DL subframe. URLCC type of transmissions however are small data units with much shorter allowable Uu delay budgets. A scheduling decision to transmit data for a URLCC user in a given DL transmission interval should be possible only in the very “last moment”.

In summary, it is difficult to support service multiplexing of different NR traffic types, i.e. eMBB and URLCC through FDM/TDM on shared NR carriers if the LTE example is followed [1].

NR design should accommodate the possibility of an FDM/TDM multiplexing approach whereby URLCC type of transmissions are scheduled, L23 and L1 processed, then can be inserted into the already processed BB samples of eMBB users who were scheduled earlier for the DL transmission interval.

# Scheduling for eMBB/URLCC and resource allocation

In order to support small payload and bursty data transmissions for URLCC with minimum incurred scheduling delay prior to the begin of their actual transmissions, multiple scheduling opportunities per DL transmission interval must be used. For the example of NR subframes of duration 0.5 ms with 15 kHz numerology, typically 2 or 3 such scheduling opportunities per subframe are needed.

URLCC transmissions do not benefit from the resource allocation flexibility in terms of frequency-domain or subband scheduling provided by LTE. In fact, even URLCC packets of payload several hundred octets will not occupy a significant portion of transmissions bandwidth. Furthermore, frequent channel state feedback will not necessarily be available as is the case for eMBB UEs. In addition, the sporadic nature of transmissions from/to URLCC devices will have as consequence that only very few URLCC devices will need to be scheduled in any given NR subframe.

We therefore think that scheduling for URLCC devices is best supported by means of pre-configuring time/frequency resources for URLCC devices similar to LTE SPS configurations. These pre-configured or “allowed” time/frequency resources will then be dynamically scheduled by the eNB (DL) or used autonomously by the URLCC UEs (UL).

In the DL example in Figure 4, there are 4 such time/frequency radio resource allocation regions, for simplicity called RRAR, configured for URLCC devices in TTIs #n and #n+3. UE 1 receives eMBB data transmissions in each of TTIs #n, #n+1, #n+2, #n+3 and #n+4. In TTI #n+1, despite the configured presence of 4 RRAR, no transmissions to other (URLCC) devices in any of these took place. In TTI #n+3, the eNB schedules a “last moment” transmission for a URLCC UE using RRAR2 and transmits the URLCC data in the corresponding radio resources, i.e. by means of puncturing into the ongoing eMBB data transmission for eMBB UE1 in that TTI.

There are at least two possibilities how to deal with the punctured transmission resources. Either the eMBB UE1 detects the actual URLCC transmission on one of the multiple scheduling opportunities in TTI #n+3, or in the subsequently received TTI #n+4, the eMBB UE1 receives DL control signaling which in addition to the scheduling parameters for the current TTI #n+4 also indicates that the previous transmission received in TTI #n+3 was punctured by another transmission in RRAR2. Given that the number of configured possible URLCC time/frequency allocation regions is known and there are only few, these can be indexed. Signaling using index values for the pre-configured and well-identified RRARs therefore also allows to use fast and robust DCI-based L1 signaling. In the example in Figure 5, only 2 bits would be required to signal the actual use of the URLCC resources in TTI #n+3. The eMBB UE1 can then set the received and decoded soft channels bits for the time/frequency resource elements associated with RRAR2 corresponding to the TB / HARQ process received in TTI #n+3 to zero to avoid buffer corruption during HARQ combining for that HARQ process.

Decoding performance for the eMBB UE1 is impacted only in that available Eb/No for the eMBB transmission is decreased by as much energy required to transmit RRAR2, but no additional penalty is incurred. Similarly, since no semi-static resources were set aside for possible transmissions from/to other devices in the configured RRARs, no system penalty in terms of spectral efficiency is incurred. All available radio resources can be used while preserving full flexibility to schedule delay-sensitive data “last moment” when needed even though other longer-delay transmissions may be concurrently ongoing.

Figure 4: Dynamic scheduling of eMBB and URLCC (UL)

In the UL example in Figure 5, UL parts of TTIs #n+1 and #n+4 are configured for UE autonomous transmissions, one for each URLCC UE 1-4. In the UL portion of TTI #n+1, UEs 1 and 3 have data to transmit, so their transmissions take place in their designated RRARs while UEs 2 and 4 do not transmit. In TTI #n+4, URLCC UEs 1, 3 and 4 have data to transmit, so URLCC UEs 1, 3 and 4 transmit URLCC data in their corresponding RRARs while RRAR 2 is unused.

The eNB can use blind detection on the RS or data symbols in a RRAR region for the configured and known possible transmission instances of URLCC devices.

Figure 5: UE-autonomous transmissions for URLCC (UL)

# Conclusion

In this contribution we discuss DL and UL scheduling and impacts onto NR resource allocation for the case that eMBB and URLCC type of transmissions are multiplexed onto NR carriers.

In our view, it is difficult to support service multiplexing of different NR traffic types, i.e. eMBB and URLCC through FDM/TDM on shared NR carriers if the LTE example if followed [1]. NR design should accommodate the possibility of an FDM/TDM multiplexing approach whereby short URLCC type of transmissions are scheduled, L23 and L1 processed, then can be inserted into the already processed BB samples of eMBB users who were scheduled earlier and for which much longer transmissions are ongoing.

We think that scheduling for URLCC devices is best supported by means of pre-configuring time/frequency resources for URLCC devices similar to LTE SPS configurations. These pre-configured or “allowed” time/frequency resources will then be dynamically scheduled by the eNB in the DL or will be used autonomously by the URLCC UEs in the UL.

In summary we propose,

Proposal 1:

Pre-configured SPS-like time/frequency resources for URLCC DL and UL transmissions are supported in NR.

Proposal 2:

URLCC DL transmissions puncture eMBB transmissions with signaling per scheduling opportunity to indicate which pre-configured time/frequency resources puncture an ongoing longer transmission.

Proposal 3:

URLCC UL transmissions are supported through eNB-side blind detection for URLCC UE transmission activity on pre-configured time/frequency resources.

# References

- R1-167331		Efficient multiplexing of traffic for different use cases; InterDigital

- R1-1610023		Frame structure for NR; InterDigital