3GPP TSG RAN WG1 Meeting #86b                                            R1-1610164

Lisbon, Portugal, October 10-14, 2016

Agenda Item:	 8.1.5.2

Source:	Qualcomm Incorporated

Title:	Multi-beam RACH design and evaluation

Document for:	Discussion

# Introduction

RAN1 agreed on evaluation assumptions for RACH during the most recent meeting [1]. This paper shows our RACH design and some results.

# Beam Sweep during Synchronization and RACH

Figure 1: Beam Sweep during Synchronization and Random Access Period

As discussed in [2], the base station can take two approaches, during the synchronization and random access period, to compensate for the high free space path loss and additional non-line-of-sight losses. First, the DL synchronization signals can be transmitted in a beamformed manner and swept through the angular coverage region (in azimuth and/or elevation). Second, the synchronization signal can be transmitted for a longer duration with a pseudo-omni beam. Beam sweep by the base station during synchronization and random access period has several advantages over pseudo-omni beam transmission or reception with longer duration. For example, the transmission of message 2 of the random access process, must be beamformed in order to meet the link budget, hence the base station and the UE need to learn the best beam pair for tx/rx during the synchronization/message-1 period. Other advantages of beamformed synchronization and random access signal were discussed in [2].

Proposal 1: The design should allow base stations to sweep beams during synchronization and random access period.

# Use of Reciprocity between Synchronization and RACH

As mentioned in previous section, the BS needs to know a “good” DL synchronization beam to meet the link budget of Message 2. If base station sweeps during both synchronization and random access period, UE could convey the beam ID of “good” DL synchronization beams to the BS in two ways.

First, the UE could transmit RACH during the entire period of the RACH subframe and explicitly convey the ID of the good DL synchronization beams. The base station does not need to use similar DL sync transmit beams and UL RACH receiver beams in this procedure. However, this process increases the possibility of interference in the system because each UE transmits for the entire duration of the subframe and may interfere other UEs transmissions.

In the other option, each UE could select the transmission time of RACH based on the symbol index of the DL synchronization subframe where it received “good” DL synchronization beam. The base station can find the symbol containing a “good” DL synchronization beam based on the transmission time of random access signal. In this procedure, the base station needs to use similar beams while transmitting DL synchronization signals and receiving UL random access signals. However, in this procedure, the random access transmission of two UEs cannot interfere with each other if their selection of “good” DL synchronization beam is not same. Besides, the use of cyclic shifts and different subcarrier region can provide additional user multiplexing capacity in RACH usbframe.

Our contribution in [3] shows that the beam strength estimation of a particular beam may remain valid for  40 ms duration at pedestrian velocity. Besides, the array gain drops by only a small amount with a reasonable amplitude error and 3-5 bit phase shifters.

Proposal 2: Base station should use beam reciprocity to the extent possible between transmitting DL synchronization signals and receiving UL random access signals.

Proposal 3: UE can select a subset of symbols of the random access subframe based on the signal strengths of DL synchronization beams to transmit RACH signal.

# RACH Subframe in Overall Frame Structure

In the time domain, some resources must be periodically allocated for the transmission of random access signals. For example, we can allocate 625 micro second worth of RACH resources in every 20 ms duration.

Figure 2: RACH Subframe in Overall Frame Structure

The UE could sweep its receiver beams and subarrays while receiving synchronization signals to find the best subarray and UL transmit beam combination before transmitting random access signals. This would allow the UE to meet the link budget for random access transmission.

Observation 1: The UE may sweep its receiver beams and subarrays while receiving synchronization signals to find the best subarray and UL transmit beam combination before transmitting random access signals.

The RACH subframe should contain multiple symbols where the base station could sweep different beams to cover a sufficiently large angular region in both azimuth and elevation. In our design, we propose to allocate 14 symbols in the RACH subframe where the base station could sweep in multiple directions.

Figure 3: RACH subframe structure

Proposal 4: In every X msec, Y msec worth of resources are allocated for the transmission of a random access burst. Each random access burst consists of N nominal random access symbols. The exact values of X, Y and N are FFS (example: X=20, Y=0.625, N=14). The values of X, Y and N should be known to the UE.

# Resource Selection in RACH Subframe

Figure 4: Time-Frequency Grid of RACH Subframe

As mentioned before, RACH subframe could contain several symbols where the base station would sweep its beams and the UE could select one out of these symbols to transmit RACH signal based on a “good” DL synchronization beam. In order to achieve this, the UE needs to know the symbol indices so that it could map best DL synchronization beams or symbols to RACH transmission time.

Proposal 5: BS needs to convey the OFDM symbol index of the transmission while transmitting synchronization signals.

In the frequency domain, there could be multiple subcarrier regions where the UEs would transmit RACH signal. This will allow further frequency domain multiplexing among UEs. The summation of the bandwidth of all subcarrier regions should be limited to the occupied bandwidth of the synchronization signal. The exact width of each subcarrier region and the number of subcarrier region can be determined based on the amount of timing error that the network can handle. For example, in one possible design, synchronization signal can occupy a bandwidth of 40 MHz. Each subcarrier region could be 4.32 MHz with 30 kHz tone spacing and there could be nine subcarrier regions. UE could select one out of nine subcarrier regions randomly to transmit RACH signal.

# Evaluation Agreements

During the recent RAN1 meeting, following evaluation parameters were agreed for link level simulations of RACH signals:

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30, 70 GHz |
| Channel Model | CDL-A (other CDL models are not precluded), AWGN - with delay scaling values of 100 ns for 4 GHz, 30 ns for 30/70 GHz - with combination of ASA and ASD scaling values in sec. 7.7.5.1 in 38.900, for above 6 GHz cases |  |
| Antenna Configuration at the TRP | (1,1,2) with omni-directional antenna element | (4,8,2), with directional antenna element (HPBW=650, directivity 8dB) |
| Antenna Configuration at the UE | (1,1,2) with omni-directional antenna element | (2,4,2), with directional antenna element (HPBW=900, directivity 5dB) |
| Antenna port virtualization | Clarified by each proponent in simulation assumptions (e.g. the beamforming method, beam directions, number of beams) |  |
| Frequency Offset | - +/- 0.05 ppm at TRP , +/-0.1 ppm at UE |  |
| UE speed | - 3km/h, 120km/h - Other values are not precluded | - 3km/h - Other values are not precluded |
| UE movement modeling | - FFS |  |
| Initial timing Offset | - Timing uncertainty derived from cell radius - (e.g. for below 6 GHz, [-10us, +10us] for 3km cell radius) - Companies report the assumed cell radius |  |

Following performance metrics were agreed:

- Detection error probability [TS36.104]

  - Detecting different preamble than the one that was sent

  - Not detecting a preamble at all

    - Threshold is set such that the false alarm probability does not exceed 0.1%

  - Correct preamble detection but with the wrong timing estimation

    - Threshold to judge wrong timing estimation is FFS

    - Definition of timing error is FFS

- Maximum coupling loss (MCL)

  - Required SINR is the one for achieving detection error  probability of 1%

- Other metrics (e.g., PAPR, occupied time, and multiplexing capacity) are not precluded.

Next two sections show our evaluation assumptions and some initial performance results for RACH results at 30 and 70 GHz.

# Evaluation Assumptions

We assume that both BS and UE have one subarrays with the defined number of antennas. The subarrays of the base station and UE are oriented towards each other. For example, it can be assumed that the base station’s subarray is oriented towards 0 degree in the azimuth and the UE’s subarray is oriented towards 180 degree azimuth in the global plane.

The different parameters that we used to simulate RACH results are given below:

| Parameter | Value |
| --- | --- |
| Number of beams at BS | 14 |
| Number of beams at UE | 1 (pseudo-omni beam) |
| Difference between Sync and RACH transmission | 20 |
| Sampling rate | 122.8 Msps |
| Tone spacing | 30 kHz |
| Symbol duration | 33.33 us |
| Cyclic prefix | 8.03 us |
| Supportable cell radius | 1.15 km |
| RACH sequence length | ZC sequence of length 139 |
| Number of subcarrier regions | 9 |
| Allowed cyclic shifts | 4 |
| Max. number of supportable UEs in a given RACH symbol | 36 |
| Max. number of supportable UEs in RACH subframe | 504 |
| AoD spread | 5 |
| AoA spread | 30 |
| ZoA spread | 10 |
| ZoD spread | 3 |

# Initial Performance Results

Observation 2: For CDL-C models at 30 GHz, our design meets 0.1% false alarm and 1% probability of missed detection at -19.5 dB pre-beamforming per tone SNR.

Observation 3: For CDL-D models at 70 GHz, our design meets 0.1% false alarm and 1% probability of missed detection at -26 dB pre-beamforming per tone SNR.

Observation 4: The timing error is distributed between -10 to +10 sample shifts.

Note that, the mentioned per tone SNR of -19.5 dB and -26 dB include the antenna element gain of the transmitter and receiver but do not include beamforming gain.

We also observe that much better results are achieved with 70GHz and CDL-D channel compared to the 30GHz system and CDL-C channel. One of the reason of CDL-D’s superior performance lies in the pseudo-omni beam pattern that is used at the UE. The gain of the beam pattern is not uniform across different angles of arrival (maximized at 0 degree). Since we have fixed the orientation of the UE and BS, the LOS cluster of CDL-D channel model is received with the highest rx beam gain of the pseudo-omni pattern used by the UE.

# Conclusions

Observation 1: The UE could sweep its receiver beams and subarrays while receiving synchronization signals to find the best subarray and UL transmit beam combination before transmitting random access signals.

Observation 2: For CDL-C models at 30 GHz, our design meets 0.1% false alarm and 1% probability of missed detection at -19.5 dB pre-beamforming per tone SNR.

Observation 3: For CDL-D models at 70 GHz, our design meets 0.1% false alarm and 1% probability of missed detection at -26.5 dB pre-beamforming per tone SNR.

Observation 4: For both CDL-C and CDL-D models, the timing error of our design is distributed between -10 to +10 sample shifts.

Proposal 1: The design should allow base stations to sweep beams during synchronization and random access period.

Proposal 2: Base station should use beam reciprocity to the extent possible between transmitting DL synchronization signals and receiving UL random access signals.

Proposal 3: UE can select a subset of symbols of the random access subframe based on the signal strengths of DL synchronization beams to transmit RACH signal.

Proposal 4: Every X msec, Y msec worth of resources are allocated for the transmission of a random access burst. Each random access burst consists of N nominal random access symbols. The exact values of X, Y and N are FFS (example: X=20, Y=0.625, N=14). The values of X, Y and N should be known to the UE.

Proposal 5: BS needs to convey the OFDM symbol index of the transmission while transmitting synchronization signals.

# References

# R1-168390, “Way forward on link level evaluation assumptions for RACH Preamble”. 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden 22nd – 26th August 2016.

# R1-166384, “Initial access in millimeter wave systems”, Qualcomm Inc. 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden 22nd – 26th August 2016.

# R1-161050, “Effect of Calibration in Beam Gain and Beam Coherence Time”, 3GPP TSG RAN WG1 Meeting #86b, Lisbon, Portugal 10th – 14th October 2016.