# 3GPP TSG RAN WG1 Meeting #86bis	R1-1610303

# Lisbon, Portugal

# 10th - 14th October 2016

Source:                    	CEWiT

Title:  	Evaluations on DMRS based open and semi-open loop MIMO schemes

Document for:        	Discussion

Agenda Item:         	7.2.2.2

# Introduction

In 3GPP RAN #71 meeting, enhancements on FD-MIMO for LTE was agreed [1] for Release 14. One of the key objectives is to study DMRS based open or semi open loop enhancement schemes with existing number of CSI-RS ports as well as the newly supported number of CSI-RS ports. Existing LTE diversity schemes like LD-CDD, SFBC are designed to support up to 4 CRS ports that cannot be directly extend for larger number of TXRUs. Hence to support high mobility users and to minimize high periodic feedback of PMIs for multiple CSI-RS ports, it is necessary to study DMRS based schemes which can exploit transmit diversity which is tolerant to channel variations at high speed.

Further in RAN1 #86 [2], it is agreed to:

Agreement:

- Support both CLASS A and CLASS B with K≥1 CSI-RS resources for the eMIMO-Type of semi-open-loop

  - For dual-stage codebook,

    - RI, i1 and CQI reporting is supported for {CLASS A} and {CLASS B with K=1 and 8Tx/alternate 4Tx codebooks}

      - FFS: reporting i11 and/or i12 for Class A codebooks

    - CRI, RI, i1 and CQI reporting is supported for CLASS B with K>1

    - FFS: reporting i1 and subsampled i2

    - FFS: Hybrid CSI with semi-open-loop

    - FFS: Reporting multiple CRIs for CLASS B K>1

  - For single-stage codebook with 2 and 4 ports (CLASS B with K>1), CRI, RI and CQI are reported

  - For single-stage codebook with 2 and 4 ports (CLASS B with K=1), RI and CQI are reported

- Support RE level transmitter operation on PDSCH for both CSI reporting and PDSCH transmission

  - Down selection from the following transmission schemes in RAN1#86bis, including possible combination

    - Precoder cycling

    - Tx diversity

    - LD-CDD

    - Layer Permutation

Conclusion:

- RAN1#86bis: companies need to justify the need for introducing new DMRS pattern(s) to support open-loop/semi-open-loop transmission scheme. Otherwise the proposal in R1-168048 is agreed.

This contribution compares semi-open loop scheme with closed loop scheme, other open loop schemes such as LD-CDD, SFBC and layer permutation for ranks greater than 1. Semi-open loop scheme is implemented using partial wideband precoding matrix indicator (I1) reporting by the UE and I2 precoder cycling over the resource blocks. Additionally, a new CQI computation mechanism and reporting scheme for the semi-open loop scheme is also proposed.

# Discussions on open/semi-open loop scheme

Closed loop transmission schemes requires high UE complexity and high feedback overhead, it is useful to increase the performance for the low or fixed UEs. However, the UEs with high speed, the PMI and the corresponding CQI fed back at a given subframe will not be valid after few subframes and hence needed more aggressive feedback updates. The CQI mismatch can lead to more transport block errors and hence higher number of HARQ retransmissions, resulting in reduced average cell throughput due to increase of CQI offset in the outer-loop link adaptation. Hence the eNB needs to reduce the dependency of the stale precoder and it’s not worth for frequent feedback reporting, high overhead and considering UE complexity. To get the average and robust performance in the high Doppler scenario, we need open/semi open loop schemes for FD-MIMO.

With the current DMRS based transmission schemes especially in TM9 and TM10, proprietary resource block level precoder cycling schemes can be implemented. In TDD, implementing this scheme could be beneficial by applying reciprocity principle. However, it has several issues like less diversity gain compared to subcarrier level cycling, deriving diversity CQI from the reported CQI etc.

To address the above issues, there can be the following alternatives. First alternate could be cyclically re-use multiple precoders over the scheduled resource blocks with more accurate CQI measurement support. An eNB would apply precoding to PDSCH transparent to a UE, which gives the opportunity to deploy open loop MU-MIMO schemes as well. The second alternative could be subcarrier level precoders cycling similar to TM3, with the existing DMRS or an extended DMRS. This will average the precoder errors for that subframe and hence expected to provide stable CQI for higher mobility with lower feedback overhead.

The semi-open loop MIMO scheme is a better candidate than open loop schemes provided there is a limited feedback on I1. In 60 kmph and 120 kmph UE velocity, the reported I1 preocder is stable. Hence, eNB use the I1 precoder and the subset of the I2 precoders from the codebook are cycled over the scheduled resource blocks. This scheme can be implemented proprietarily in TM9/TM10. However, the current CQI measurement and reporting mechanism in TM9/TM10 result in mismatch of CQI between the reported and actual CQI seen by the UE and it limits the performance. Hence, there is a scope for improvement in CQI measurement for this scheme. We propose to compute the CQI by considering the reported I1 and I2 precoder cycling over the resource blocks.

# Evaluations semi-open loop scheme

Fig. 1: PDF of ∆I1 in 10 ms periodicity

We studied the stability of the wideband long term precoding indicator (I1). The Fig. 1 shows the PDF of ∆I1 which is the difference between the successive I1 reports. From the Fig. 1, it is noted that the reported I1 is not varied much over the time, and hence we can use I1 feedback with sufficiently longer periodicity (in the order of 10 or 20 ms) and I2 can be cycled over the resource blocks. Furthermore, in RAN1 #86, RI, i1 and CQI reporting is agreed for {CLASS A}.

Fig. 2: Performance of closed-loop and semi-open loop schemes

We evaluated the performance of closed loop and semi-open loop for different CQI periodicity (2 ms, 5 ms, 10ms and 20 ms) with UE speed of 120 kmph in Fig. 1. These simulations are done with 8Tx antennas and other detailed simulation assumptions are placed in the appendix. In semi-open loop scheme, we used only wideband I1 feedback and the corresponding wideband CQI.  Here, wideband I1 and wideband CQI selection is assumed I2 precoder cycling. In closed loop scheme, we used wideband I1, I2 and wideband CQI feedback. The results show that the performance of semi-open loop is more robust compared to closed loop with reduced feedback overhead. From 10 ms PMI and CQI periodicity onwards, the average spectral efficiency for semi-open loop scheme is higher than closed loop schemes. Similar trends also observed in cell-edge spectral efficiency from 5 ms periodicity onwards. Furthermore, the feedback overhead is very less considering only I1 feedback and less frequent reporting. In this case, the semi-open loop scheme is more robust and save lot of feedback overhead.

Fig 3: Performance of Semi-open loop schemes over LD-CDD and SFBC

We compared the performance of semi-open loop scheme in 4Tx alternate codebook with LD-CDD and SFBC for CQI periodicity of 10 ms. For SFBC, even number of RBs were scheduled to make sure that pairing subcarriers always available in every OFDM symbol. The results show that, semi-open loop scheme is outperforming other open loop schemes. Hence, we propose to adopt the precoder cycling scheme with limited feedback including RI, I1 and CQI.

Proposal 1: Proposing to adopt DMRS based precoder cycling scheme with limited feedback (RI, I1 and CQI)

Proposal 2: Proposing to consider DMRS based open loop MU-MIMO scheme.

Proposal 3: Proposing a CQI measurement scheme at UE assuming I2 precoder cycling with I1 for the semi-open loop precoder cycling scheme

# Conclusions

Base on the discussions above, we have the following proposals.

Proposal 1: Proposing to adopt DMRS based precoder cycling scheme with limited feedback (RI, I1 and CQI)

Proposal 2: Proposing to consider DMRS based open loop MU-MIMO scheme.

Proposal 3: Proposing a CQI measurement scheme at UE assuming I2 precoder cycling with I1 for the semi-open loop precoder cycling scheme

# References

- RP-160623, New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE, Samsung.

- RAN1#86, Chairman notes

- R1-167807, Evaluations on DMRS based Semi-open loop MIMO schemes

# Appendix – Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenarios | 3D-UMa ISD 500m |
| Polarized antenna modeling | Model -2 from 36.873 |
| Traffic model | Full buffer |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| Metrics | Mean, 5% UPT |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 2GHz |
| Network synchronization | Synchronized |
| UE Speed | 120km/h |
| UE distribution | Randomly and uniformly distributed over area. 100% of users outdoors in vehicles |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 assumptions |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Feedback | Baseline: PUCCH 1-1 for the closed-loop Semi-open loop: PUCCH 1-1 with only I1 |
|  | Implicit CSI feedback (CQI, PMI and RI) triggered per 2 ms, 5ms ,10 ms and 20 ms |
|  | Feedback delay is 5 ms |
| Overhead | 3 symbols for DL CCHs and 2 CRS ports DMRS overhead - 12 REs for both closed-loop and semi-open loop |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| CSI-RS periodicity | 5, 10 and 20 msec |