3GPP TSG RAN WG1 Meeting #87	R1-1611191

Reno, USA 14th - 18th November 2016

Agenda Item:	6.2.1.1.1

Source:	Huawei, HiSilicon

Title:	Remaining issues for SPS DCI design

Document for:	Discussion and decision

# Introduction

At the RAN1 #86bis meeting, the details of DCI format for SL SPS were discussed, and most of the content was taken as working assumption/agreement:

- The following two fields are additionally included in the existing dynamic scheduling DCI (i.e., DCI 5A).

  - Working assumption:

    - SL SPS configuration index : 3 bits

    - Activation/release indication : 1 bit

- Each SL SPS DCI contains a single SPS configuration index.

- If the number of information bits in SL SPS DCI format mapped onto a given search space is less than the payload size of DCI format 0 mapped onto the same search space, zeros shall be appended to SL SPS DCI format until the payload size equals that of DCI format 0 including any padding bits appended to DCI format 0.

  - FFS whether the size of SL SPS DCI format can be larger than the size of DCI format 0 on the same search space.

Similarly, decisions were taken for the UL SPS DCI design:

- Working assumption: A V2X UL SPS RNTI different from the legacy SPS C-RNTI is defined.

- The following existing field in DCI 0 (for the legacy UL SPS activation/release) is reused to indicate V2X UL SPS configuration index.

  - Working assumption: Cyclic shift DM RS (3bits)

    - TPC command (2 bits) can be considered as an alternative if this working assumption is not confirmed.

- Each UL SPS DCI contains a single SPS configuration index.

In this contribution, we discuss the remaining issues of DCI design for both the sidelink and uplink, and propose to confirm the working assumptions. The details of SL SPS and UL SPS procedures are discussed in the accompanied contribution [1].

# DCI design for SL SPS

Based on the working assumptions DCI format 5A is as described in Table 1 as below.

Table 1: Fields for SPS scheduling grant

| Fields | Bits | Descriptions |
| --- | --- | --- |
| CIF | 3 | To support cross-carrier scheduling |
| Lowest index of subchannel | 0-5 | Lowest index of the subchannel allocated to the initial transmission |
| Time gap | 4 | Time gap between initial transmission and retransmission |
| Frequency resource location | 0-8 | Frequency resource location of the initial transmission and retransmission |
| SPS index | 3 | Differentiate multiple SPS processes |
| Activation/Release | 1 | Activation or release |
| Total | 24 |  |

The payload size of the new DCI format for SL SPS for all system bandwidths and several typical subchannel values is calculated in Table 2. The size of DCI format 5A is always less that the size of DCI format 0 in CSS. Thus, we propose to confirm the working assumption.

Proposal 1: Confirm the working assumption for the 3 bit SL SPS configuration index and 1 bit Activation/release indication fields for SL SPS DCI format.

Table 2: Comparison of size for format 0 and format 5A with the SPS fields

| Size of field | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Number of subchannels | 1 | 3 | 5 | 10 | 15 | 20 |
| Size of subchannel independent fields (includes CIF, Time Gap, SPS index, Activation/Release) | # 11 | # 11 | # 11 | # 11 | # 11 | # 11 |
| Lowest index of subchannel | 0 | 2 | 3 | 4 | 4 | 5 |
| Frequency resource location | 0 | 3 | 4 | 6 | 7 | 8 |
| Total number of bits | # 11 | # 16 | 18 | 21 | 22 | 24 |
| DCI 0 for FDD (in CSS) | 19 | 21 | 23 | 25 | 26 | 27 |

# DCI design for UL SPS

The existing fields in DCI format 0 are listed in Table 3.

Table 3. Existing DCI format 0.

| Field | Bits | Description |
| --- | --- | --- |
| CIF | 0 or 3 | Carrier indicator |
| Format 0 / format 1A differentiation | 1 | Uplink grant or downlink assignment (Flag for format0/format1A differentiation) |
| Hopping flag | 1 | Frequency hopping on/off |
| RB assignment | 5-13 | Frequency resource allocation |
| MCS | 5 |  |
| New Data Indicator | 1 | Toggled for each new transport block |
| TPC | 2 | Power control of PUSCH |
| Cyclic shift for DMRS | 3 |  |
| UL index (TDD only) | 0 or 2 | Used to indicate which uplink subframe(s) the grant is valid for. Only necessary for TDD. |
| DAI (TDD only) | 0 or 2 | Used to indicate how many assignments the UE should have received. Only necessary for TDD |
| CQI request | 1-3 | Aperiodic CQI triggering |
| SRS request | 0 or 1 | Aperiodic SRS triggering |
| RA type | 0 or 1 |  |
| Total without padding | 19-34 |  |

The fields TPC and CS for DMRS are not needed for V2X. Thus, we propose to confirm the working assumption taken at RAN1#86b:

Proposal 2: Confirm the working assumption to reuse the 3 bit Cyclic shift DM RS field to indicate V2X UL SPS configuration index.

SPS RNTI for CRC scrambling

The UL SPS RNTI for V2X should be different from legacy SPS C-RNTI in order to distinguish the existing DCI for dynamic scheduling. Thus, the working assumption can be confirmed:

Proposal 3: Confirm the working assumption using V2X UL SPS RNTI different from the legacy SPS C-RNTI.

# Conclusion

Based on the above discussion, we propose that:

Proposal 1: Confirm the working assumption for the 3 bit SL SPS configuration index and 1 bit Activation/release indication fields for SL SPS DCI format.

Proposal 2: Confirm the working assumption to reuse the 3 bit Cyclic shift DM RS field to indicate V2X UL SPS configuration index.

Proposal 3: Confirm the working assumption using V2X UL SPS RNTI different from the legacy SPS C-RNTI.

# References

R1-1611135, “Remaining issues for SPS procedures”, Huawei, HiSilicon, RAN1#87, Nov. 10-14, 2016.