3GPP TSG RAN WG1 Meeting #87	R1-1611247

Reno, Nevada, USA, November 14 - 18, 2016

Agenda Item:	7.1.3.4

Source:	Huawei, HiSilicon

Title:	Discussion on MIMO calibration

Document for:	Discussion and decision

Introduction

At RAN1#86, it was agreed that

- For the purpose of calibration, companies are encouraged to provide baseline results for NR MIMO in the following meetings, including link-level and system level simulations

  - Discuss further on baseline simulation assumptions and metrics for calibration in email discussion after RAN1#86

Further, in a follow up email discussion [86-20] MIMO calibration for NR, it was agreed that the phased approach will be used for calibration, which include 3 phases:

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

- Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

- Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

Further, at RAN1#86bis, it was agreed that

- Companies are encouraged to provide/update results for Phase 1 NR MIMO calibration in RAN1#87, including link level and system level simulations

  - Extend the email discussion for further clarification on simulation assumptions and metrics, if needed

- Including discussions on simulation assumptions and metrics for  Phase 2 NR MIMO calibration, including link level and system level simulations

And in the follow up email discussion [86b-19] NR MIMO calibration, simulation assumptions and metrics for Phase 2 NR MIMO calibration is being discussed.

For phase 2, the baseline of MIMO evaluation will be set up, and be used for performance comparison for any enhanced NR MIMO features. This contribution discusses the Phase 2 simulation assumptions.

Purpose of phase 2 calibration

After phase 1 calibration, which aims to align the companies’ implementation on channel model, basic beamforming behaviors, network layout and user distributions, phase 2 calibration needs to focus on the discussion of baseline feature and related assumption that will be used for performance comparison for any of the enhanced NR MIMO features. After setting up the baseline features and simulation assumptions, companies could provide simulation results of the baseline features, as well as the simulation results of their proposed NR MIMO features. By this means, it could facilitate the understanding of each company’s proposal, and the performance comparison of different proposals.

Therefore, the purpose of phase 2 calibration should focus on the discussion of baseline feature that will be used for performance comparison with enhanced NR MIMO features, and the simulation assumptions related to baseline features.

Proposal 1: Phase 2 calibration should focus on the alignment of MIMO baseline features and performance that will be used for performance comparison with enhanced NR MIMO features.

Proposal on phase 2 simulation assumptions

For the above purpose, baseline MIMO features and its related simulation assumptions are discussed in this section. It would be good to employ mature features that already exist in 3GPP specification as baseline. Therefore, it is proposed to use LTE Rel-13 MIMO features as baseline.

Proposal 2: For NR MIMO performance comparison, LTE Rel-13 MIMO features listed in Table 1 should be used as baseline.

Table 1 List of key Rel-13 features for MIMO

| Key Features | Description |
| --- | --- |
| CSI-RS enhancement | 16 CSI-RS resources are available |
| DMRS enhancement | 4 orthogonal DMRS ports with OCC=4 |
| SRS capacity enhancement | UpPTS symbols increasing |
|  | RPF=4 |
| Class A reporting (non-precoded CSI-RS) | 2D codebook for 16 ports for Class A reporting |
| Class B reporting (beamformed CSI-RS) | For K>1 CSI-RS resources, CRI feedback is available |
|  | For K=1 CSI-RS resource, ports selection codebook is available |
| Non-PMI based CSI reporting | Non-PMI feedback but with RI and CQI |
| Measurement restriction (MR) | Introduce channel MR for TM9 Introduce channel and interference MR for TM10 |
| New CSI process | Support multiple CSI-RS resources in a process |
|  | Support multiple CSI-IM in a process |

The simulation assumptions for system-level evaluation are listed in Table 2~Table 4.

Table 2 System-level simulation assumption for baseline MIMO feature (for phase 2)

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Indoor hotspot | Dense urban | Urban macro |
| Layout | Follow Table A.2.1-1 in [1] |  |  |
| Inter-BS distance | ISD=20m (12 TRPs) | Follow Table A.2.1-1 in [1] Macro layer 200m | Follow Table A.2.1-1 in [1] 500m |
| Carrier frequency | Follow Table A.2.1-1 in [1] |  |  |
| Simulation bandwidth | Follow Table A.2.1-1 in [1] |  |  |
| Channel model | Follow Table A.2.1-1 in [1] |  |  |
| BS Tx power | Follow Table A.2.1-1 in [1] |  |  |
| UE Tx power | Follow Table A.2.1-1 in [1] |  |  |
| BS antenna configuration | Baseline: - 4GHz: (M,N,P,Mg,Ng) = (4, 4, 2, 1, 1), (dH,dV) = (0.5, 0.5)λ. - 30GHz: (M,N,P,Mg,Ng) = (4, 8, 2, 1, 1), (dH,dV) = (0.5, 0.5)λ. - 70GHz: (M,N,P,Mg,Ng) = (8, 16, 2, 1, 1), (dH,dV) = (0.5, 0.5)λ. | Follow Table A.2.1-4 in [1] Baseline: - 4GHz: (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8) - 30GHz: (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ. - 70GHz: (M,N,P,Mg,Ng) = (8,16,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (8.0, 4.0) λ. |  |
| BS antenna height | Follow Table A.2.1-1 in [1] |  |  |
| BS antenna element gain + connector loss | For below 6GHz: 0dBi For above 6GHz: 5dBi | Follow Table A.2.1-1 in [1] For below 6GHz: 8dBi For above 6GHz: 8dBi |  |
| BS antenna element pattern | For below 6GHz: Omni-directional For above 6GHz: Boresight direction is perpendicular to the ceiling. 90 degree HPBW in Azimuth and zenith | For below 6GHz: Follow TR36.873 For above 6GHz: Follow Table A.2.1-6 in [1] |  |
| BS receiver noise figure | Follow Table A.2.1-1 in [1] |  |  |
| BS receiver type | MMSE-IRC |  |  |
| BS side TXRU mapping | For below 6 GHz: Follow TR36.897; TXRU virtualization only in the vertical dimension; 16 / 32 / 64 / 128 TXRUs can be considered. For above 6 GHz: A single TXRU is mapped per panel per polarization |  |  |
| UE antenna configuration | - 4 GHz: (M,N,P,Mg,Ng) = (1,1,2,1,1); Omni-directional antenna with polarization angles 0 and 90 deg. - 30 GHz: (M,N,P,Mg,Ng) = (2, 4, 2, 1, 2); (dH,dV) = (0.5, 0.5)λ. Inter-panel distance is 6.0λ. (inter-panel distance is defined as the distance of the two parallel panels, see Figure 2 of [2]) Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90deg - 70 GHz: (M,N,P,Mg,Ng)= (4, 4, 2, 1, 2); (dH,dV) = (0.5, 0.5)λ. Inter-panel distance is 6.0λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90deg |  |  |
| UE antenna height | Follow Table A.2.1-1 in [1] |  |  |
| UE antenna gain | Follow Table A.2.1-1 in [1] |  |  |
| UE antenna element gain | For below 6GHz: Omni-directional For above 6GHz: Follow Table A.2.1-8 in [1] |  |  |
| UE receiver noise figure | Follow Table A.2.1-1 in [1] Below 6GHz: 9 dB; Above 6GHz: 13dB (baseline), 10dB (high performance) |  |  |
| UE receiver type | MMSE-IRC |  |  |
| UE side TXRU mapping | A single TXRU is mapped per panel per polarization. |  |  |
| Inter-panel calibration | Calibrated or non-calibrated |  |  |

Table 3 Additional parameter for below 6GHz baseline MIMO feature (for phase 2)

| Parameter | Value |
| --- | --- |
| TXRU-to-antenna element mapping | The TXRU virtualization of baseline is static within simulation time for below 6GHz evaluation. |
| Numerology and TTI length | Normal TTI (1ms length), with 15kHz sub-carrier spacing |
| Constraints for beam range of beamformed CSI-RS | The same as phase 1 calibration for NR MIMO. |
| Scheduler | PF scheduler |
| Traffic model | Full buffer and FTP model as in Table A.2.1-1 in [1] Traffic model of the enhanced feature evaluation should be the same as is used in baseline evaluation. |
| UE distribution | Follow Table A.2.1-1 in [1] |
| DL feedback scheme | PUSCH 3-2 |
| DL BF scheme | Option 1: Digital BF based on SVD according to SRS measurement Option 2: codebook based precoding |
| MIMO mode | TM10 with a single CSI process |
| Maximal layer number | Up to 8 layer for both MU- and SU-MIMO |
| Control overhead | Companies report |

Table 4 Additional parameter for above 6GHz baseline MIMO feature (for phase 2)

| Parameter | Value |
| --- | --- |
| TXRU-to-antenna element mapping | Analog beam selection is used. - Beam pair selection between each TRP and UE is based on method 2 in phase 1 calibration. - The beam pair between each TRP and UE is static within the simulation time. - TXRU to antenna element mapping is the same across all TXRUs. |
| Beam switching | Round robin beam switching for each TRP; switch among the beams which are selected by one or more serving UEs. |
| CSI-RS transmission | CSI-RS transmission for each analog beam |
| Numerology and TTI length | [0.25ms] TTI length, with [60kHz] sub-carrier spacing |
| Constraints for beam range of beam selection | The same as phase 1 calibration for NR MIMO. |
| Scheduler | PF scheduler; the users under the same analog beam is scheduled. |
| Traffic model | Full buffer and FTP model as in Table A.2.1-1 in [1] Traffic model of the enhanced feature evaluation should be the same as is used in baseline evaluation. |
| UE distribution | Follow Table A.2.1-1 in [1] |
| DL feedback scheme | PUSCH 3-2 |
| DL BF scheme | Option 1: Digital BF based on SVD according to SRS measurement Option 2: codebook based precoding (if FDD is considered) |
| MIMO mode | TM10 with a single CSI process |
| Maximal layer number | Up to 8 layer for both MU- and SU-MIMO |
| Control overhead | Companies report |

Proposal 3: Use Table 2~Table 4 as system-level simulation parameters for baseline MIMO features (for phase 2 calibration).

Conclusions

In this contribution, phase 2 MIMO calibration assumption and baseline feature selection is discussed. The following proposals are made.

Proposal 1: Phase 2 calibration should focus on the alignment of MIMO baseline features and performance that will be used for performance comparison with enhanced NR MIMO features.

Proposal 2: For NR MIMO performance comparison, LTE Rel-13 MIMO features listed in Table 1 should be used as baseline.

Proposal 3: Use Table 2~Table 4 as system-level simulation parameters for baseline MIMO features (for phase 2 calibration).

- References

R1-168526, TR38.802 v0.1.0 “Study on New Radio (NR) Access Technology Physical Layer Aspects”, Aug 2016.

R1-166109, “Antenna structure: impact on MIMO transmission and remaining modeling issues”, Huawei, HiSilicon, Aug 2016.