3GPP TSG-RAN WG1 Meeting #87	R1-1611283

Reno, USA, 14th – 18th November 2016

Agenda item:		6.2.2.1.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Design on Non-precoded CSI-RS codebook

Document for:		Discussion and Decision

# Introduction

The WI of further enhancement on eFD-MIMO was agreed in RAN#71, with objectives of new CSI-RS design and mechanism for reducing the overhead for CSI-RS transmission. Following conclusions have been achieved in RAN1#86b,

- Agree on all (N1, N2) combinations on slide 4 of R1-1010560

- Agree on the following table for (O1, O2) combinations (Alt1 of the left table on slide 4 of R1-1610560)

In this contribution, we will provide our understanding and proposals on codebook design supporting up to 32 antenna ports.

# Codebook Design for newly Supported Port Layouts

## Codebook design for rank 1/2

A CSI reporting is associated with at least two PMIs and corresponding codebook for deriving PMI. The double codebook structure has been introduced since Rel-10 and used in Rel-13 to support antenna configurations with 12/16 antenna ports. The double codebook structure also can be used for support of {20, 24, 28, 32} CSI-RS ports. To minimize specification and implementation impact, the codebook structure of Rel 10/13 can be reused for new antenna port configurations.

Proposal 1: For supporting {20, 24, 28, 32} CSI-RS ports, Rel-13 codebook design structure and principles should be reused.

The final precoding matrix therefore can be expressed as the following

where W1 is a long-term and wide-band feedback which provides a group of beams based on long-term channel statistic. W2 is a relatively short-term and sub-band/wide-band feedback. The long-term column selection processing is defined by col(·) function, which can be configured by higher layer signaling, for example by Table 1. The mapping method from 2-D beam grid (Config. 2/3) to 1-D beam grid can be expressed in Table 1.

Table 1: Beam grid mapping method for long term column selection Config. 2/3

| Configuration | Beam index (h,v) (indexed by 0,1,2,3) |
| --- | --- |
| Config 2 | (0,0) (1,0) (0,1) (1,1) |
| Config 3 | (0,0) (2,0) (1,1) (3,1) |

The structure of W1 can be expressed as

where i1,1 and i1,2 denote indices of matrix X1 and X2, respectively. And N1 and N2 denote the number of antenna ports of first dimension and second dimension, respectively. O1 and O2 denote the oversampling factor of first and second dimension, respectively. s1 and s2 denotes the grouping space between beam groups of X1 and X2, respectively.

The long-term precoder is controlled by two feedback PMIs (i1,1, and i1,2), which indicates the first dimension beam group index and the second dimension beam group index, respectively. The beam group X1 and X2 can be expressed as follows:

where each column can be expressed as

where p and q denotes the column index in beam group of 1st dimension and 2nd dimension, respectively.

Proposal 2: Rel-13 Design principle for W1 can be reused to generate new W1 supporting up to 32 transmitting antenna ports.

The short term codebook design can be expressed as follows:

- For configuration 2/3:

where k1 and k2 denotes the column index representing the first and the second selected column vectors respectively.

For rank 1, the possible combination set of  is:

For rank 2, the possible combination set of  is:

Proposal 3: Codebook configuration definition of 1-4 should be reused for new designed codebook.

Proposal 4: W2 design from Rel 13 should be reused in new codebook design for rank 1 and rank 2.

## Codebook design for rank 3/4

For rank3 and rank4 codebook design, we reuse the Rel-13 design to guarantee the orthogonality between different ranks with selected beams. The W1 can be designed as rank 1 and rank 2, with following update:

Where  and p denotes beam index shift, which can be defined as the following:

and  can be determined by parameter K which will be fed back from UE, the detailed definition as follows

Table 2:  and  value definition

| K |  | 0 | 1 | 2 |
| --- | --- | --- | --- | --- |
| If N1>1 and N2>1 |  | O1 | 0 |  |
|  |  | 0 | O2 |  |
| If N2=1 |  | O1 | 2O1 | 3O1 |
|  |  | 0 | 0 | 0 |

The short term codebook design can be expressed as follows:

- For configuration 2/3,

where k1 and k2 denotes the column index selected for the first and the second column vectors respectively.

For rank 3, the possible beam selection combination set is:

Proposal 5: W2 design from Rel 13 should be reused in new codebook design for rank 3.

For rank 4, the possible combination set of  is:

# Performance Evaluation

To verify system performance of proposed codebooks, we have provided non-full buffer system performance of FDD system with different traffic loading. Unless it is stated, SU/MU MIMO dynamic switching scheme is adopted and FDD system is assumed for all cases below. We have used proposed codebook in Sections 2 for following simulation cases with the maximal transmission rank of one per UE. The number of receiving antenna is 4 at UE side. For a long term column selection, configuration 2 is assumed only. The codebook dimension parameters (N1, N2) represent the horizontal and vertical domains respectively. For TXRU virtualization, our simulations following the table below:

Table 3: TXRU Virtualization

| No. of vertical antenna ports (N2) | Number of antenna elements per port (K) | Number of antenna elements per column (M) |
| --- | --- | --- |
| 2 | 4 | 8 |
| 3 | 3 | 9 |
| 4 | 2 | 8 |
| 5 | 2 | 10 |
| 6 | 1 | 6 |
| 7 | 1 | 7 |
| 8 | 1 | 8 |

Table 4: Non-full buffer performance with low traffic load of proposed codebook design

| Scenario (Config.) | Antenna Configuration (N1, N2)=(H,V); Overhead of W1 | (O1, O2) | RU & Performance |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Resulting RU | 5% UPT | 50% UPT | Mean |  |
| 3D-UMi, 200m/2GHz (Config.2) | 16 ports=(4,2); 7 bits | (4,4) | 20.87% | 16.13 (100.00%) | 37.38 (100.00%) | 29.84 (100.00%) |  |
|  | 24 ports= (4,3); 7 bits | (4,4) | 19.66% | 17.62 (109.25%) | 39.22 (104.90%) | 32.53 (109.03%) |  |
|  | 32 ports= (4,4); 8 bits | (8,4) | 19.30% | 18.43 (114.29%) | 40.82 (109.18%) | 35.51 (119.02%) |  |
|  | 16 ports= (2,4); 7 bits | (4,4) | 20.01% | 17.07 (100.00%) | 37.04 (100.00%) | 31.50 (100.00%) |  |
|  | 20 ports= (2,5); 7 bits | (8,4) | 21.43% | 18.02 (105.86%) | 39.60 (106.93%) | 34.58 (109.77%) |  |
|  | 24 ports= (2,6); 7 bits | (8,4) | 20.93% | 20.41 (119.90%) | 39.60 (106.93%) | 34.88 (110.71%) |  |
|  | 28 ports= (2,7); 7 bits | (8,4) | 18.59% | 20.73 (121.76%) | 40.00 (108.00%) | 35.28 (111.99%) |  |
|  | 32 ports= (2,8); 8 bits | (8,4) | 16.68% | 22.17 (130.24%) | 40.40 (109.09%) | 37.66 (119.55%) |  |

Table 5: Non-full buffer performance with medium traffic load of proposed codebook design

| Scenario (Config.) | Antenna Configuration (N1, N2)=(H,V); Overhead of W1 | (O1, O2) | RU & Performance |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Resulting RU | 5% UPT | 50% UPT | Mean |  |
| 3D-UMi, 200m/2GHz (Config.2) | 16 ports=(4,2); 7 bits | (4,4) | 41.73% | 8.33 (100.00%) | 29.85 (100.00%) | 21.93 (100.00%) |  |
|  | 24 ports= (4,3); 7 bits | (4,4) | 41.01% | 10.93 (131.15%) | 33.06 (110.74%) | 26.15 (119.23%) |  |
|  | 32 ports= (4,4); 8 bits | (8,4) | 40.02% | 14.04 (168.42%) | 35.40 (118.58%) | 29.16 (132.97%) |  |
|  | 16 ports= (2,4); 7 bits | (4,4) | 41.67% | 10.93 (100.00%) | 32.79 (100.00%) | 25.61 (100.00%) |  |
|  | 20 ports= (2,5); 7 bits | (8,4) | 41.85% | 12.66 (115.82%) | 32.79 (100.00%) | 27.20 (106.23%) |  |
|  | 24 ports= (2,6); 7 bits | (8,4) | 40.10% | 13.42 (122.82%) | 34.48 (105.17%) | 27.85 (108.78%) |  |
|  | 28 ports= (2,7); 7 bits | (8,4) | 37.91% | 15.04 (137.59%) | 34.78 (106.09%) | 28.77 (112.37%) |  |
|  | 32 ports= (2,8); 8 bits | (8,4) | 36.13% | 16.67 (152.50%) | 35.40 (107.96%) | 30.55 (119.31%) |  |

Observation 1: Considerable system performance gain can be obtained at medium to high traffic loading by increasing the number of vertical antenna ports in 3D-UMi.

# Conclusions

In this contribution, we have provided our view on codebook design for up to 32 CSI-RS ports with 1D or 2D antenna array for 3D MIMO. The system level simulation results are provided to confirm performance gain by using proposed codebooks.

Proposal 1: For supporting {20, 24, 28, 32} CSI-RS ports, Rel-13 codebook design structure and principles should be reused.

Proposal 2: Rel-13 Design principle for W1 can be reused to generate new W1 supporting up to 32 transmitting antenna ports.

Proposal 3: Codebook configuration definition of 1-4 should be reused for new designed codebook.

Proposal 4: W2 design from Rel 13 should be reused in new codebook design for rank 1 and rank 2.

Proposal 5: W2 design from Rel 13 should be reused in new codebook design for rank 3.

Observation 1: Considerable system performance gain can be obtained at medium to high traffic loading by increasing the number of vertical antenna ports in 3D-UMi.

# References

- RP-160623 New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE

# Appendix: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Homogeneous scenarios | 3D-UMi with ISD = 200m in 2GHz, 41 dBm |
| Polarized antenna modelling | Model -2 from 36.873 |
| Duplex | FDD |
| Traffic model | Full Buffer (Mean Throughput, 5% UE Throughput) FTP Model 1 with packet size 0.5 Mbytes |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| System bandwidth | 10MHz |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | According to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Ideal channel estimation and interference modelling |
| Receiver UE Rx configuration | LMMSE-IRC receiver |
|  | 4 Rx x-polar (+90/0) |
| Feedback | PUSCH 3-2 for non-reciprocity operation |
| Feedback Transmission scheme | CQI and RI reporting triggered per 5ms |
|  | CQI Feedback delay is 5 ms and ideal channel estimation at the UE |
|  | MU-MIMO with rank adaptation |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Scheduler | Frequency selective scheduling |