3GPP TSG RAN WG1 Meeting #87		                      R1-1611477

Reno, USA 14th - 18th November 2016

Agenda Item:  7.1.3.4

Source:	Spreadtrum Communications

Title:	Phase 1 system level calibration results

Document for:	Discussion and decision

# Introduction

Phased approach on the calibration of the massive MIMO simulations was discussed in ‎[1] and further e-mail discussion has been conducted after the RAN1 #86bis in [86b-19]. This contribution contains massive MIMO calibration results according to the agreed phase 1 parameters. The included results are system level results in Urban macro and Indoor hotspot scenarios.

# System level simulation results

The system level assumptions are captured in Table 1 being inline with the agreed parameters. During the simulation the UE selects the analog transmit and receive beam pair which maximizes the received power. This involves also the UE receiver panel selection since the panel boresights are pointing to the opposite directions.

The geometry factor is shown in Figure 1 for reference and the downlink SINR with and without beamforming is depicted in Figure 2. The beamforming effect from the interfering cells has been taken into account in the SINR calculation. Further, the SINR from all UEs is collected in the statistics even though significant amount of UEs would be in outage in Urban macro case and not necessarily being scheduled in practice.

Table 1. Simulation parameters.

| parameter | value |
| --- | --- |
| scenario | Urban macro and Indoor hotspot (12 TRPs), (‎[1], ‎[2] and ‎[3]) |
| carrier frequency | 30GHz |
| TRP antenna configuration, (M,N,P,Mg,Ng) | Urban macro : (4,8,2,2,2) Indoor hotspot : (4,8,2,1,1) |
| TRP antenna radiation pattern | Urban macro : H and V ‎[2] Indoor hotspot : H and V(boresight direction perpendicular to the ceiling) ‎[4] |
| UE antenna configuration, (M,N,P,Mg,Ng) | (2,4,2,1,2) Θmg,ng=90; Ω0,1=Ω0,0+180 |
| UE antenna radiation pattern | As in ‎[2], H and V |
| Number of TXRUs | 1 TXRU per panel per polarization |
| Analog precoding | Horizontal and vertical DFT precoding assuming oversampling factor of 1 |
| Digital precoding | SVD, unquantized |
| Number of UEs per cell | 10 |
| MIMO scheme | SU-MIMO, 1-layer |
| UE receiver | MMSE-IRC |
| scheduling | Round robin, full band allocation |
| Traffic model | Full buffer |

Figure 1. The geometry factor distributions.

Figure 2. SINR with and without beamforming in Urban Macro Scenario.

Figure 3. SINR with and without beamforming in Indoor hotspot Scenario.

# Conclusions

Massive MIMO phase 1 calibration results were presented in this contribution. The attached results are for the Urban macro and Indoor hotspot system level scenarios.

# Reference

- ZTE, ZTE Microelectronics, “Summary of [86-20] email discussion on MIMO calibration for NR”, R1-1608661

- 3GPP TR 38.900, “Study on channel model for frequency spectrum above 6 GHz”

- NTT DOCOMO, INC, “Offline dicussion further updated sumary of evaluation assumptions for NR“, R1-168547

- ZTE, ZTE Microelectronics, “Further clarification on assumptions for Phase 1 NR-MIMO calibration”