3GPP TSG-RAN WG1 #87 	R1-1612089

14th – 18th November 2016

Reno, USA

Agenda item:	7.1.5.1

Source: 	Qualcomm Incorporated

Title: 	Control Channel Complexity Considerations

Document for:	Discussion/Decision

# Introduction

The control channel performance for LDPC, TBCC, and Polar codes was investigated in the companion contribution [1]. There it was observed that decoding enhancements could be enabled to improve the performance relative to legacy control channels, in particular by introducing list decoding. The purpose of this document is provide more detailed analysis of implementation complexity and latency costs for such enhancements, relative to the well-established TBCC performance for the current LTE control channel. As the wireless network ecosystem has grown accustomed to the area efficiency and latencies of TBCC, it becomes important to understand how different these metrics become when more complicated decoding enhancements are introduced.

In this contribution we focus on the comparison between Polar codes and TBCC.

# Computational Complexity

## TBCC decoder

| TBCC Decoder Component | Complexity |
| --- | --- |
| Branch metric calculation | min2CX+1-2, CX×S2X×2K/X |
| Trellis development for state determination | S2X ×K/X+S(2X-1)×K/X |
| Trellis development for list decoding | S(L+2X-1) ×K/X +S(2X-1)L×K/X |
| Handling of the final stage and backtrace | LlogL+KL |
| Parameters: - Number of info bits (containing CRC): K - Number of coded bits per info bit: C - Number of states: S - Number of input states per state: 2X (Radix 2X) - List Size: L |  |

## Polar SC-List Decoder

| Polar SC-List Decoder Component | Complexity |
| --- | --- |
| Single Parity Check | abs(): N*log(N)*L sign():N*log(N)*L min(): 0.5N*log(N)*L |
| Repetition | add()/sub(): 0.5N*log(N)*L |
| Path Metric Calculation | abs(): LN add():LN |
| Sorting | comp(): 2LK(log22L) |
| Parameters: - Number of info bits (containing CRC): K - Number of coded bits: N - List Size: L |  |

## Computational complexity comparison

In this subsection, we provide a complexity study for the K=40, N=120 scenario, which represents a typical control channel use case for DCI. This was also presented prior in [2], where more details on the simulation assumptions can be found. Recall from [1] that significant performance gains were realized with list decoding for both TBCC and Polar codes. By using the analysis of computation complexity in the previous subsections, in addition the performance achieved with these techniques, we provide a performance vs. computational complexity comparison in Figure 1., as a function of the list size.  The following observation can be made.

Observation 1:

- For R=1/3 and 40-bit DCI, the list size for the Polar list decoder needs to be nearly 2x that of the TBCC list decoder to achieve the same SNR for 10e-3 BLER.

- Polar list decoding is needed to achieve similar performance as TBCC decoder.

- For larger list sizes, TBCC(L=16) and Polar(L=32) begin to have the similar computational complexity and performance tradeoffs.

Figure 1 Computational Complexity vs Performance

# Implementation complexity

## TBCC Viterbi Decoder Implementation Complexity

To reduce decoding latency, we use a radix-4 Viterbi decoder, where two consecutive trellis steps are merged and processed simultaneously. In this section, we only discuss the implementation complexity of an L=1 decoder. The implemented decoder can be used to perform the state-determination pass in addition to the Viterbi decoding.

### Branch Metrics

Each of the 4.2M radix-4 branch metrics is the sum of two radix-2 branch metrics:

γlSi'', S= γl-1Si'', Sj'+ γl(Sj', S)

There are 2.2.2M radix-2 branch metrics in the merged radix-4 trellis stage, each of which is calculated according to

γlSi', S=j=13yl(j)xl(j)

xl ∈{-1, 1}, so multiplying by it can only change the sign of the other operand and can be implemented using a subtractor. Therefore, a single radix-2 branch metric calculator requires three subtractors and two adders.

Calculating the radix-4 branch metrics requires 4.2M adders and calculating the component radix-2 branch metrics requires 5.2.2.2Madders for a total of 24.2M adders.

### State Metrics (Forward Recursion)

The state metric, αl, for an even-indexed trellis stage, Sl, which corresponds to two bits l and l-1, is calculated using an add-compare-select (ACS) unit:

αlS=max0 ≤ j<4(αl-2Si''+ γlSi'', S)

The ACS unit comprises four adders and three maximum value calculations (comparators). To prevent numerical saturation of the state metrics, a constant offset is subtracted from all metrics in a trellis stage when any of the metric exceeds a threshold. The total implementation complexity of the ACS unit and the offsets is 5.2M adders and 3.2M comparators.

The backtracking step is excluded from the complexity logic implementation analysis since it only comprises look-ups from memory. It will be included in the memory complexity analysis.

### Memory Requirements

The TBCC requires four different memories to be implemented. The first is the input LLR memory, which stores two sets of N channel LLRs of Qc bits each. Double buffering the input LLRs enables the decoder to decode a codeword while receiving another. The state-metric memory stores 2M metrics of Qi bits each. The survivor memory stores 2M M-bit indices.

To facilitate backtracking, the two input bits associated with the winning branch at each state are stored for the entire trellis, requiring K2M bits.

The total internal memory (in bits) is 2M(Qi+M+K).

### Area Estimate

We use two Viterbi decoders for convolutional codes in the literature as the bases of the our area estimate for the TBCC decoder. The first is an asynchronous design using a window Viterbi decoder for 802.11a code with M = 6 and rate = ½ [7]. The other is for the same code, but is radix-4 and implements enough traceback memory for K = 64 bits [8].

The code rate in a Viterbi decoder affects the complexity of the branch calculation unit (BMU), which will scale linearly with the inverse of the rate. So the scaling factor will be (1/2)/(1/6) = 3.

The traceback memory (TMU) in a non-window decoder will scale linearly with K therefore we scale the TMU in [8] by a factor of 200/64. In a window decoder, the window size 5 times the constraint length, which would be 30 for [7]. Therefore we scale that TMU by  factor of 200/30. The table below lists the scaling results, where it can be seen that the expected decoder area is less than 0.05 mm2 in 14 nm technology.

|  | Asynchronous Design |  | Synchronous Design |  |
| --- | --- | --- | --- | --- |
| K, R | 30, 1/2 | 200, 1/6 | 64, 1/2 | 200, 1/6 |
| TMU | 0.0872 | 5.8 | 1.27 | 3.97 |
| BMU | 0.06 | 0.18 | 0.11 | 0.33 |
| Others | 1.03 | 1.03 | 1.68 | 1.68 |
| Total (mm2) | 1.96 | 7.02 | 3.1 | 5.98 |
| Total area in 14 nm (mm2) |  | 0.042 |  | 0.036 |

## Polar List Decoder Implementation Complexity

A polar code of length N is the concatenation of two polar sub-codes of length N/2. This concatenation process is applied recursively until sub-codes of length 1 are reached. These length-1 sub-codes either carry an information bit, or a frozen bit. Successive-cancellation (SC) decoding calculates the input to each sub-code recursively until length-1 sub-codes are reached.

### Memory Requirements

The decoder requires 2NQc bits to store two sets of channel LLRs. In addition, each list element stores i=1log2(N2)N2i≈N LLRs for a total of LNQi bits of internal LLR memory.

The list decoder also stores an N-bit estimated codeword and N internal bit estimates (also called partial sums) per list item [3].

While the list decoder also stores L path metrics (of Qm bits each), the size of this memory is negligible compared to the rest and we ignore it in this analysis. Therefore, the total memory required by the list decoder is approximately 2N(Qc + 1) + N*L(Qi + 1) bits.

In [5], recalculating LLR values corresponding to the larger stages in the decoder instead of storing them was proposed as method to reduce the decoder memory requirements. The output LLRS for stages log2N2, log2N4, log2N8, and log2N16 are not stored, reducing the internal LLR memory to i=1log2(N32)2i≈N16 LLRs per list element.

The internal memory for the polar list decoder becomes N16LQi+2NL bits. The impact of the LLR recalculation on implementation complexity is discussed in a later section. The memory values reported in [3] include additional memory overhead not captured in this estimate.

To maintain clock frequency, pipeline registers are inserted at the output of the stage log2N4 recalculation. These pipelining registers are excluded from our analysis.

### F and G blocks:

The decoder implements L*P/2 f and g blocks. This number is limited by the available memory width per list item (P).

Each f block performs signasignbmin⁡(a, b) and consists of two 2s-complement-to-sign-magnitude converters, a minimum-value calculator, and one sign-magnitude-to-2s-complement converter. The converters can be implemented using one adder each and the minimum value calculator can be implemented using a comparator.

Each g block performs b+a or b-a depending on a combination of estimated bits us and consists of an adder and a subtractor.

The total complexity of the f and g logic is L*P/2 * (3 + 2) = 5/2LP adders and L*P/2 comparators.

When the LLR recalculation method is used to reduce memory [5], an additional LP4+P8+P16=716LP f and g blocks are required, increasing the total complexity to 7516LP adders and 1516LP comparators.

### Path-metric Sorting

The list of path metrics is sorted using a bitonic sorting network with 2L inputs. The basic building block in a sorting network is a 2-input sorter composed of a comparator and two 2x1 multiplexers. A sorting network with M inputs has i=1log2MiM/2=M4log2M(log2M+1) 2-input sorters, leading to an implementation complexity of L2log22L(log22L+1) comparators.

### Maximum Likelihood Decoder

To decrease decoding latency, the decoders in [3] employ elements of simplified successive cancellation (SSC) list decoding [4], with a restricted sub-code size of 4 bits. One major block in the SSC list decoder is the maximum likelihood (ML) decoding block.

In a 4-bit sub-code SSC list decoder, the ML decoder must decode all sub-codes of length 4 with 3, 2, and 1 information bits. The implementation requires enumerating up to eight codewords, calculating the ML metric of each, and finding the codeword with the largest metric. This requires 24 adders to calculate the metrics and 3 comparators to find the maximum in each list item.

## Summary

|  | TBCC (L=1) | Polar |
| --- | --- | --- |
| Adders | 29.2M | 7516LP + 24L |
| Comparators | 3.2M | L2log22L(log22L+1)+1516LP + 3L |
| Internal Memory (bits) | 2M(Qi+M+K) | N16LQi+2NL |

### Example

To numerically compare the two decoders, we use the following parameters:

- Code: N = 1000, K = 200

- TBCC: M = 6, Qi = 10.

- Polar: P = 64.

|  | TBCC | Polar (L = 8) [3] | Polar (L = 32) |
| --- | --- | --- | --- |
| Adders/Comp. | 2,048 | 3,208 (1.5x) | 3,800 (1.8x) |
| Memory (bits) | 13,824 | 23,000 (1.6x) | 80,000 (5.8x) |

For the polar decoder with list size 32, only the sorting logic and memory were increased to accommodate the larger list size, the estimate only included 8 path processing elements. This was done to limit the increase in decoder implementation complexity and is also performed in [3]. While this approach saves area, it increases decoding latency.

### Area Estimate

Here, we extrapolate the area of the TBCC decoder and the polar list decoder with L = 32. For the TBCC decoder, we estimate the area as < 0.1 mm2 based on the scaling performed in Section 3.1.4. For the polar decoder with list size 32, we calculated the area required by the additional memory based on the numbers reported in [3] and added to the area of the L = 8 decoder.

|  | TBCC | Polar (L = 8) [3] | Polar (L = 32) |
| --- | --- | --- | --- |
| Area in 14 nm (mm2) | <0.1 | 0.24 (>2.4x) | 0.29 (> 2.9x) |

Observation 2:

- Polar list decoders are expected to require a significantly higher area relative to the TBCC decoder.

# Latency

## TBCC decoding latency

In the radix-4 decoder, the state-determination pass requires K/2 clock cycles. Similarly, the forward require K/2 clock cycles. Due to the additional branch information stored, the backtracking can be greatly accelerated and will require K/8 cycles when four trellis stages are processed simultaneously. The total latency will be 9/8K cycles.

## Polar SC-List Decoder Latency

Using a sorting network, one can sort 2L path metrics in (log2(2L)+1) log2 (2L)/2 cycles assuming each stage in the sorting network is performed in a cycle. A less conservative estimate assumes that 4 stages can be performed in a clock cycle, reducing the sorting latency to ⌈(log2(2L)+1) log2 2L/(2*4)⌉ cycles. Since sorting occurs only after an information bit is estimated, the per-codeword latency due to sorting in SC-list decoding is
K(log2(2L)+1) log2 2L2*4

cycles. Therefore, the total latency of an SC-list decoder is
2s=2log2N2sPN2s+K(log2(2L)+1) log2 2L2*4

clock cycles.

SSC-list decoding has lower latency, but it is dependent on the location of frozen bits (the code construction parameters) in addition to the code length and rate. The latency of an SSC-list in clock cycles is the sum of the latency of all sub-codes in the pruned code graph:

i2NiP+ Si

where Ni is the length of sub-code i, 2Ni/P is the number of cycles required to calculate the sub-decoder’s input, and Si is the latency due to sorting and pipelining in the sub-code decoder. Note that although latency might be reduced further by increasing memory width P, this will come at a larger cost of logic (and thus area which scales linearly). Moreover, the sorting latency eventually becomes the bottleneck. The sorting could also be reduced, but at greater cost to hardware implementation [6].

Si depends on the type of sub-code:

- It is 0 if the sub-code will not be directly decoder, i.e. if the decoder will perform an F or a G function.

- It is the latency of a 2L-input sorting network for rate-1 sub-codes.

- It varies based on sub-code length and rate for ML decoding.

## Summary

|  | TBCC | Polar SSC-List |
| --- | --- | --- |
| Latency (cycles) | 9K8 | i2NiP+ Si |

### Example

We calculate the latency for the TBCC decoder and compare with the latency reported in [3] for the polar decoder with L = 8. Because the number of computational elements remained the same in the complexity analysis when increasing the list size from 8 to 32, the latency of the polar decoder with list size = 32 will be at least four times that of the L = 8 decoder.

- Code: N = 1000, K = 200

|  | TBCC | Polar (L = 8) | Polar (L = 32) |
| --- | --- | --- | --- |
| Latency @ 1 GHz (us) | 0.225 | 0.47 (2x) | 1.880 (8.4x) |

Observation 3:

- Polar list decoders can have significantly higher latency than the TBCC decoder.

Proposal 1: TBCC should be adopted for the EMBB control channel to maintain high area efficiencies and very low latencies which will be critical for NR.

# Conclusions

For K = 200, N = 1000, we have the following results:

|  | TBCC | Polar (L = 8) [3] | Polar (L = 32) |
| --- | --- | --- | --- |
| Adders/Comp. | 2,048 | 3,208 (1.5x) | 3,800 (1.8x) |
| Memory (bits) | 13,824 | 23,000 (1.6x) | 80,000 (5.8x) |
| Estimated Area in 14 nm (mm2) | <0.1 | 0.24 (>2.4x) | 0.29 (>2.9x) |
| Latency @ 1GHz (us) | 0.225 | 0.47 | 1.9 |

Observation 1:

- For R=1/3 and 40-bit DCI, the list size for the Polar list decoder needs to be nearly 2x that of the TBCC list decoder to achieve the same SNR for 10e-3 BLER.

- Polar list decoding is needed to achieve similar performance as TBCC decoder.

- For larger list sizes, TBCC(L=16) and Polar(L=32) begin to have the similar computational complexity and performance tradeoffs.

Observation 2:

- Polar list decoders are expected to require a significantly higher area relative to the TBCC decoder.

Observation 3:

- Polar list decoders can have significantly higher latency than the TBCC decoder.

Proposal 1: TBCC should be adopted for the EMBB control channel to maintain high area efficiencies and very low latencies which will be critical for NR.

# References

- Qualcomm, Incorporated, R1-1612088, “Control channel performance”, RAN1 #87, Reno, USA, 2016.

- Qualcomm, Incorporated, R1-1610141, “Short blocklength design”, RAN1 #86bis, Lisbon, Portugal, 2016.

- Huawei and HiSilicon, R1-1608865 “Design aspects of polar code and LDPC for NR,” RAN1-86bis, Lisbon, Portugal, 2016.

- Lin et al. “Design of a Power-Reduction Viterbi Decoder for WLAN Applications,” IEEE TCAS-I, Vol. 52, No. 6, 2005.

- Lin et al, “A High Throughput List Decoder Architecture for Polar Codes,” T-VLSI, Vol 24, No. 6, Jun. 2016.

- Huawei, HiSIlicon  R1-167213  “Computational and implementation complexity of channel coding schemes,” RAN1 86, Gothenburg, Sweden.

- Kawokgy and Salama, “Low-power asynchronous Viterbi decoder for wireless applications,” ISLPED’04, 2004.

- Lin et al. “Design of a Power-Reduction Viterbi Decoder for WLAN Applications,” IEEE TCAS-I, Vol. 52, No. 6, 2005.