3GPP TSG RAN WG1 Meeting #87                 		R1-1612112

Reno, USA 14th - 18th November 2016

Source:	Panasonic

Title: 	Discussion on size alignment between SL SPS DCI and DCI format 0

Agenda Item:		6.2.1.1.1

Document for:		Discussion and Decision

Introduction

Followings were agreed in RAN1#86bis meeting [1],

Agreement:

- An SL SPS RNTI different from SL dynamic scheduling RNTI is defined.

- The following two fields are additionally included in the existing dynamic scheduling DCI (i.e., DCI 5A).

  - Working assumption:

    - SL SPS configuration index : 3 bits

    - Activation/release indication : 1 bit

- Each SL SPS DCI contains a single SPS configuration index.

- If the number of information bits in SL SPS DCI format mapped onto a given search space is less than the payload size of DCI format 0 mapped onto the same search space, zeros shall be appended to SL SPS DCI format until the payload size equals that of DCI format 0 including any padding bits appended to DCI format 0.

  - FFS whether the size of SL SPS DCI format can be larger than the size of DCI format 0 on the same search space.

This document will discuss FFS point and show our view on this aspect.

Discussion

Based on the analysis in following table 1, in the case that SL SPS DCI schedules 20MHz bandwidth and DCI format 0 schedules 1.4MHz bandwidth, the size of SL SPS DCI format is larger than that of DCI format 0 on the same search space. Some detailed assumptions on DCI content could refer to Appendix part.

Table 1 Size comparison between SL SPS DCI and DCI format 0

| SL SPS DCI \ DCI format 0 | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1.4MHz | 11 \ 23 | 11 \ 25 | 11 \ 27 | 11 \ 29 | 11 \ 30 | 11 \ 31 |
| 3MHz | 16 \ 23 | 16 \ 25 | 16\ 27 | 16 \ 29 | 16\ 30 | 16 \ 31 |
| 5MHz | 18 \ 23 | 18\ 25 | 18\ 27 | 18\ 29 | 18\ 30 | 18 \ 31 |
| 10MHz | 21 \ 23 | 21\ 25 | 21\ 27 | 21\ 29 | 21\ 30 | 21\ 31 |
| 15MHz | 22 \ 23 | 22 \ 25 | 22 \ 27 | 22 \ 29 | 22 \ 30 | 22 \ 31 |
| 20MHz | 24 \ 23 | 24\ 25 | 24\ 27 | 24\ 29 | 24\ 30 | 24 \ 31 |

So our observation is

Observation 1: There is a case that the size of SL SPS DCI format is larger than that of DCI format 0 on the same search space, especially when SL SPS DCI schedules 20MHz but DCI format 0 schedules 1.4MHz.

The behaviour should be clarified for this case. There would be several ways to solve the problem including the following,

Option 1: DCI format 0 is padded to align the size of SPS SL DCI.

Option 2: SPS SL DCI is padded to align the size of TM-specific DCI

Option 3: There is some restriction on field usage in SPS SL DCI or DCI format 0. For example in case SPS SL DCI is used to schedule 20MHz but DCI format 0 is used for schedule 1.4MHz, the allowed sub-channel size for sidelink should be larger than 6 PRBs so that the total size of SL SPS DCI could be smaller than or equals to  23 bits. Another way is to restrict SPS configuration index from 3 bits to 2 bits.

Among the three options, option 1is simplest but it will impact performance of Uu DCI a little bit. Option 2 is only workable in case TM-specific DCI size is larger than SPS SL DCI. Option 3 would impact flexibility of SL SPS resource allocation. Each option has its pros and cons and we are fine to go with any option as far as the raised problem is solved.

Proposal 1: The behaviour when the size of SL SPS DCI format is larger than that of DCI format 0 on the same search space should be clarified among one of the following options: 1) DCI format 0 is padded to align the size of SPS SL DCI 2) SPS SL DCI is padded to align the size of TM-specific DCI 3) Restriction on field usage of SPS SL DCI or DCI format 0.

Conclusion

In this contribution we mainly discussed the case that the size of SL SPS DCI format is larger than the size of DCI format 0 on the same search space and would like to propose,

Proposal 1: The behaviour when the size of SL SPS DCI format is larger than that of DCI format 0 on the same search space should be clarified among one of the following options:

1) DCI format 0 is padded to align the size of SPS SL DCI

2) SPS SL DCI is padded to align the size of TM-specific DCI

3) Restriction on field usage of SPS SL DCI or DCI format 0.

- Reference

[1] RAN1#86bis chairman notes

Appendix

Table 2 Size calculation on SL SPS DCI (adjacent resource allocation between SA and data)

|  | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier indicator | 3 | 3 | 3 | 3 | 3 | 3 |
| Lowest index of the subchannel allocation | 0 | 2 | 3 | 4 | 4 | 5 |
| Frequency resource location | 0 | 3 | 4 | 6 | 7 | 8 |
| Time gap between initial transmission and retransmission | 4 | 4 | 4 | 4 | 4 | 4 |
| SL SPS configuration index | 3 | 3 | 3 | 3 | 3 | 3 |
| Activation/release indication | 1 | 1 | 1 | 1 | 1 | 1 |
| total payload size | 11 | 16 | 18 | 21 | 22 | 24 |
| Assumption on subchannel size in terms of PRB | 6 | 5 | 5 | 5 | 5 | 5 |

Table 3 Size calculation on DCI format 0

|  | 1.4MHz | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier indicator | 3 | 3 | 3 | 3 | 3 | 3 |
| Flag for format0/format1A differentiation | 1 | 1 | 1 | 1 | 1 | 1 |
| Hopping flag/RA Type 1 MSB | 1 | 1 | 1 | 1 | 1 | 1 |
| Resource block assignment and hopping resource allocation | 5 | 7 | 9 | 11 | 12 | 13 |
| Modulation and coding scheme and redundancy version | 5 | 5 | 5 | 5 | 5 | 5 |
| New data indicator | 1 | 1 | 1 | 1 | 1 | 1 |
| TPC command for scheduled PUSCH | 2 | 2 | 2 | 2 | 2 | 2 |
| Cyclic shift for DM RS and OCC index | 3 | 3 | 3 | 3 | 3 | 3 |
| UL index or Downlink Assignment Index (DAI) | 0 | 0 | 0 | 0 | 0 | 0 |
| CSI request | 1 | 1 | 1 | 1 | 1 | 1 |
| SRS request | 0 | 0 | 0 | 0 | 0 | 0 |
| Resource allocation type | 1 | 1 | 1 | 1 | 1 | 1 |
| Total payload size | 23 | 25 | 27 | 29 | 30 | 31 |