3GPP TSG RAN WG1 Meeting #87			R1-1612429

Reno, USA 14th - 18th November 2016

Agenda item:	6.2.2.4.2

Source: 	Samsung

Title: 	Remaining Issues on Hybrid CSI

Document for:	Discussion and Decision

# Introduction

The following agreement on mechanism 1 of hybrid CSI was made in RAN1#86 [1]:

Agreement - … - For PUCCH-based P-CSI - Report i1(1) and RI(1) in one subframe - FFS: Using either PUCCH format 2 or 3 - ... - For PUSCH-based A-CSI - FFS: what CSI(s) will be reported from UE when aperiodic CSI reporting is triggered - Option 1: UE reports both CSI of 1st eMIMO-type and CSI of 2nd eMIMO-type. - Option 2: UE reports either one of the 2 eMIMO-types. - CSI encoding/mapping mechanisms is FFS.

This contribution focuses on how to resolve the highlighted FFS points in order to complete the specification of mechanism 1 hybrid CSI.

# Open issues and solutions

The above two open issues are discussed in this section.

  - PUCCH format used for 1st eMIMO-Type

For PUCCH-based P-CSI, the 1st eMIMO-Type only requires one-subframe wideband CSI reporting which consists of i1(1) and RI(1). Based on the current CLASS A codebooks (and their potential extensions in Rel.14 to accommodate 20, 24, 28, and 32-port CSI-RS), it is expected that the payload of i1(1) ranges from [6 to 11] bits. Together with RI(1), the total CSI reporting payload for the 1st eMIMO-Type ranges from [7 to 12] bits. This is illustrated in Table 1. This table is constructed assuming that similar to Rel. 13, the oversampling factor (O1,O2) = (8,8) is supported for Rel. 14 antenna port layouts with N1 < N2.

Table 1 Payload size for 1st eMIMO-Type: i1(1)

|  | Range of i1(1) payload size for different (N1,N2) and (O1,O2) |  |
| --- | --- | --- |
|  | 8, 12 & 16 ports (Rel.13) | 20, 24, 28, 32 ports (Rel.14) |
| i1(1) for RI=1 | 6-9 | 7-10 |
| i1(1) for RI=3 | 8-10 | 9-11 |

In light of the payload size, PUCCH format 3 is a better alternative for reporting i1(1)+RI(1) of the 1st eMIMO-Type at least for the following reasons:

- Since i1(1)+RI(1) of the 1st eMIMO-Type is mostly reported less often than the CSI of the 2nd eMIMO-Type, a loss in i1(1)+RI(1) report causes a more severe impact on the system performance. Therefore, i1(1)+RI(1) needs to be well-encoded and more protected. Since the maximum payload size for PUCCH format 2 is 11 bits, the decoding performance (related to the coverage) of i1(1)+RI(1) will be poor if PUCCH format 2 is used. Performance comparison between format 2 and 3 PUCCH associated with the payload sizes in Table 1 is given in the Appendix.

- Although the number of resources for PUCCH format 3 is smaller than PUCCH format 2 (5 versus 6), the use of PUCCH format 3 for reporting i1(1)+RI(1) is not as often as the CSI for the 2nd eMIMO-Type. Therefore, the eNB can “multiple-book” the assignment of PUCCH format 3 resources (when the resource is not used in between reporting instances, the resource can be used by other UEs).

- The specification impact pertaining to the use of PUCCH format 3 for reporting i1(1)+RI(1) is marginal. Regardless of the number of component carriers, PUCCH format (PF) resource usage follows legacy-like procedures as follows.

  - A PF-3 resource is allocated for CSI reporting and/or A/N

  - CSI-only reporting: Use PF-3 resource allocated for CSI

  - A/N-only: If A/N is for Pcell only, use PF-1A/1B resource allocated for A/N. Otherwise use PF-3 resource allocated for A/N

  - CSI and A/N collision:

    - UE drops P-CSI when UE transmits A/N only for the PCell

    - Otherwise, UE follows Rel-13 procedure as if PF-2 were used for P-CSI reporting

- Finally, using PUCCH format 3 allows some more additional bits for, e.g. HARQ-ACK (which can be multiplexed with i1(1)+RI(1)), future extension (including Rel.14 Class A codebook extension and advanced CSI).

Proposal: Use PUCCH format 3 to report i1(1)+RI(1) in one subframe for the 1st eMIMO-Type

- A PF-3 resource is allocated for CSI reporting and/or A/N

- CSI-only reporting: Use PF-3 resource allocated for CSI

- A/N-only: If A/N is for Pcell only, use PF-1A/1B resource allocated for A/N. Otherwise use PF-3 resource allocated for A/N

- CSI and A/N collision:

  - UE drops P-CSI when UE transmits A/N only for the PCell

  - Otherwise, UE follows Rel-13 procedure as if PF-2 were used for P-CSI reporting

  - PUSCH-based A-CSI

The following three alternatives are possible:

- Alt1. Report both the 1st and the 2nd eMIMO-Types (1 possibility)

- Alt2. Report either only the 1st eMIMO-Type or the 2nd eMIMO-Type (2 possibilities)

- Alt3. Report either only the 1st eMIMO-Type, the 2nd eMIMO-Type, or both (3 possibilities)

First, we consider triggering an A-CSI report associated with only one of the two eMIMO-Types. If there is no interdependence between CSI calculations across the two eMIMO-Types, such A-CSI triggering does not cause any additional complication (either specification impact or performance).

That is, when an A-CSI reporting associated with the 1st eMIMO-Type (CLASS A with N(1) ports) is triggered, the reported i1(1)+RI(1) is intended to assist the eNB to perform UE-specific beamforming on CSI-RS.

On the other hand, when an A-CSI reporting associated with the 2nd eMIMO-Type (CLASS B K=1 with N(2) ports) is triggered, the UE measures a previously beamformed CSI-RS resource and reports CQI+PMI+RI associated with the N(2)–port CSI-RS resource (N(1) >N(2)). Overall, the UE shall not assume that the measured CSI-RS is beamformed with the PMI reported in the 1st eMIMO-Type.

Note that the feedback overhead associated with the 2nd eMIMO-Type can be much higher than that of the 1st eMIMO-Type. Therefore, supporting mechanism to trigger only A-CSI report for the 1st eMIMO-Type is clearly beneficial. Therefore, Alt1 is perhaps the least desirable alternative.

Second, comparing Alt2 and Alt3 amounts to assessing whether reporting A-CSI for both eMIMO-Types within one reporting instance is beneficial. As discussed before, CSI calculation associated with the two eMIMO-Types are independent of each other (since they are targeted for different purposes). In addition, since the precoding information for the 1st eMIMO-Type (i1(1)+RI(1)) is intended to assist the eNB in beamforming the CSI-RS, adding this CSI to an A-CSI report for the 2nd eMIMO-Type does not seem to be beneficial – at least from network performance perspective.

One may argue that triggering A-CSI only for the 1st eMIMO-Type (at most 11 bits regardless of the A-CSI mode – since i1(1)+RI(1) is always wideband – which results in a total payload of 19 bits with an 8-bit CRC) is wasteful. Hence triggering it together with an A-CSI for the 2nd eMIMO-Type is more economical. This argument, however, is weak at least for the following reasons:

- The eNB can trigger this report together with a grant for UL transmission. Therefore, this A-CSI report is multiplexed with data transmission.

- When the eNB needs to acquire A-CSI for the purpose of beamforming CSI-RS, triggering an A-CSI report with a potentially much higher payload (for the 2nd eMIMO-Type) is unnecessary and indeed wasteful – especially if such additional A-CSI is of no use/need to the eNB.

Therefore, reporting A-CSI for both eMIMO-Types within one reporting instance does not seem beneficial.

Proposal: Support triggering A-CSI report of only one of the two eMIMO-Types

- A third possibility of triggering A-CSI report containing both the eMIMO-Types is not supported

# Conclusions

To complete the specification support for hybrid CSI in Rel.14, the following proposals are made:

- Use PUCCH format 3 to report i1(1)+RI(1) in one subframe for the 1st eMIMO-Type

  - A PF-3 resource is allocated for CSI reporting and/or A/N

  - CSI-only reporting: Use PF-3 resource allocated for CSI

  - A/N-only: If A/N is for Pcell only, use PF-1A/1B resource allocated for A/N. Otherwise use PF-3 resource allocated for A/N

  - CSI and A/N collision:

    - UE drops P-CSI when UE transmits A/N only for the PCell

    - Otherwise, UE follows Rel-13 procedure as if PF-2 were used for P-CSI reporting

- Support triggering A-CSI report of only one of the two eMIMO-Types

  - A third possibility of triggering A-CSI report containing both the eMIMO-Types is not supported

# Appendix

In this section, BLER comparison between PUCCH format 2 and 3 with 6, 9, and 11-bit payload is shown in Figure 1. Simulation assumptions are given in Table 2. Observe that at BLER = 0.01 (typical operating target of PUCCH), 2-3dB coverage gain of format 3 over format 2 can be observed – depending on the payload size.

Table 2 Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Antenna | 1×2 |
| Channel model | CDL-B |
| Number of UEs | 1 |
| Channel estimation | Actual |

Figure 1 BLER results for PUCCH format 2 and 3

# References

- 3GPP, RAN1#86, Chairman’s Notes